Arc/Info implementation of the GEOLOGY map of Kansas

By Gina Ross and Rick Brownrigg

Automated Cartography

Kansas Geological Survey

April, 1992

This write-up describes an Arc/Info implementation of the 1:500,000 scale Geologic Map of

Kansas.  Map M-23, 1991.  This work was performed by the Automated Cartography section of

the Kansas Geological Survey, under contract to the Kansas Water Office.  The goal of this

project was to capture, in Arc/Info structures, the spatial and attribute information portrayed on

this map.

Source:

The data for Map M-23 was digitized from an original paper copy publication of the Geologic

Map of Kansas, Map M-1, 1964.  That map incorporated a 1:500,000 scale, USGS base map.

Because of this source and scale, there are some edge matching problems with the state boundary

as maintained by the Kansas Cartographic Database (KCD).

Data Organization:

Two classes of spatial feature are represented in the Arc/Info version of the geology map:

polygons depicting major geologic units, and lines demarking the bases of key geologic map

formations within the units.  These data are organized into two work spaces: GEOPOLY for the

polygonal data, and GEOLINES for the line data.  The data in each work space are further tiled

into coverages based upon county boundaries.  The coverages in both work spaces bear the name

of the corresponding county.  There are thus 105 coverages of polygonal data in the GEOPOLY

work space.  Within the GEOLINE work space, there are 54 coverages; this feature-class spans

only the eastern half of the State.

All coverages are stored as double-precision, with coordinates in units of feet from a projected

coordinate space.  The specific projection used is a standard lambert conformal projection for

Kansas, and may be described by the following excerpt of a command-file for the Arc/Info

PROJECT command:

INPUT

PROJECTION LAMBERT

UNITS FEET

PARAMETERS

33 00 00       /* standard parallels

45 00 00       /* standard parallels

-98 18 45      /* central meridian

36 52 30       /* latitude of origin

0              /* false easting (null)

0              /* false northing (null)

Each work space also holds an INFO table containing attributes for the corresponding feature

class.  These attributes effectively capture the information contained in the legend of the published

version of Geologic Map of Kansas; i.e, the tables contain the geologic names and ages of the

features, plus some miscellaneous cartographic items.  This information is reconciled with the

spatial data through the relational capabilities of Arc/Info.

Attribute Tables:

Attribute tables from the GEOPOLY work space:

Each polygon coverage has an associated PAT and AAT, hence, there are 105 of each.  One

additional table, FORMATIONS, exists to hold the geologic information, such as formation/unit

names, geologic age, rock-types, etc.  The PATs link the spatial, polygonal data to the

information kept in the FORMATIONS table by a common item, FORM-ID.  The AATs hold a

"feature code" for the arcs that comprise the polygons of each coverage; the principle usage for

this information is cartographic.  A description of the items in the three classes of attribute table

follows; all 105 of the PATs/AATs have an identical form, so a generic instance of each is

presented:

County.PAT:

DATAFILE NAME: county.PAT

5 ITEMS:       STARTING IN POSITION 1

COL  ITEM NAME      WDTH OPUT TYP  N.DEC

1    AREA           8    18   F    5

9    PERIMETER      8    18   F    5

17   county #       4    5    B    0

21   county-ID      4    5    B    0

25   FORM-ID        2    4    B    0

Where county is the name of a polygon coverage from the GEOPOLY work space.

-    FORM-ID is a numeric encoding of the type of geologic structure represented by the polygons.

This item also serves as a relate-key into the FORMATIONS table.

-    The remaining items are standard Arc/Info-maintained items for a PAT.

County.AAT:

DATAFILE NAME: county.AAT

8 ITEMS:       STARTING IN POSITION 1

COL  ITEM NAME      WDTH OPUT TYP  N.DEC     ALTERNATE NAME

1    FNODE#         4    5    B    0

5    TNODE#         4    5    B    0

9    LPOLY#         4    5    B    0

13   RPOLY#         4    5    B    0

17   LENGTH         8    18   F    5

25   county#             4    5    B    0

29   county-ID      4    5    B    0

33   ACODE          2    4    B    0

Where county is the name of a polygon coverage from the GEOPOLY work space.

-    ACODE is a feature code for each arc of the coverage:

4210 - arc is a generic geologic contact

4220 - arc is an "approximate" geologic contact

4310 - arc is coincident with a state boundary

4320 - arc is coincident with a county boundary

-    the remaining items are standard for an Arc/Info-maintained items for an AAT.

FORMATIONS:

DATAFILE NAME: FORMATIONS

6 ITEMS:       STARTING IN POSITION 1

COL  ITEM NAME      WDTH OPUT TYP  N.DEC

1    FORM-ID        2    3    B    0

3    FORMATION      65   65   C    -

68   SYSTEM         15   15   C    -

83   SERIES         25   25   C    -

108  ERA            10   10   C    -

118  ROCKTYPE       18   18   C    -

-    FORM-ID is a unique numerical key for all the records of this table.  It corresponds to the

various geologic structures represented in the polygonal coverages.

-    FORMATION is the name of the corresponding geologic structure.

-    SYSTEM gives the geologic age of the structure.

-    SERIES identifies the major "series" grouping of the structure.

-    ERA is the geologic time-era for the structure.

-    ROCKTYPE denotes the type of rock for the structure.  This field contains either "SEDIMENTARY"

or "CRETACEOUS IGNEOUS".

NOTE: for structures of rock-type "CRETACEOUS IGNEOUS", the items SYSTEM, SERIES, and

ERA do not apply, and are thus left blank.

Attribute tables from the GEOLINES work space:

Each line coverage has an associated AAT; there are 54 such tables.  One additional table,

BASELINES, exists to encode the geologic/cartographic information represented by the lines.

The AATs are related to the BASELINES table by a common item, BASE-ID.  A description of

the items in the two classes of attribute table follows; all the AATs have an identical form, so a

generic instance is presented:

County.AAT:

DATAFILE NAME: county.AAT

10 ITEMS:      STARTING IN POSITION 1

COL  ITEM NAME      WDTH OPUT TYP  N.DEC

1    FNODE#         4    5    B    0

5    TNODE#         4    5    B    0

9    LPOLY#         4    5    B    0

13   RPOLY#         4    5    B    0

17   LENGTH         8    18   F    5

25   county#        4    5    b    0

29   county-ID      4    5    B    0

33   FCODE          2    4    B    0

35   FORM-ID        2    4    B    0

37   BASE-ID        2    3    B    0

Where county is the name of a line coverage from the GEOLINES work space.

-    FCODE is merely a convenience item; it is a cartographic code denoting the line-coloring

used in the published version of the Geologic Map of Kansas:

FCODE     color

4211      orange

4212      green

4213      red

4214      purple

4215      blue

4260      orange-dashed (glacial boundary)

-    FORM-ID is a key into the FORMATIONS table of the GEOPOLY work space, and represents

the geologic structure (polygon) in which the line is contained.

-    BASE-ID is a numeric encoding of the formation-base represented by the lines.

BASE-ID also serves as a relate-key into the BASELINES attribute table.

-    The remaining items are all standard Arc/Info-maintained items for an AAT.

BASELINES:

DATAFILE NAME: BASELINES

4 ITEMS:       STARTING IN POSITION 1

COL  ITEM NAME      WDTH OPUT TYP  N. DEC

1    BASE-ID        2    2    B    0

3    FORM-ID        2    3    B    0

5    LABEL               2    2    C    -

7    BASE           35   35   C    -

-    BASE-ID is the unique key for records of this table.  These records represent the various

formation-bases denoted in the legend of Geologic Map of Kansas.  This item is the relate-key

for the AATs.  Note that like the published map, not every formation listed in this table has

corresponding lines in the coverages.

-    FORM-ID gives the relate-key of the geologic structure (polygons) that contains this formation-base.

It establishes a relate-item with table FORMATIONS of the GEOPOLY work space.

-    LABEL is a cartographic item; it gives the two-character abbreviation used to label the line-work

on the published version of Geologic Map of Kansas.

-    BASE holds the names of the formations.