Until the entire nation is complete and HUC_8 codes are stable, the Geospatial Data Gateway will only be serving two WBD layers; HU12 linework and HU12 polygons (which consists of HUC_8, HUC_10, and HUC_12 codes, as well as HUC_10 and HUC_12 names).  If you would like to derive HUC_2, HUC_4, HUC_6 codes and names or HUC_8 names, these values can be dissolved from WBD HU12 then joined to the USGS layer (HUC250K) for naming purposes (if ordering from the Gateway, the HUC250K USGS layer is listed as “8-Digit Hydrologic Units  1:250,000”.  Below is a description of how these values can be derived:

- Add Gateway WBD polygon layer to an ArcMap document (it is better to do an Extract by Area of Interest – by state) when ordering WBD data.

- Add a Field to the Attribute Table that values for HUC_2, HUC_4, or HUC_6 will be calculated.

3)  Next, Right click the new field’s header and select “Calculate Values”:

Say “Yes” to the pop up window below:

Depending on whether you want the HUC_2, HUC_4, or HUC_6 codes, you would calculate the last digit in the formula either 2, 4, or 6 accordingly:

- Next, dissolve for the HUC_4 (HUC_2 or HUC_6) results so that only one result exists per value:

The resulting shapefile of dissolved areas should display in ArcMap automatically.  The next step will be to create a new field for the item name that will be joined and calculated from the USGS HUC250K layer that can also be downloaded from the Geospatial Data Gateway:

5)  If you ordered the USGS HUC250K layer from the Gateway, add it to ArcMap in order to do the join and populate the HUC names for the corresponding HUC codes that you just dissolved.

6) Join and populate a new HUC_X_NAME column that you create in your Dissolved layer:

JOIN:

7.)  Next, calculate the HU_X_NAME column you created earlier with the joined values from the huc250k_a_extract layer.  Depending on which name values you are deriving, you will calculate based on the following:

HU_2 = REGION

HU_4 = SUBREGION

HU_6 = BASIN

HU_8 = SUBBASIN

In this example, we are calculating the HUC_4_NAME, so the calculation is being derived from the SUBREGION names:

Click “Yes” below:

Complete the Field Calculator with the appropriate field from the huc250k_a_extract layer:

Resulting in the following populated field:

If you had done these edits during an Edit Session, save your edits, and Remove your joins.