The LEO System: a copyrighted product of the Kansas Geological Survey, Kansas Geological Survey, 1999R.

Collins, Ph.D. Kansas Geological Survey  --  785-864-3965

Documentation prepared by David Collins.

***********************************************************************

6 -- A version (3.6) of the LEO System for conversion between location references based on the Public Land Survey System and location in geographic (longitude,latitude) coordinates or Universal Transverse Mercator (UTM) projected (x,y) coordinates. In lieu of full documentation, the following information is provided for your use:

*********************************************************************** operation: executable file 'leo3.6' and the binary database 'LEO3BASE' must be in the same directory.

files to show appropriate input file format and resulting output:

a  --  file with footages from implied or specified corner arranged from largest to smallest descriptors

variables: TSHP,RANG,EW,SECT,SUBD(4),CORN,FTNS,FTEW

format(2(1x,I2),1x,A1,1x,I2,4(1x,a2),1x,a2,2f8.0) 8 21 E 33                  -681.   1982.14 15 W 14             SE    657.  -1982.18 42 W 16             NW  -1323.     25.a   95.07919 39.31559   98.95392 38.82909  101.97436 38.49083acomma (not available in version compile 3-17-98),21,E,33,,,,,,-681.,1982.,15,W,14,,,,,SE,657.,-1982.,42,W,16,,,,,NW,-1323.,25.acomma   95.07919 39.31559   98.95392 38.82909  101.97436 38.49083

b  --  file with footages from implied or specified corner arranged from smallest to largest descriptors

variables: FTNS,FTEW,CORN,SUBD(4),SECT,TSHP,RANG,EW

format(2f8.0),1x,A2,4(1x,A2),3(1x,I2),1x,A1) -1320.  -2343. NE             16 22  5 E  1322.    603. SW             34 21  4 E  1660.    669.                15 19  4 E   346.   2644. SW             17 17  4 E  1971.  -2010. SE             20 17  4 E   992.   -338.                16 17  2 E -2287.   1328. NW              3 17  2 E   653.      2. SW              4 17  1 Eb   96.88490 38.14019   96.98458 38.17682   96.98253 38.39639   97.01306 38.56670   97.01069 38.55673   97.20695 38.56861   97.20132 38.60314   97.33422 38.59675

a   --   file with subdivisions arranged from largest to smallest location descriptors

variables: TSHP,RANG,EW,SECT,SUBD(4),CORN,FTNS,FTEW	format(2(1x,I2),1x,A1,1x,I2,4(1x,a2),1x,a2,2f8.0)20  3 E 21 NE SW NE SE N                   11 20 W 14 NE NE                           11 20 W 14 NE NE SE SE  c                  11 20 W 14 NE          N                   11 20 W 14 NE NE SE SE SW                  33 41 W 26 NW SE SW NW SW                  20 43 W 23 ne nw se s2 ca  --

LEO3.6 determines (longitude,latitude) pair for specified corner or side mid-point, or returns four corners of smallest subdivision if no corner or mid- is specified   97.10028 38.30005   99.50429  39.10285   99.49966  39.10287   99.50436  39.09922   99.49974  39.09924   99.50031 39.09969   99.50429 39.10285   99.50089 39.09924  101.75004 37.14978  102.03317 38.30278acomma  --  comma delimited data (not available in version compile 3-17-98)	(requires comma after last data element if missing)	variables: TSHP,RANG,EW,SECT,SUBD(4),CORN,FTNS,FTEW,3,E,21,NE,SW,NE,SE,N,,,,20,W,14,NE,NE,,,,,,,20,W,14,NE,NE,SE,SE, c,,,,20,W,14,NE,  ,  ,  ,N,,,,20,W,14,NE,NE,SE,SE,SW,,,,41,W,26,NW,SE,SW,NW,SW,,,,43,w,23,ne,nw,se,s2,c,,,acomma   97.10028 38.30005   99.50429  39.10285   99.49966  39.10287   99.50436  39.09922   99.49974  39.09924   99.50031 39.09969   99.50429 39.10285   99.50089 39.09924  101.75004 37.14978  102.03317 38.30278b  --  file with subdivisions arranged from smallest to			largest location descriptors	variables: FTNS,FTEW,CORN,SUBD(4),SECT,TSHP,RANG,EW	format(2f8.0),1x,A2,4(1x,A2),3(1x,I2),1x,A1)                N  SE NE SW NE 21 20 23 E                         NE NE 14 11 20 W                NE SE SE NE NE 14 11 20 W                SW NW SW SE NW 29 33 43 W                W  SE SE NW NE 23 20 43 Wb  --  LEO3.6 determines UTM zone and corresponding X,Y coordinates  336683.00  4240243.15 15  456394.91  4328103.00 14     456794.69  4328103.50 14  456386.72  4327700.00 14     456785.88  4327700.50 14  456787.75  4327801.22 14  764549.08  4115397.21 13  759438.56  4243368.98 13bcomma  --  comma delimited data (not available in version compile 3-17-98)	variables: FTNS,FTEW,CORN,SUBD(4),SECT,TSHP,RANG,EW,,N,SE,NE,SW,NE,21,20,23,E,,,,,NE,NE,14,11,20,W,,NE,SE,SE,NE,NE,14,11,20,W,,SW,NW,SW,SE,NW,29,33,43,W,,W,SE,SE,NW,NE,23,20,43,Wbcomma --  LEO3.6 determines UTM zone and corresponding 			X,Y coordinates  336683.00  4240243.15 15  456394.91  4328103.00 14     456794.69  4328103.50 14  456386.72  4327700.00 14     456785.88  4327700.50 14  456787.75  4327801.22 14  764549.08  4115397.21 13  759438.56  4243368.98 13b14  --  file with subdivisions arranged from smallest to			largest location descriptors (same as input2b)	variables: FTNS,FTEW,CORN,SUBD(4),SECT,TSHP,RANG,EW	format(2f8.0),1x,A2,4(1x,A2),3(1x,I2),1x,A1)                N  SE NE SW NE 21 20 23 E                NE SE SE NE NE 14 11 20 W                SW NW SW SE NW 29 33 43 W                W  SE SE NW NE 23 20 43 Wb14  --  LEO3.6 determines X,Y coordinates for user specified zone 			(e.g., zone = 14)  861426.58  4246678.70 14  456787.75  4327801.22 14  231659.45  4115517.21 14  234758.53  4243557.48 14  --   file with decimal degrees	variables: PLON,PLAT; (longitude,latitude)	format: comma or space delimited, longitude positive or negative	(output format, which is: (1x,F11.5,F9.5), may be used for input).0,37.5.88510,38.13997.98501,38.17687.98219,38.39632.01253,38.56678.01103,38.55632.20639,38.56860.20162,38.60349.33399,38.59667 --  PLSS location in footages from nearest surveyed section corner,		formatted from least to most specific (township, range, 		direction, section, nearest corner, feet north, feet east)29 34 W 26             NE  -1450.   -491.22  5 E 16             NE  -1400.  -2401.21  4 E 34             SW   1341.    480.19  4 E 15             SW   1635.    766.17  4 E 17             SE    342.  -2496.17  4 E 20             SE   1823.  -2108.17  2 E 16             SE    988.   -179.17  2 E  3             NW  -2160.   1240.17  1 E  4             SW    624.     68.a   --   file with UTM coordinates (metric)	variables: UTMx,UTMy,zone		UTM zones in Kansas:			zone 15 -- 90-96 degrees west longitude			zone 14 -- 96-102 degrees west longitude			zone 13 -- 102-108 degrees west longitude	format: comma or space delimited, longitude positive or negative	(UTM output format, which is: (2f12.2,1x,i2), may be used for input)  320749.72  4353651.00 15  504100.03  4297504.00 14  240466.00  4264165.50 14a  --   three subdivisions arranged largest to smallest 8 21 E 33 NW NE SW    NE14 15 W 14 SE SW SE    C 18 42 W 17 NE SE NE    C b  --  file with UTM coordinates (metric) same as last two 			coordinate pairs from input4a	variables: UTMx,UTMy (requires user identification of zone)   504100.03 4297504.00240466.00,4264165.50b14  --  four subdivisions arranged largest to smallest with correct 			zone identification (zone = 14)14 15 W 14 SE SW SE SW NE18 42 W 17 NE SE NE SW NEb15  --  four subdivisions arranged largest to smallest with incorrect 			zone identification (zone = 15)                                            -10  [invalid longitude]18 13 E 16 NE NW SW NE C ***********************************************************************:conversion programs contained in LEO3, and earlier versions LEO, are normally discussed in terms of conversions between 'legal' geographic reference systems (and now the projected UTM map system).  'Legal' in this context refers only to the fact the format of location descriptions conform to the nomenclature of Public Land Survey System (PLSS) which is used in actual LEGAL of surveyed property.  LEO3 is NOT intended for use in such locations for use in actual legal proceedings.  are many irregular section (not accurately described a four sided polygon) for which the LEO system can only produce approximations of the correct conversion.  The conversion for many partial sections or sections which otherwise have subdivisions described as 'lots' remain inadequate.  The most errors of this type will be associated with locations in western parts of the western tier of sections in Range 8 East (increasing with each Township south), the western tier of sections in 31-35 of Range 17 East, the western tier of sections in 1-5 of Range 9 West, and the western tier of sections in 1-5 of Range 25 West; or with locations in the northern parts the northern tier of sections in Township 26 of Ranges 3-41 West., for the vast majority of sections in Kansas, the LEO3 system provide conversions which are satisfactory for many applications.provides longitude,latitude coordinates with a precision of fiveplaces in decimal degree notation.  This is comparable to one meter precision (i.e., plus or minus three feet).corner locations used in LEO3BASE are derived from USGS 7.5'maps at a scale of 1:24,000.  Location accuracies, with National Map Accuracy Standards plus repeatability of the Survey's digitizing equipment, are on the order of -50 feet of true ground position.**********************************************************************BEST results in use of LEO3 will be obtained if input datataken from differential processing of Global Positioning data which is then converted using LEO3 to footages from corners of the Public Land Survey System.  The reverse can be used to relocate positions originally located by footages from a PLSS section corner.  Using geographic output from LEO3 as input to GPS equipment, the GPS can then be used as a navigating device to find the in the field.**********************************************************************user is advised that Public Land Survey descriptions other thanmeasurements from surveyed section corners are not generally for the purpose of determining a point location.  Therefore, such as those commonly used in older oil and gas records to well locations must be considered to be approximations of true location.  example, "the center of the SW quarter of the NE quarter of NW quarter of the NW quarter of section 27, Township 15 South, 29 West," which would be entered as:----:   15 29 W 27 NW NW NE SW  Cas:15,29,W,27,nw,nw,se,sw,c,,, as:                 C  SW NE NW NW 27 15 29 Was:,,C,SW,NE,NW,NW,27,15,29,Wlocated at:(lon,lat) = (100.53822,38.72758)this geographic coordinate is input to LEO3 with output option 2, resulting location will be:   15 29 W 27             NW   -496.    827.is 496 feet south and 827 feet east of the NW corner of 27.this location is provided as input to LEO3 as:   15 29 W 27             NW   -496.    827.conversion to geographic coordinates will again produce (using option 2):   (lon,lat) = (100.53822,38.72758)geographic coordinates, converted to PLSS format using option 0, give the original description:   15 29 W 27 NW NW NE SW  C , a different location:   15 29 W 27             NW   -420.    750.   (lon,lat) = (100.53849,38.72779)entered in LEO3 as (lon,lat) using option 0 will also return:   15 29 W 27 NW NW NE SW  C the approximate location.                      Variables:                      ---------- SOURCE CODES    (provided interactively in all cases)	SORCE and RESULT = indicate the direction of the conversion:		  1=Legal; Public Land Survey location reference		  2=Geographic; longitude,latitude coordinates		  3=UTM; Universal Transverse Mercator (x,y) in meters PUBLIC LAND SURVEY (PLSS) LOCATION DESCRIPTION	TSHP = Township number, 1-35	RANG = RANG number, 1-25 if EW='E', 1-43 if EW='W'	EW = RANG direction from central meridian 'E' or 'e'= East,		W' or 'w'= West	SECT = Section number, 1-36, =0 to specify the township		as the area for corner/side midpoint selection or as		the four corner locations for the 4-corner option	SUBD=specify 1-4 subdivisions of the section,each applied to	the result of the previous ones.  Subdivisions are defined as	selection of a half or quarter of the area being divided,and	are specified by letters representing points of the compass:	1/4 Area....    NE or Ab....Northeast  (b=blank)			NW or Bb....Northwest			SW or Cb....Southwest			SE or Db....Southeast			      bb....Unused	1/2 Area..... Nb or N2......North			Sb or S2....South			Eb or E2....East			Wb or W2....West			      bb....Unused      SUBD(1)=The largest (1/n section) subdivision,and the one                that is applied first (to the whole section).      SUBD(2)=The second largest (1/n-1/n section) subdivision.      SUBD(3)=The second smallest (1/n-1/n-1/n section) subdivision      SUBD(4)=The smallest (by area 1/n-1/n-1/n-1/n section) and the		last subdivision to be applied to the result of the		preceeding three subdivisions in SUBD(1-3)		(where each 1/n represents either 1/2 or 1/4)	CORNR = If SOURC=1 (Legal), CORNR specifies, in the same form 		as the SUBD values, one of nine points in the target		area defined by the legal description,including the		subdivisions in SUBD.  The nine points are the four		corners,the side midpoints (S,N...) and the center.		When SORCE=2 (Geographic) and OPTION = 0, CORNR is 		set to indicate the closest one of the nine points 		within the fourth subdivision to the point described 		in PLON,PLAT 	FTNS = Footage applied to the north if > 0, south (if < 0)	FTEW = Footage applied to the east if > 0, west if < 0	  NOTE: footages are applied from the section or township		corner implied by the signs of the footage values,		unless a corner is identified explicitly by CORNR.		    Implicit:	FTNS<0 and FTEW<0 imply CORNR=NE				FTNS<0 and FTEW>0 imply CORNR=NW				FTNS>0 and FTEW>0 imply CORNR=SW				FTNS>0 and FTEW<0 imply CORNR=SE		    Explicit:	If the section is not laid out on				true NS-EW lines, then (as an example)				some locations in the section may lie				to the north and east of the SE corner.				To calculate the geographic coordinates				of such a point, set:				   FTNS>0 and FTEW>0, and				   CORNR=SE (to over-ride implicit SW)	  NOTE: when both footages are 0,the lengths of the section 		sides (in feet) are returned in LONX (1-4,1) OPTIONS WHEN INPUTING PLSS LOCATION DESCRIPTIONS    (currently required with each location record in batch input)	OPT=Specifies method of locating point in the target area		within the section (see Subdivision).  When SORCE=1,		the option implied by the OPT value is:		0=return four corners of the smallest area specified 		     by SUBD or, if requested, its center, a corner or		     a side midpoint (must use CORNR to select closest 		     significant point)		1=select the corner/side midpoint specified in SUBD		     of a selected section (SECT) or township (SECT=0)		2=apply given footages from implied section corner		     (or return section side lengths when footages=0)		3=return the locations of the four corners of the		     specified section or the township (SECT=0)		4=select the center (as in 0), but in degrees,		     minutes,and seconds of lon/lat,the DMS option		     (may use CORNR to select closest significant point)		5=(as in 1) select the specified corner or side		     midpoint of a section, but return in DMS form		6=apply footages from the implied section or township		     corner, but return in DMS form in LONX (1,1-3)		     and LATX (1,1-3)		7=return the section or township four corners,		     but in the DMS form	      Options 0/4 will return four corners if a point location	      (i.e., center, corner or side mid-point) is not explicitly	      requested.  With these options, "C" means 'CENTER.'	      Options 1/5 will return four corners if a point location	      (i.e., center, corner or side mid-point) is not explicitly	      requested.  With these options, "C" means 'SW CORNER.'	      Options 1/5 will NOT return a CENTER location.	      Options 3/7 will only return section or township corners.	      No subdivisions are considered in these option.	NOTE: For OPT=0-2, the geographic location is returned via PLON	      and PLAT with a positive longitude. All other options use	      LONX and LATX to return 1 or 4 values, (all in DMS except	      the basic 4-corners option=3), with positive longitude(s) OPTIONS WHEN INPUTING GEOGRAPHIC LOCATION COORDINATES    (currently sets OPT=0 if batch processing)	OPT=Specifies method of locating point in the target area		within the section. When SORCE=2,the option implied by 		the OPT value is:		0=Input is in decimal degrees (in PLON < 0, PLAT)		1=Input is in the DMS form (in LONX > 0, LATX) GEOGRAPHIC LOCATION DESCRIPTIONS	PLON = Longitude of Point for or from conversion (positive)	PLAT = Latitude of Point	LONX,LATX = longitude,latitude in degrees, minutes, seconds for		the DMS option, if SORCE=2, the geographic location is		stored in LONX and  LATX with degrees of longitude in		LONX(1,1) and minutes and seconds in LONX(1,2-3).  The		corresponding degrees,minutes,seconds of latitude are 		stored in LATX(1,1-3).  When SORCE=1 (Legal),the		converted output location is stored in the same manner, 		as well as in PLON and PLAT.  For the 4 corners option, 		the 4 corner locations are stored similarly with the NW		corner in LON/LATX(1,1-3), the NE corner in (2,*)		the SW in (3,*) and the SE in (4,*). OPERATION STATUS CODES	STAT = code indicating success or failure of the requested		operation.  The codes are:		  1=SUCCESS		 10=Warning: Footage is measured from a protracted		    corner.  No PLSS corners are available for this		    section if option searched for closest corner.		 16=Warning: N-S footage exceeds average distance		    between north and south section corners.		 17=Warning: E-W footage exceeds average distance 		    between east and west section corners.		 34=Part of the requested subdivision lies outside the		    state.  Information is provided for the portion of		    the subdivision in Kansas.		 35=Part of the requested subdivision lies outside the		    state. The requested point in that subdivision is		    in Kansas.		 36=Part of the requested subdivision lies outside the		    state. The requested point in that subdivision also		    lies outside Kansas. Information is provided for		    the corresponding point in the partial subdivision.		 41=Footage is measured from protracted (NE) 			section corner.		 42=Footage is measured from protracted (NW) 			section corner.		 43=Footage is measured from protracted (SW) 			section corner.		 44=Footage is measured from protracted (SE) 			section corner.		 45=Combination of STAT=16 and STAT=41		 46=Combination of STAT=17 and STAT=41		 47=Combination of STAT=16 and STAT=42		 48=Combination of STAT=17 and STAT=42		 49=Combination of STAT=16 and STAT=43		 50=Combination of STAT=17 and STAT=43		 51=Combination of STAT=16 and STAT=44		 52=Combination of STAT=17 and STAT=44		 -1=Township number not permitted		 -2=Range number not permitted		 -3=RANG direction (EW) not 'E' or 'W'		 -4=Section number not permitted		 -5=Illegal/unrecognized subdivision spec  #1		 -6=    "        "            "       "    #2		 -7=    "        "            "       "    #3		 -8=    "        "            "       "    #4		 -9=Illegal conversion direction (SORCE) value		-10=Invalid Longitude for LEO/KGS		-11=Invalid Latitude for LEO/KGS		-12=Invalid value for OPT		-13=Database (LEOBASE) I/O error		-14=Thrashing error on the Township level		-15=Thrashing error on the Section level		-16=Illegal footages from explicit corner		-17=Error in degrees,minutes,or seconds value		-18=Missing SUBD value for OPT=1 or 5		-19=Township record has no usable sections		-20=Point-in-Polygon Error		-21=One township corner missing from the Kansas PLSS		-22=Two township corners missing from the Kansas PLSS		-23=Three township corners missing from the Kansas PLSS		-24=All four township corners missing from the Kansas		    PLSS		-25=Illegal or unrecognized CORNER specification		-26=Option is not allowed for townships		-27=Illegal record # in township search		-28=Failed search within incomplete township		-31=One or more section corner protracted or now in		    Missouri		-32=Section is not within the Kansas PLSS or no longer		    in Kansas		-33=Requested subdivision is entirely outside the state		-37=The requested point lies outside the state.		    Only one corner of the specified subdivision lies		    in Kansas.		-51=Implied corner of origin for footage measurement is		    ambiguous.  Location could be due north of SE or SW		    corner.		-52=Implied corner of origin for footage measurement is		    ambiguous.  Location could be due south of NE or NW		    corner.		-53=Implied corner of origin for footage measurement is		    ambiguous.  Location could be due east of SW or NW		    corner.		-54=Implied corner of origin for footage measurement is		    ambiguous.  Location could be due west of NE or SE		    corner.		-99=Special section requires non-standard subdivision CODES REFLECTING BASIS FOR NONSTANDARD SUBDIVISION OF SECTION IN PLSS    (even if subdivided in four equal parts by LEO3)	These codes are not yet fully implemented in LEO3.21	The code is provided as output based on evaluation of the 	section flag (see SFLAG).  Therefore, a section code of '1111' 	may exist in LEO3BASE which will eventually be changed to permit 	more appropriate (and hopefully more accurate conversion between 	PLSS style location descriptions and geographic coordinates.	Standard section subdivision (for full sections) in the Public 	Land Survey System (PLSS) was accomplished by bisecting sides of 	sections to establish quarter corners and then connecting 	opposite quarter corners to establish boundaries of the quarter 	sections.  The process would then be repeated to establish 	quarter-quarters, etc.  If a standard section could not be 	surveyed because of the existance of excluded water bodies 	(RIVER), indian lands (TREATY), military reserves or land grants 	(RESERVE), state boundaries (STLINE) or survey adjustment -- 	normally along sections at the north or west edge of each 	township (SLACK*)	NONSTD = 2, if STLINE = 1	       = 4, if RIVER = 1	       = 6, if STLINE = 1, and RIVER = 1	       = 8, if RESERVE = 1	       = 10, if STLINE = 1, and RESERVE = 1	       = 12, if RIVER = 1, and RESERVE = 1	       = 14, if STLINE = 1, and RIVER = 1, and RESERVE = 1	       = 16, if TREATY = 1	       = 18, if STLINE = 1, and TREATY = 1	       = 20, if RIVER = 1, and TREATY = 1	       = 22, if STLINE = 1, and RIVER = 1, and TREATY = 1	       = 24, if RESERVE = 1, and TREATY = 1	       = 26, if STLINE = 1, and RESERVE = 1, and TREATY = 1	       = 28, if RIVER = 1, and RESERVE = 1, and TREATY = 1	       = 30, if STLINE = 1, and RIVER = 1, and RESERVE = 1,				 and TREATY = 1	       = 32, if SLACKN = 1, or SLACKW = 1, or SLACKNW =1	       = 34, if STLINE = 1 & SLACK	       = 36, if RIVER = 1 & SLACK	       = 38, if STLINE = 1, and RIVER = 1 & SLACK	       = 40, if RESERVE = 1 & SLACK	       = 42, if STLINE = 1, and RESERVE = 1 & SLACK	       = 44, if RIVER = 1, and RESERVE = 1 & SLACK	       = 46, if STLINE=1, and RIVER=1, and RESERVE=1 & SLACK	       = 48, if TREATY = 1 & SLACK	       = 50, if STLINE = 1, and TREATY = 1 & SLACK	       = 52, if RIVER = 1, and TREATY = 1 & SLACK	       = 54, if STLINE=1, and RIVER=1, and TREATY=1 & SLACK	       = 56, if RESERVE = 1, and TREATY = 1 & SLACK	       = 58, if STLINE=1, and RESERVE=1, and TREATY=1 & SLACK	       = 60, if RIVER=1, and RESERVE=1, and TREATY=1 & SLACK	       = 62, if STLINE = 1, and RIVER = 1, and RESERVE = 1,				 and TREATY = 1 & SLACK		= NONSTD+100, if one or more corners of the section		must be redefined from the values given in LEO3BASE.		= NONSTD+200, if the section involved in either		conversion is in some way irregular, requiring that		subdivision of the section be performed in a non-std		manner (i.e. other than the standard four equal parts		subd.)	       = 0,if the section is subdivided in the Public Land 		Survey in the standard fashion FLAGS GIVING INFORMATION ABOUT TOWNSHIPS, SECTIONS OR SECTION CORNERS      TFLAG = township flag, = 36 if all 36 sections have 4 defined          corners, = 0 if no sections have 4 defined corners, or = n,         0< n <36, n = number of sections with 4 defined corners	SFLAG = Section flag	     For sections with 4 defined corners in LEO3BASE which are	     sufficient for subdivided by standard procedures:  		SFLAG = four digit code (>0) with each digit having 		a value of 1, 2, 3, or 4.  The digits reflect the		source status of each of the four corners as follows:		   1 = corner surveyed in PLSS, with location in KS.		   2 = corner not surveyed in Kansas PLSS. LEO3BASE		       uses protracted location inside Kansas borders.		   3 = corner not surveyed in Kansas PLSS. LEO3BASE		       uses protracted location outside Kansas borders.		   4 = corner surveyed in Kansas PLSS, with location 		       now in Missouri due to shift in state boundary 		       associated with shift in location of Missouri 		       River.		The digits refer, in order (left to right), to the NE, 		NW, SW, and SE corner locations (similar to the USGS		use of 'a-b-c-d' with 'NE-NW-SW-SE.'  Therefore, SFLAG		for sections which LEO3 will subdivide by standerd 		procedures will have non-sero digits with values from 		1 to 4 and at least one digit with a value of 1 or 2.  		For example:		Section flag	Meaning		    1111	all corners surveyed and in Kansas		    1311	the NW corner location is protracted				  into Missouri.		    1121	the SW corner location is protracted				  into excluded areas in Kansas.		    4111	the NE corner is a surveyed location				  of the Kansas Public Land Survey				  System which is now in Missouri.	     For partial sections falling along the Missouri, Oklahoma 	     or Missouri borders (other than the Missouri River):		SFLAG = 5000 + normal code		On the Missouri line, 3113 becomes 8113.		The section at the SE corner of Kansas has SFLAG=8133.		On the Oklahoma line, 1133 becomes 6133.		The section at the SW corner of Kansas has SFLAG=6333.		On the Colorado line, 1331 becomes 6331.	     For partial sections resulting from areas excluded from the	     PLSS by water (major rivers):		SFLAG = 500 + normal code	     For partial sections resulting from areas excluded from	     the PLSS by milirary reserves (e.g., Ft. Riley and Ft. 	     Leavenworth) or prior land grants (e.g., the Half-Breed 	     Lands north of the Kansas River between Topeka and 	     Lawrence, or the Republican River Bridge Company Lands on 	     the south side of the Ft. Riley Reserve):		SFLAG = 50 + normal code	     For sections in which the PLSS was completed in two stages	     as the result of lands originally excluded from the PLSS 	     (primarily as Indian lands with a few land grants):		SFLAG = 5 + normal code			In most cases these sections are complete 			(i.e., they are approximately 640 acres - one 			square mile).  Due to the separate surveys on			either side of old Indian boundaries, these 			sections have a mixture of standard and 			nonstandard subdivisions and may require care			in the interpretation of legal descriptions 			of locations within the sections.  Footage 			measurements would generally be preferable.	     For sections which do not exist because the area where they	     would be located falls completely within lands excluded	     from the Kansas Public Land Survey System):		SFLAG = 0, or -n as defined below (when some form of			processing or comment is deemed appropriate)	     For sections which require some nonstandard treatment for 	     accurate subdivision:		SFLAG = -n, where n is the normal code for SFLAG.			When SFLAG<0 the system calls FEDX, a subroutine			for special handling.  Within FEDX, the process 			is directed to a statement number which matches			the township record number in LEO3BASE of the 			township which contains that section.  The 			township record number is based on Township, 			Range, and Range direction (E or W) as follows:			If range direction is east:				TRECB = (TSHIP - 1) * 68  +  RANGE + 43			If range direction is west:				TRECB = (TSHIP - 1) * 68  +  44 - RANGE			Routines for any sections which require special 			handling within a township will be found in FEDX			after the statement number matching that			township's record number in LEO3BASE.	 SCOR = corner flag		If the absolute value of SFLAG = abcd (non-zero), then:			SCOR(1) = a, or SCOR(1) = a-5 if a>5			SCOR(2) = b, or SCOR(2) = b-5 if b>5			SCOR(3) = c, or SCOR(3) = c-5 if c>5			SCOR(4) = d, or SCOR(4) = d-5 if d>5		The value of SCOR reflects the source status of each		section corner as described for SFLAG.  This source		status is based on the corner's representation on USGS		7.5' quadrangle maps or, where that is unclear, on		information obtained from county surveyor's or the		original plats of the PLSS maintained by the Kansas		Historical Society in Topeka, Kansas.