Digital maps of the altitude of the land surface for

the State of Kansas.

Documentation prepared by K.E. Juracek and C.V. Hansen,

U.S. Geological Survey, Lawrence, Kansas.

1. Abstract

A total of 54 digital hypsography maps were created

from 1:100,000-scale digital line graph (DLG) hypsography

data sets produced by the U.S. Geological Survey. Together,

the digital maps provide statewide topographic information

for Kansas. The digital altitude contours, mapped in the

Lambert Conformal Conic projection, have a contour interval

of either 5 or 10 meters.

Descriptors: altitude, land surface, Kansas

2. Applications that use this data

Digital contour maps of land-surface altitude have

multi-use potential.

2.1 Intended use of data

These digital maps were developed to be part of a multi-

layer digital data base pertaining to the major aquifers in

Kansas. Potential uses of these digital maps include

studies addressing the location, use, management, and pro-

tection of ground-water resources in Kansas. Additionally,

these digital maps are useful for a variety of other

natural-resources management and research applications.

2.2 Limitations of data

Because these digital maps were developed from medium-

scale source information (1:100,000), they are useful for

regional-level analyses. They are not intended for use in

detailed, site-specific investigations.

3. Attribute discussion

The line attributes are contained in the INFO file for

each digital map with the .AAT suffix. Listed below, as an

example, are the attribute names and data types for the

digital map TOPEKA_TOPO. All 54 digital maps have the same

set of line attributes.

ITEM NAME         WDTH OPUT TYP N.DEC  ALTERNATE NAME

FNODE#              4    5  B    -

TNODE#              4    5  B    -

LPOLY#              4    5  B    -

RPOLY#              4    5  B    -

LENGTH              8   18  F    5

TOPEKA_TOPO#        4    5  B    -

TOPEKA_TOPO-ID      4    5  B    -

ALTITUDE_METERS     5    5  I    -    ALTITUDE

ALTITUDE_METERS2    5    5  I    -    ALTITUDE2

CODE                4    4  I    -

A description of each attribute item follows:

FNODE#

Definition: See Understanding GIS--The ARC/INFO Method.

Revision 6. 1992. Environmental Systems Research

Institute, Inc. Redlands, CA. p. 5-9.

TNODE#

Definition: See Understanding GIS--The ARC/INFO Method.

Revision 6. 1992. Environmental Systems Research

Institute, Inc. Redlands, CA. p. 5-9.

LPOLY#

Definition: See Understanding GIS--The ARC/INFO Method.

Revision 6. 1992. Environmental Systems Research

Institute, Inc. Redlands, CA. p. 5-9.

RPOLY#

Definition: See Understanding GIS--The ARC/INFO Method.

Revision 6. 1992. Environmental Systems Research

Institute, Inc. Redlands, CA. p. 5-9.

LENGTH

Definition: The length of the arc (in meters).

TOPEKA_TOPO#

Definition: See Understanding GIS--The ARC/INFO Method.

Revision 6. 1992. Environmental Systems Research

Institute, Inc. Redlands, CA. p. 5-9.

TOPEKA_TOPO-ID

Definition: See Understanding GIS--The ARC/INFO Method.

Revision 6. 1992. Environmental Systems Research

Institute, Inc. Redlands, CA. p. 5-9.

ALTITUDE_METERS

Definition: The altitude of the land surface (in

meters).

Legitimate Entries: Varies depending on the digital

map. Generally, legitimate values include -9,999 and

any integers between 200 and 1,550 meters. A value

of -9,999 indicates that the arc is not a contour.

ALTITUDE_METERS2

Definition: The altitude of the land surface (in

meters). This attribute provides the additional in-

formation necessary to describe "carrying contours."

A carrying contour represents the merger of two or

more contours into a single contour (that is, in an

area of very steep slope). Together, the attributes

ALTITUDE_METERS and ALTITUDE_METERS2 indicate the

range of altitudes represented by the carrying

contour.

Legitimate Entries: Varies depending on the digital

map. Generally, legitimate values include -9,999 and

any integers between 200 and 1,550 meters. A value

of -9,999 indicates that the arc is not a contour.

CODE

Definition: A 3- or 4-digit integer that indicates the

type of feature represented (for example, contour or

other linear feature), the confidence in the feature's

location (for example, approximate, inferred, specu-

lative), and the cartographic symbol with which the

feature is displayed (for example, continuous line,

dashed line).

Legitimate Entries: The legitimate entries for this

attribute are summarized in the following table (and

are explained in section 4 of this documentation):

CODE  TYPE               CONFIDENCE    LOOK

111 CONTOUR              APPROXIMATE CONTINUOUS

114 CONTOUR              APPROXIMATE CONTINUOUS-HACHURE

122 CONTOUR              INFERRED    LONG-DASH

125 CONTOUR              INFERRED    LONG-DASH-HACHURE

133 CONTOUR              SPECULATIVE SHORT-DASH

136 CONTOUR              SPECULATIVE SHORT-DASH-HACHURE

611 CONTOUR-CARRYING     APPROXIMATE CONTINUOUS

614 CONTOUR-CARRYING     APPROXIMATE CONTINUOUS-HACHURE

622 CONTOUR-CARRYING     INFERRED    LONG-DASH

625 CONTOUR-CARRYING     INFERRED    LONG-DASH-HACHURE

713 CONTOUR-SUPPLEMENTAL APPROXIMATE SHORT-DASH

716 CONTOUR-SUPPLEMENTAL APPROXIMATE SHORT-DASH-HACHURE

722 CONTOUR-SUPPLEMENTAL INFERRED    LONG-DASH

723 CONTOUR-SUPPLEMENTAL INFERRED    SHORT-DASH

725 CONTOUR-SUPPLEMENTAL INFERRED    LONG-DASH-HACHURE

726 CONTOUR-SUPPLEMENTAL INFERRED    SHORT-DASH-HACHURE

733 CONTOUR-SUPPLEMENTAL SPECULATIVE SHORT-DASH

736 CONTOUR-SUPPLEMENTAL SPECULATIVE SHORT-DASH-HACHURE

811 CONTOUR-AMENDED      APPROXIMATE CONTINUOUS

814 CONTOUR-AMENDED      APPROXIMATE CONTINUOUS-HACHURE

822 CONTOUR-AMENDED      INFERRED    LONG-DASH

825 CONTOUR-AMENDED      INFERRED    LONG-DASH-HACHURE

911 CONTOUR-UNDERWATER   APPROXIMATE CONTINUOUS

914 CONTOUR-UNDERWATER   APPROXIMATE CONTINUOUS-HACHURE

922 CONTOUR-UNDERWATER   INFERRED    LONG-DASH

925 CONTOUR-UNDERWATER   INFERRED    LONG-DASH-HACHURE

1011 LINE-CLOSURE        APPROXIMATE CONTINUOUS

1014 LINE-CLOSURE        APPROXIMATE CONTINUOUS-HACHURE

1022 LINE-CLOSURE        INFERRED    LONG-DASH

1025 LINE-CLOSURE        INFERRED    LONG-DASH-HACHURE

4. Procedures used to create or automate data

The digital maps of the altitude of the land surface

were developed from 1:100,000-scale digital line graph

(DLG) hypsography data produced by the U.S. Geological

Survey. The source maps are compliant with National Map

Accuracy Standards.

The DLG data were transferred from 9-track tape to a

Unix-based Data General workstation.

In DLG format, each 30- by 60-minute topographic quad-

rangle data set is composed of eight digital files. The

conversion of the DLG files into digital maps (i.e., Arc/

Info coverages) was achieved with the assistance of

several Fortran and Arc Macro Language (AML) programs.

A Fortran program was used to check the number of major

and minor attribute pairs for lines in each of the eight

DLG files. The digital maps created from each of the

eight DLG files must contain the same number of major

and minor attribute pairs in order to be successfully

combined into a single digital quadrangle map.

An AML program was used to convert each of the eight

DLG files in a quadrangle data set into a separate digital

map. A second AML program was used to join the eight

digital maps for line features in a quadrangle into a

single digital map. During this procedure, all "neat lines"

were deleted, and edgematching was performed (with a match

tolerance of 25.4 meters). The resultant topographic-

contour digital map is identified with a naming convention

that appends "_TOPO" to each quadrangle name.

The digital map coordinates then were projected from the

Universal Transverse Mercator (UTM) projection to a Lambert

Conformal Conic projection (with the parameters that were

approved as a standard by the Kansas State GIS Policy

Board). Kansas spans three UTM zones, which does not allow

all the digital quadrangle maps to be viewed simultaneously

or combined as a single digital map in their original

coordinate systems. Conversely, the Lambert Conformal Conic

projection can accommodate such tasks.

Another AML program was used to identify the unique

occurrences of major and minor attribute pairs in each

digital quadrangle map. This information was used to

convert the data in the major and minor attribute pairs

for the digital maps into a single, consistent format.

The items ALTITUDE_METERS, ALTITUDE_METERS2, TYPE, CON-

FIDENCE, LOOK, and CODE were added to the line-feature

attribute files (those with the suffix .AAT in Info)

using an AML program. Using another AML program, these

items were populated based on the data contained in

the major and minor attribute pairs for each digital map

as described in the DLG Data Users Guide (U.S. Geological

Survey, 1989). The items ALTITUDE_METERS, ALTITUDE_METERS2,

and CODE are defined in section 3 (Attribute discussion)

of this documentation.

The item TYPE is a 1-2 digit integer that indicates the

type of linear feature. Specifically, the legitimate types

of features for TYPE (and their associated 1-2 digit

integer) are as follows: contour (1), contour-carrying (6),

contour-supplemental (7), contour-amended (8), contour-

underwater (9), and line-closure (10).

The item CONFIDENCE is a 1-digit integer that indicates

the perceived accuracy in the value (that is, altitude)

and location of each contour. The legitimate entries for

this attribute (and their associated 1-digit integer) are

approximate (1), inferred (2), and speculative (3). Ini-

tially, all contours were assigned a confidence of 1 (that

is, approximate). If a contour had a minor code of 614

(that is, best estimate of contour value), then the confi-

dence was changed to inferred. If a contour is supple-

mental, its confidence was perceived to be lower. Thus,

a supplemental contour having an originally assigned con-

fidence of approximate or inferred was recoded as having

a confidence of inferred or speculative, respectively.

The item LOOK is a 1-digit integer that indicates the

type of cartographic symbol used to depict each contour.

The legitimate entries for this attribute (and their

associated 1-digit integer) are provided in the following

table:

CONFIDENCE      LOOK                  VALUE

----------      ----                  -----

APPROXIMATE     CONTINUOUS              1

APPROXIMATE     CONTINUOUS-HACHURE      4

INFERRED        LONG-DASH               2

INFERRED        LONG-DASH-HACHURE       5

SPECULATIVE     SHORT-DASH              3

SPECULATIVE     SHORT-DASH-HACHURE      6

The attributes TYPE, CONFIDENCE, and LOOK are combined

to provide the attribute CODE (as defined in section 3

above). Then, after a successful spot check of the recoding

of the data in the major and minor attribute pairs to the

added attribute items, the major and minor attribute pairs

and the items TYPE, CONFIDENCE, and LOOK were removed from

the line-feature attribute files.

To save computer disk space and drawing time, an AML

program was used to consolidate separate line segments

where possible (that is, unsplit) in each of the line-

feature digital quadrangle maps.

5. Revisions made to data (rev. number, date, description)

VERSION   COMPLETION DATE      DESCRIPTION

-------   ---------------      -----------

1.0      May, 1995           Completion of processing

described in section 4.

6. Reviews applied to data (date, person, description)

The digital data set received technical reviews from two

individuals in June 1995. The review materials provided

to each reviewer included the following: (1) the digital

map of the Great Bend 1:100,000-scale topographic quad-

rangle in Arc/Info export format; (2) the paper map of the

Great Bend 1:100,000-scale topographic quadrangle; (3) the

draft of the documentation that will accompanying each of

the 54 digital maps; and (4) paper copies of all Fortran

programs and AMLs used in the conversion of the DLG

digital data. For review purposes, the digital map in-

cluded the original DLG major/minor codes. The major/

minor codes are not included with the final product. The

reviewers' comments, and authors' responses, are sum-

marized below.

REVIEWER: Tom Mettille, Kansas Geological Survey

Mr. Mettille evaluated the digital data set in terms of

accuracy, appropriateness of methods used to create the

digital data set, attribute accuracy and completeness,

topological consistency, and documentation. He found

the digital map to be accurate with no discrepancies in

comparison to the source material. The methods used to

create the digital data set also were found to be

acceptable and logical. The attribution was found to be

accurate and complete.

Mr. Mettille identified a couple of possible problems

with regard to topology. Specifically, he found that

the digital map had 424 dangling nodes and 2,523 pseudo

nodes. A followup check by the authors determined that,

of the 424 dangling nodes in the digital map, 412 were

the result of the "neat" line being removed during the

processing (that is, the 412 dangling nodes were all

located around the outer edge of the digital map). The

remaining 12 dangling nodes were locations where the

contours had gaps because the contour spacing was so

close as to not allow enough room for the contour to

be placed between the neighboring contours. These gaps

exist in the source material (and were verified on the

paper quad map) and were not a result of processing.

The majority of the pseudo nodes were located on

closed contours and cannot be removed. In sum, the

authors believe that no additional revision is needed.

REVIEWER: Ron Zelt, U.S. Geological Survey, Nebraska

District

Mr. Zelt evaluated the digital data set in terms of

accuracy, appropriateness of methods used to create the

digital data set, attribute accuracy and completeness,

topological consistency, and documentation. He found

the digital data set to be acceptable in terms of

positional and contextual accuracy as compared to the

source material. Attribution was also found to be com-

plete and accurate. No topological problems were ident-

ified. Included in his topological review was a check

for any edgematching problems were the eight original

DLG files were joined to create the complete 30- by

60-minute digital map. No edgematching problems were

found.

Mr. Zelt identified several issues with regard to the

format and content of the documentation. Specifically,

the issues concern providing documentation that is

compliant with the Federal Geographic Data Committee's

guidelines. The authors have addressed all of the

issues that were identified. Also, Mr. Zelt noted that

the naming convention used for the digital data set is

no longer supported by the GIS software and required

modification. Specifically, the modification involved

renaming all digital maps by replacing the "." in the

name with a "_". For example, the digital data set

"TOPEKA.TOPO" was renamed as "TOPEKA_TOPO."

7. Related spatial and tabular data sets and programs

This digital map is one of a set of 54 digital maps

that together provide land-surface altitude information

for the entire State of Kansas. The complete list of

the 54 digital maps, as well as the 30- by 60-minute

topographic quadrangle maps that they represent, is pro-

vided below. The digital map names correspond to the

original names used for the DLGs.

DIGITAL MAP NAME             TOPOGRAPHIC MAP

----------------             ---------------

ATCHISN_TOPO                 Atchison, KS-MO

BELOIT_TOPO                  Beloit, KS

BLUERPD_TOPO                 Blue Rapids, KS

BNNYRES_TOPO                 Bonny Reservoir, CO-KS

BRLNGTN_TOPO                 Burlington, CO-KS

BUTLER_TOPO                  Butler, MO-KS

CFFVLLE_TOPO                 Coffeyville, KS

CHANUTE_TOPO                 Chanute, KS

CHYWELS_TOPO                 Cheyenne Wells, CO-KS

CLAYCEN_TOPO                 Clay Center, KS

CNCLGRV_TOPO                 Council Grove, KS

CNCRDIA_TOPO                 Concordia, KS

DODGE_TOPO                   Dodge City, KS

ELDRADO_TOPO                 El Dorado, KS

EMPORIA_TOPO                 Emporia, KS

GARNETT_TOPO                 Garnett, KS

GOODLND_TOPO                 Goodland, KS

GRTBEND_TOPO                 Great Bend, KS

HAYS_TOPO                    Hays, KS

HEALY_TOPO                   Healy, KS

HUGOTON_TOPO                 Hugoton, KS

HUTCHSN_TOPO                 Hutchinson, KS

JOPLIN_TOPO                  Joplin, MO-KS

KINSLEY_TOPO                 Kinsley, KS

KSCITY_TOPO                  Kansas City, MO-KS

LAMAR_TOPO                   Lamar, CO-KS

LARNED_TOPO                  Larned, KS

LAWRNCE_TOPO                 Lawrence, KS

LEOTI_TOPO                   Leoti, KS

LIBERAL_TOPO                 Liberal, KS-OK

MANHTN_TOPO                  Manhattan, KS

MDLODGE_TOPO                 Medicine Lodge, KS

NEVADA_TOPO                  Nevada, MO-KS

NORTON_TOPO                  Norton, KS

OAKLEY_TOPO                  Oakley, KS

OBERLIN_TOPO                 Oberlin, KS

OLATHE_TOPO                  Olathe, KS-MO

PLNVLLE_TOPO                 Plainville, KS

PRATT_TOPO                   Pratt, KS

PROTECT_TOPO                 Protection, KS-OK

RUSSELL_TOPO                 Russell, KS

SAINTJO_TOPO                 Saint Joseph, MO-KS

SALINA_TOPO                  Salina, KS

SCTCITY_TOPO                 Scott City, KS

SEDAN_TOPO                   Sedan, KS

SHSPGS_TOPO                  Sharon Springs, KS

SMITHCN_TOPO                 Smith Center, KS

SPRGFLD_TOPO                 Springfield, CO-KS-OK

STFRNCS_TOPO                 Saint Francis, KS

TOPEKA_TOPO                  Topeka, KS-MO

TWOBRES_TOPO                 Two Buttes Reservoir, CO-KS

ULYSSES_TOPO                 Ulysses, KS

WELNGTN_TOPO                 Wellington, KS

WICHITA_TOPO                 Wichita, KS

Additional digital maps developed for the major

aquifers in Kansas (that is, the glacial-drift, Dakota,

High Plains, and Ozark aquifers) provide information on

the geographic extent, altitude of the top and bottom of

the aquifers, and the potentiometric surface (that is,

altitude of the water table). Also, a digital map pro-

viding information on the extent of the alluvial aquifers

in Kansas is available. These coverages are available

from the State Data Access and Support Center.

8. Reference cited

U.S. Geological Survey, 1989, Digital Line Graphs from

1:100,000-Scale Maps: U.S. Geological Survey Data

Users Guide 2, 88 p.