Project Summary

In January 2011 – May 2011 Kucera International Inc. under contract with the State of Kansas performed ground GPS surveying services to establish lidar control points and FEMA type land cover checkpoints for an aerial lidar survey of a 15 county and one city area in central Kansas.  The ground survey work was performed by licensed land surveyors using Trimble 5700 and Ashtech ZExtreme dual frequency GPS receivers using static observations throughout the process.  Both the lidar control points and land cover checkpoints surveying were tied to the same group of monuments and CORS stations.

Base Point Observations

In all FEMA Block areas a “Base” point was set at the approximate center of block area and continuous GPS data was collected during control and checkpoint positioning.  These Base points were either existing NGS points, or when not available a new semi-permanent point was set.  A minimum of three (3), five (5) hour observations were made at each of these Base points as per USGS/FEMA requirements.  The Base point raw data was submitted through OPUS to obtain a CORS referenced position and orthometric height.  All local CORS stations within a fifty (50) radius of each Base area were utilized for the vector processing. Post processing was performed with Spectrum GPS vector software.  Only “Fixed” ambiguity resolved vectors were accepted.   All data was processed utilizing the NGS “Precise” obit ephemeris.   Processed vectors were entered into the StarNet Least Squares adjustment program holding the published CORS and existing control point values fixed.   The adjusted results are in the GRS-80 reference ellipsoid with incorporating the 2009 NGS produced Geoid.  Horizontal accuracy of 2nd Order Class I (1:50,000) and vertical NAVD88 accuracy at 2nd Order (+/- 0.06’) were achieved.   The geodetic positions were transformed to UTM Zone 14 grid in meters and  elevations output in NAVD88 orthometric heights.  A map showing base occupations is included below.

LIDAR Ground Control

The ground control consisted of 126 points spread through the project area at a spacing of approximately 15-25 miles.    The control points were marked in the field with PK nails or other semi-permanent monumentation and were photographed and field referenced.  The project bare earth lidar return was checked against and adjusted to these points.  A map showing the control point distribution and a listing of the control are included below.

| STATE OF KANSAS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2011 AERIAL LIDAR SURVEY PROJECT |  |  |  |  |  |  |  |
| GROUND CONTROL POINT LISTING |  |  |  |  |  |  |  |
| GPS Day | Station | Latitude | Longitude | Easting | Northing | H-Ell | H-MSL |
|  |  | (D M S Hem) | (D M S Hem) | (m) | (m) | (m) | (m) |
| 11 | KAN_84 | 37 59 56.62344N | 97 30 42.48139W | 630665.206 | 4206755.811 | 395.021 | 423.711 |
| 11 | KAN_85 | 37 56 39.80200N | 97 22 19.07301W | 643049.759 | 4200894.817 | 407.473 | 436.319 |
| 11 | KAN_87 | 38 07 52.07888N | 97 08 44.47542W | 662520.151 | 4221988.844 | 403.506 | 432.549 |
| 11 | KAN_88 | 38 01 45.40438N | 97 09 35.57002W | 661500.074 | 4210661.592 | 400.754 | 429.762 |
| 12 | KAN_073 | 38 13 58.58177N | 97 31 08.36066W | 629619.842 | 4232697.511 | 427.356 | 455.843 |
| 12 | KAN_079 | 38 10 25.74075N | 97 23 28.06272W | 640924.979 | 4226323.699 | 425.240 | 453.984 |
| 12 | KAN_080 | 38 10 06.78800N | 97 41 02.99634W | 615264.240 | 4225334.518 | 415.552 | 443.788 |
| 12 | KAN_082 | 38 03 07.05089N | 97 41 02.25554W | 615465.700 | 4212397.402 | 417.377 | 445.698 |
| 12 | KAN_083 | 38 06 21.87137N | 97 33 20.14508W | 626635.171 | 4218569.581 | 408.001 | 436.545 |
| 12 | KAN_086 | 38 03 48.90861N | 97 19 06.34045W | 647515.100 | 4214204.863 | 420.106 | 448.948 |
| 12 | KAN_122 | 38 11 47.99680N | 97 01 44.53227W | 672590.248 | 4229471.902 | 376.571 | 405.740 |
| 13 | KAN0020 | 38 15 38.57693N | 98 23 16.29575W | 553553.674 | 4234919.781 | 493.895 | 520.838 |
| 13 | KAN022 | 38 18 15.52298N | 98 14 57.76005W | 565629.802 | 4239846.417 | 480.933 | 508.146 |
| 13 | KAN_017 | 38 17 24.53236N | 98 32 32.28683W | 540025.928 | 4238107.391 | 515.762 | 542.315 |
| 14 | KAN_021 | 38 12 09.87744N | 98 12 20.65674W | 569542.190 | 4228608.358 | 472.142 | 499.574 |
| 14 | KAN_023 | 38 28 02.16461N | 98 19 51.43531W | 558366.061 | 4257873.426 | 497.469 | 524.237 |
| 14 | KAN_024 | 38 30 27.37025N | 98 10 06.36235W | 572503.743 | 4262464.651 | 508.921 | 536.069 |
| 14 | KAN_025 | 38 22 35.96300N | 98 06 18.88437W | 578154.281 | 4247986.047 | 502.837 | 530.277 |
| 14 | KAN_026 | 38 29 35.96820N | 97 55 28.14680W | 593792.510 | 4261100.661 | 476.041 | 503.691 |
| 14 | KAN_027 | 38 22 33.76283N | 97 55 28.09202W | 593945.711 | 4248086.805 | 467.288 | 495.032 |
| 14 | KAN_028 | 38 31 20.79831N | 98 01 01.07214W | 585692.954 | 4264241.727 | 480.286 | 507.750 |
| 14 | KAN_029 | 38 11 10.32089N | 97 55 19.65986W | 594395.852 | 4227023.610 | 452.285 | 480.199 |
| 14 | KAN_067 | 38 13 01.87220N | 98 01 56.44472W | 584707.242 | 4230355.324 | 460.228 | 487.950 |
| 14 | KAN_072 | 38 17 00.72614N | 97 49 58.67003W | 602068.552 | 4237918.657 | 451.063 | 479.006 |
| 14 | KAN_075 | 38 25 12.67371N | 97 47 02.86721W | 606140.020 | 4253137.232 | 435.115 | 463.006 |
| 15 | KAN_005 | 38 21 42.02841N | 98 55 19.14091W | 506815.856 | 4245947.682 | 558.408 | 584.414 |
| 15 | KAN_117 | 38 21 10.89339N | 97 20 05.58021W | 645493.949 | 4246297.214 | 428.540 | 457.332 |
| 15 | KAN_118 | 38 17 55.79469N | 97 07 56.45123W | 663314.040 | 4240621.863 | 393.203 | 422.290 |
| 15 | KAN_119 | 38 26 03.55461N | 97 12 20.39406W | 656610.048 | 4255530.254 | 393.464 | 422.524 |
| 15 | KAN_120 | 38 29 29.18512N | 96 57 29.40003W | 678073.171 | 4262318.950 | 403.642 | 432.939 |
| 15 | KAN_121 | 38 20 01.69240N | 96 55 47.29692W | 680939.831 | 4244880.225 | 391.566 | 420.858 |
| 16 | KAN_001 | 38 34 48.18140N | 98 58 16.14858W | 502512.664 | 4270176.398 | 577.835 | 603.813 |
| 16 | KAN_002 | 38 27 04.53647N | 99 00 47.34356W | 498852.489 | 4255885.280 | 558.313 | 584.272 |
| 16 | KAN_009 | 38 31 26.81442N | 98 46 22.71841W | 519789.303 | 4263993.702 | 541.774 | 567.766 |
| 16 | KAN_010 | 38 35 38.94482N | 98 50 19.43910W | 514043.812 | 4271753.019 | 577.368 | 603.334 |
| 16 | KAN_011 | 38 40 52.90531N | 98 46 31.04649W | 519544.954 | 4281441.968 | 564.480 | 590.459 |
| 16 | KAN_012 | 38 39 44.77684N | 98 27 42.85334W | 546815.439 | 4279455.392 | 556.475 | 582.768 |
| 16 | KAN_013 | 38 36 31.54376N | 98 38 10.71110W | 531665.415 | 4273424.670 | 545.926 | 571.982 |
| 16 | KAN_014 | 38 31 32.47376N | 98 32 34.64329W | 539839.162 | 4264242.693 | 524.346 | 550.587 |
| 16 | KAN_015 | 38 28 05.98615N | 98 35 26.76407W | 535699.867 | 4257858.539 | 522.916 | 549.121 |
| 17 | KAN_154 | 37 43 46.81750N | 100 06 21.65005W | 402538.028 | 4176397.884 | 754.297 | 780.273 |
| 17 | KAN_155 | 37 45 18.80653N | 100 04 04.03572W | 405938.968 | 4179193.879 | 741.637 | 767.611 |
| 17 | KAN_157 | 37 43 04.83483N | 99 54 23.55573W | 420103.272 | 4174915.062 | 716.426 | 742.615 |
| 17 | KAN_158 | 37 41 28.93730N | 99 54 23.78350W | 420069.092 | 4171959.606 | 724.177 | 750.401 |
| 18 | KAN_030 | 38 34 08.51956N | 98 19 03.17018W | 559451.902 | 4269174.256 | 519.584 | 546.276 |
| 18 | KAN_031 | 38 37 12.38113N | 98 12 06.56446W | 569484.070 | 4274922.790 | 463.854 | 490.813 |
| 18 | KAN_032 | 38 43 33.31217N | 98 24 23.75005W | 551581.635 | 4286529.459 | 494.258 | 520.667 |
| 19 | KAN_034 | 38 40 03.45194N | 98 05 00.92207W | 579724.265 | 4280292.058 | 440.735 | 467.936 |
| 19 | KAN_036 | 38 50 50.32023N | 98 16 52.18713W | 562379.261 | 4300078.216 | 518.597 | 545.287 |
| 19 | KAN_037 | 38 50 29.35899N | 98 08 54.43255W | 573901.733 | 4299531.082 | 480.779 | 507.728 |
| 19 | KAN_038 | 38 46 10.18070N | 98 02 37.69646W | 583067.457 | 4291631.725 | 460.362 | 487.564 |
| 19 | KAN_039 | 38 38 19.68864N | 97 55 26.33769W | 593647.311 | 4277244.451 | 443.927 | 471.480 |
| 19 | KAN_040 | 38 50 30.80089N | 97 57 56.22984W | 589768.882 | 4299739.327 | 428.627 | 455.954 |
| 19 | KAN_044 | 38 56 37.50626N | 97 55 42.92248W | 592849.866 | 4311080.388 | 398.892 | 426.236 |
| 19 | KAN_091 | 38 40 16.42643N | 97 45 35.60523W | 607880.308 | 4281023.169 | 383.215 | 411.015 |
| 20 | KAN_042 | 39 01 51.33883N | 98 19 55.56955W | 557809.490 | 4320420.837 | 410.414 | 437.173 |
| 20 | KAN_043 | 39 01 49.04693N | 98 05 54.24143W | 578038.466 | 4320524.670 | 388.749 | 415.903 |
| 20 | KAN_047 | 39 05 22.23684N | 98 12 24.12669W | 568606.897 | 4327009.192 | 448.665 | 475.642 |
| 20 | KAN_049 | 39 00 59.19752N | 97 58 06.65252W | 589298.464 | 4319107.432 | 385.044 | 412.317 |
| 20 | KAN_077 | 38 31 18.70083N | 97 37 46.67761W | 619459.572 | 4264609.035 | 400.491 | 428.495 |
| 20 | KAN_078 | 38 27 51.11485N | 97 31 07.96846W | 629217.695 | 4258359.872 | 428.593 | 456.870 |
| 20 | KAN_090 | 38 50 52.22129N | 97 45 41.13576W | 607481.378 | 4300620.120 | 369.003 | 396.639 |
| 20 | KAN_093 | 38 44 24.17101N | 97 32 46.31592W | 626349.495 | 4288933.260 | 371.138 | 399.219 |
| 20 | KAN_094 | 38 37 47.11008N | 97 36 47.44398W | 620713.230 | 4276603.282 | 369.427 | 397.475 |
| 20 | KAN_095 | 39 00 58.27965N | 97 47 08.08186W | 605136.589 | 4319274.578 | 372.149 | 399.558 |
| 20 | KAN_199 | 38 32 11.11213N | 97 37 47.10682W | 619425.107 | 4266224.448 | 403.579 | 431.586 |
| 21 | KAN_048 | 39 08 22.01864N | 98 01 57.67661W | 583597.398 | 4332697.098 | 396.474 | 423.658 |
| 21 | KAN_050 | 39 12 16.46875N | 97 55 47.17230W | 592406.722 | 4340024.403 | 426.728 | 453.878 |
| 21 | KAN_097 | 39 07 49.69287N | 97 42 11.92680W | 612077.870 | 4332055.618 | 372.691 | 399.943 |
| 21 | KAN_098_2 | 39 03 34.10678N | 97 38 48.34643W | 617083.279 | 4324247.865 | 346.960 | 374.332 |
| 21 | KAN_099 | 39 17 31.38935N | 97 39 50.73814W | 615203.483 | 4350037.385 | 407.883 | 434.872 |
| 21 | KAN_54 | 39 17 29.93463N | 97 55 47.57942W | 592282.755 | 4349687.720 | 453.911 | 480.949 |
| 21 | KAN_58 | 39 22 43.27212N | 97 58 50.51001W | 587791.715 | 4359296.698 | 386.744 | 413.691 |
| 21 | KAN_92 | 38 55 04.85205N | 97 38 57.53985W | 617095.207 | 4308545.762 | 346.698 | 374.336 |
| 21 | KAN_98 | 39 03 34.10688N | 97 38 48.34628W | 617083.282 | 4324247.868 | 346.945 | 374.317 |
| 22 | KAN_041 | 39 05 19.25427N | 98 26 34.07987W | 548188.799 | 4326765.511 | 480.635 | 507.204 |
| 22 | KAN_045 | 39 14 03.15714N | 98 28 17.24274W | 545616.387 | 4342900.757 | 472.690 | 499.158 |
| 22 | KAN_046 | 39 14 03.71308N | 98 17 30.00021W | 561133.277 | 4343023.834 | 437.262 | 464.012 |
| 22 | KAN_051 | 39 23 36.88260N | 98 23 45.48419W | 552013.390 | 4360627.896 | 447.395 | 473.877 |
| 22 | KAN_052 | 39 23 09.67928N | 98 14 49.84640W | 564832.716 | 4359885.560 | 441.961 | 468.580 |
| 22 | KAN_053 | 39 17 30.47381N | 98 07 01.43314W | 576140.490 | 4349530.100 | 431.311 | 458.245 |
| 22 | KAN_059 | 39 34 02.05439N | 98 03 48.11794W | 580453.896 | 4380145.496 | 429.159 | 455.752 |
| 22 | KAN_063 | 39 38 21.67115N | 97 57 13.54768W | 589775.446 | 4388253.127 | 413.536 | 440.165 |
| 22 | KAN_066 | 39 53 09.87367N | 98 12 45.63249W | 567317.700 | 4415409.503 | 501.874 | 527.921 |
| 22 | KAN_070 | 39 59 15.67413N | 98 09 28.51629W | 571892.959 | 4426730.142 | 488.941 | 514.977 |
| 22 | KAN_071 | 39 58 19.75237N | 97 57 08.24178W | 589469.463 | 4425192.070 | 472.784 | 498.964 |
| 23 | KAN_060 | 39 49 41.85450N | 98 29 08.85871W | 544002.311 | 4408826.162 | 555.833 | 581.757 |
| 23 | KAN_061 | 39 38 23.83003N | 98 18 41.19805W | 559082.736 | 4388023.236 | 441.077 | 467.373 |
| 24 | KAN_101 | 39 12 16.96909N | 97 30 00.73530W | 629497.889 | 4340565.741 | 429.682 | 456.747 |
| 24 | KAN_102 | 39 08 47.44238N | 97 22 10.58111W | 640890.972 | 4334301.049 | 399.283 | 426.537 |
| 24 | KAN_103 | 39 00 06.48315N | 97 28 25.48032W | 632161.072 | 4318084.388 | 346.221 | 373.757 |
| 24 | KAN_104 | 39 06 45.76604N | 97 10 01.45003W | 658470.127 | 4330883.930 | 344.173 | 372.293 |
| 24 | KAN_105 | 39 07 04.45744N | 96 58 22.96777W | 675233.316 | 4331816.801 | 355.212 | 384.296 |
| 24 | KAN_106 | 39 01 55.51925N | 97 15 34.57525W | 650640.672 | 4321778.575 | 343.663 | 371.720 |
| 25 | KAN_111 | 38 49 12.81003N | 97 03 23.26000W | 668726.484 | 4298621.390 | 370.697 | 400.026 |
| 25 | KAN_112 | 38 51 45.79060N | 96 55 35.34085W | 679904.302 | 4303585.692 | 322.118 | 351.594 |
| 25 | KAN_113 | 38 42 59.90943N | 96 55 40.71626W | 680142.189 | 4287370.588 | 378.651 | 408.062 |
| 25 | KAN_114 | 38 35 42.34055N | 96 56 54.13054W | 678670.907 | 4273841.348 | 421.637 | 450.984 |
| 25 | KAN_125 | 38 46 06.34745N | 96 47 50.69187W | 691356.154 | 4293383.266 | 407.966 | 437.477 |
| 25 | KAN_126 | 38 50 30.87009N | 96 35 43.86200W | 708684.117 | 4301980.411 | 423.173 | 452.968 |
| 25 | KAN_127 | 38 51 04.51481N | 96 31 12.56303W | 715197.342 | 4303192.676 | 430.218 | 460.118 |
| 25 | KAN_128 | 38 48 40.99351N | 96 24 04.90335W | 725633.561 | 4299054.559 | 417.071 | 447.153 |
| 26 | KAN_109 | 38 45 16.17280N | 97 16 46.15700W | 649500.535 | 4290938.270 | 350.352 | 379.219 |
| 26 | KAN_115 | 38 34 18.71811N | 97 07 58.30944W | 662654.887 | 4270920.616 | 422.004 | 451.171 |
| 26 | KAN_116 | 38 31 19.74828N | 97 18 37.31016W | 647292.374 | 4265104.343 | 407.122 | 436.012 |
| 26 | KAN_130 | 38 37 24.85552N | 96 42 29.40695W | 699512.797 | 4277496.666 | 438.011 | 467.587 |
| 26 | KAN_131 | 38 34 47.65405N | 96 33 13.15020W | 713094.939 | 4272997.644 | 373.217 | 403.058 |
| 26 | KAN_132 | 38 38 03.68053N | 96 22 39.73396W | 728251.000 | 4279464.325 | 353.015 | 383.165 |
| 26 | KAN_133 | 38 30 09.97993N | 96 41 09.93465W | 701772.499 | 4264138.287 | 378.191 | 407.851 |
| 26 | KAN_148 | 38 28 04.89754N | 96 19 03.99776W | 734006.994 | 4261154.413 | 354.042 | 384.378 |
| 27 | KAN_136 | 38 18 16.04244N | 96 37 43.26986W | 707345.196 | 4242256.365 | 411.367 | 441.106 |
| 27 | KAN_138 | 38 07 47.94626N | 96 33 17.50420W | 714312.686 | 4223061.832 | 382.600 | 412.422 |
| 27 | KAN_139 | 38 13 26.99764N | 96 30 26.77660W | 718188.749 | 4233624.749 | 364.804 | 394.735 |
| 27 | KAN_140 | 38 06 39.68158N | 96 21 47.87321W | 731165.565 | 4221417.470 | 453.180 | 483.323 |
| 27 | KAN_141 | 38 11 13.91640N | 96 20 55.88166W | 732190.136 | 4229907.864 | 356.905 | 387.137 |
| 27 | KAN_147 | 38 41 34.63648N | 96 19 00.93555W | 733351.636 | 4286121.362 | 400.430 | 430.643 |
| 27 | KAN_152 | 38 34 13.92176N | 96 05 51.28461W | 752862.240 | 4273115.053 | 350.385 | 381.106 |
| 27 | KAN_153 | 38 43 50.59635N | 95 57 52.78766W | 763855.523 | 4291270.203 | 326.381 | 357.254 |
| 28 | KAN_145 | 38 19 55.74129N | 96 03 39.61763W | 756893.982 | 4246757.296 | 311.297 | 342.135 |
| 28 | KAN_149 | 38 17 20.48311N | 96 12 31.80565W | 744114.571 | 4241569.490 | 343.420 | 373.961 |
| 28 | KAN_150 | 38 12 00.79663N | 95 58 49.10291W | 764427.646 | 4232341.796 | 315.412 | 346.238 |
| 28 | KAN_151 | 38 31 08.32162N | 95 57 18.85971W | 765455.318 | 4267794.210 | 298.888 | 329.862 |
| 29 | KAN_123 | 38 15 43.25691N | 96 50 12.53462W | 689254.708 | 4237099.840 | 352.837 | 382.254 |
| 29 | KAN_134 | 38 24 27.32184N | 96 44 44.93316W | 696822.739 | 4253445.539 | 356.020 | 385.585 |
| 29 | KAN_137 | 38 10 23.61592N | 96 45 17.44933W | 696665.852 | 4227417.287 | 399.174 | 428.708 |

FEMA Land Cover Checkpoints

For the land cover accuracy checkpoint survey the project area was divided into 12 contiguous area “blocks” (FEMA01 – FEMA12), each covering approximately 1000 square miles.  For each significant land cover type within each block, at least 20 ground checkpoints spread through the cover type were surveyed.    The land covers surveyed in each block were brushlands/low trees (BR), high grass/weeds/crops (HG), and bare earth/low grass/pavement (BE).  In FEMA Block 6 (Saline County/City of Salina) urban (UR) land cover points were also surveyed.  A total of 760 land cover checkpoints were surveyed and used for calculation of the fundamental (FNA), supplemental (SVA) and consolidated (CVA) vertical accuracies of the lidar return, as documented in Kucera’s lidar accuracy report.

All GPS observation data was process using the Novatel Waypoint Grafnet network adjustment program, with the results being checked against an NGS OPUS solution.  All processed ground point elevations were reported to within 2cm accuracy.

A map of the land cover control distribution and a land cover control listing are included below.

| STATE OF KANSAS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2011 AERIAL LIDAR SURVEY PROJECT |  |  |  |  |  |  |  |  |
| FEMA CHECK POINT LISTING |  |  |  |  |  |  |  |  |
| FEMA BLOCK | Point Name | Latitude | Longitude | Easting | Northing | H-Ell | H-MSL |  |
|  |  | (D M S Hem) | (D M S Hem) | (m) | (m) | (m) | (m) |  |
| FEMA01 | CBE02 | 38 25 48.57366 | 96 22 31.04444 | 729108.345 | 4256806.893 | 369.004 | 338.765 |  |
| FEMA01 | CBE14 | 38 21 19.80123 | 96 22 27.97424 | 729418.785 | 4248522.880 | 380.878 | 350.616 |  |
| FEMA01 | CBE15 | 38 20 50.18059 | 96 22 24.45142 | 729530.298 | 4247612.128 | 376.570 | 346.308 |  |
| FEMA01 | CBE16 | 38 15 32.87094 | 96 22 29.96240 | 729674.488 | 4237825.925 | 464.454 | 434.241 |  |
| FEMA01 | CBE17 | 38 05 24.09575 | 96 22 43.55571 | 729875.104 | 4219048.850 | 492.879 | 462.775 |  |
| FEMA01 | CBR02 | 38 25 49.62747 | 96 22 27.08065 | 729203.545 | 4256842.121 | 365.689 | 335.448 |  |
| FEMA01 | CBR16 | 38 21 14.10395 | 96 22 31.07919 | 729348.406 | 4248345.092 | 375.223 | 344.963 |  |
| FEMA01 | CBR17 | 38 05 25.48650 | 96 21 51.52437 | 731141.603 | 4219127.627 | 481.398 | 451.262 |  |
| FEMA01 | CHG02 | 38 26 44.48843 | 96 22 22.46254 | 729267.292 | 4258536.664 | 375.802 | 345.569 |  |
| FEMA01 | CHG15 | 38 20 51.52410 | 96 22 24.40489 | 729530.249 | 4247653.580 | 375.432 | 345.170 |  |
| FEMA01 | CHG16 | 38 15 35.79603 | 96 22 29.32123 | 729687.513 | 4237916.545 | 464.151 | 433.937 |  |
| FEMA01 | CHG17 | 38 05 21.86748 | 96 22 46.35649 | 729808.804 | 4218978.230 | 499.609 | 469.507 |  |
| FEMA01 | LBE24 | 38 30 29.00060 | 96 15 54.64538 | 738464.536 | 4265732.263 | 352.344 | 321.906 |  |
| FEMA01 | MHG01 | 38 29 35.48646 | 96 23 23.17799 | 727645.697 | 4263766.635 | 409.273 | 379.099 |  |
| FEMA01 | LBE01 | 38 25 13.71668 | 96 12 04.72404 | 744330.039 | 4256179.385 | 355.740 | 325.123 |  |
| FEMA01 | LBE03 | 38 25 15.72652 | 96 10 44.83318 | 746265.889 | 4256300.450 | 338.486 | 307.825 |  |
| FEMA01 | LBE04 | 38 24 28.98435 | 96 09 44.10736 | 747783.169 | 4254904.583 | 345.441 | 314.748 |  |
| FEMA01 | LBE05 | 38 24 05.58647 | 96 10 17.40000 | 746997.657 | 4254158.376 | 349.072 | 318.396 |  |
| FEMA01 | LBE06 | 38 23 54.88894 | 96 11 23.15675 | 745412.367 | 4253779.761 | 345.616 | 314.977 |  |
| FEMA01 | LBE07 | 38 25 35.36275 | 96 13 02.97424 | 742897.031 | 4256803.949 | 367.742 | 337.159 |  |
| FEMA01 | LBE10 | 38 39 44.33900 | 96 16 47.93046 | 736666.376 | 4282815.552 | 434.909 | 404.632 |  |
| FEMA01 | LBE16 | 38 31 04.11036 | 95 57 28.35112 | 765229.705 | 4267656.754 | 330.233 | 299.262 |  |
| FEMA01 | LBE22 | 38 43 53.63545 | 96 15 38.65731 | 738111.015 | 4290551.602 | 445.180 | 414.888 |  |
| FEMA01 | LBR01 | 38 38 40.52787 | 96 15 39.68540 | 738374.870 | 4280897.308 | 430.842 | 400.519 |  |
| FEMA01 | LBR05 | 38 34 09.01211 | 96 15 38.88976 | 738644.019 | 4272526.757 | 396.693 | 366.299 |  |
| FEMA01 | LBR07 | 38 25 17.82660 | 96 11 27.59565 | 745226.721 | 4256333.505 | 351.418 | 320.781 |  |
| FEMA01 | LBR08 | 38 25 13.70163 | 96 12 19.69483 | 743966.938 | 4256167.898 | 360.409 | 329.801 |  |
| FEMA01 | LBR09 | 38 23 59.85860 | 96 11 29.45088 | 745254.988 | 4253928.324 | 345.712 | 315.076 |  |
| FEMA01 | LBR10 | 38 24 30.26459 | 96 08 52.08924 | 749043.870 | 4254983.014 | 336.508 | 305.789 |  |
| FEMA01 | LHG01 | 38 39 08.66257 | 96 15 40.09866 | 738338.961 | 4281764.437 | 433.531 | 403.215 |  |
| FEMA01 | LHG04 | 38 31 16.94703 | 95 59 29.46961 | 762282.860 | 4267955.974 | 358.542 | 327.623 |  |
| FEMA01 | LHG05 | 38 34 38.07855 | 96 15 40.66807 | 738574.246 | 4273421.618 | 399.804 | 369.418 |  |
| FEMA01 | LHG06 | 38 31 14.42720 | 96 05 50.13757 | 753064.865 | 4267581.788 | 388.101 | 357.347 |  |
| FEMA01 | LHG07 | 38 29 46.12329 | 96 15 42.12763 | 738807.142 | 4264419.352 | 347.307 | 316.854 |  |
| FEMA01 | LHG08 | 38 30 23.85876 | 96 11 39.29517 | 744655.322 | 4265760.104 | 358.613 | 328.021 |  |
| FEMA01 | LHG22 | 38 43 29.46667 | 96 16 45.43977 | 736520.380 | 4289758.326 | 443.675 | 413.412 |  |
| FEMA01 | LBE09 | 38 27 06.93406 | 95 58 07.58396 | 764520.338 | 4260312.645 | 360.012 | 329.056 |  |
| FEMA01 | LBE11 | 38 33 22.92966 | 95 59 13.87406 | 762533.239 | 4271852.704 | 360.739 | 329.828 |  |
| FEMA01 | LBE12 | 38 36 29.89175 | 95 59 28.15147 | 761998.616 | 4277605.903 | 344.868 | 313.989 |  |
| FEMA01 | LBE14 | 38 18 28.70420 | 95 57 30.12376 | 765955.716 | 4244364.513 | 331.314 | 300.362 |  |
| FEMA01 | LBE15 | 38 15 58.84934 | 96 09 44.10446 | 748266.822 | 4239176.822 | 368.562 | 337.956 |  |
| FEMA01 | LBE17 | 38 11 12.67401 | 96 11 54.09342 | 745374.395 | 4230257.681 | 346.216 | 315.715 |  |
| FEMA01 | LBE18 | 38 11 10.43477 | 96 05 06.97185 | 755283.192 | 4230494.422 | 371.893 | 341.228 |  |
| FEMA01 | LBE21 | 38 43 32.62639 | 95 57 59.68964 | 763707.199 | 4290710.601 | 362.110 | 331.239 |  |
| FEMA01 | LBR02 | 38 36 29.83290 | 95 59 12.28148 | 762382.615 | 4277616.690 | 338.795 | 307.909 |  |
| FEMA01 | LBR03 | 38 41 42.53853 | 96 04 06.84745 | 754947.930 | 4287027.262 | 362.377 | 331.671 |  |
| FEMA01 | LBR04 | 38 32 42.98152 | 95 59 14.44945 | 762559.708 | 4270620.545 | 332.500 | 301.584 |  |
| FEMA01 | LBR06 | 38 29 00.29037 | 95 58 05.34610 | 764459.440 | 4263809.471 | 372.863 | 341.903 |  |
| FEMA01 | LBR14 | 38 22 25.80426 | 95 59 24.56580 | 762937.053 | 4251583.613 | 333.585 | 302.652 |  |
| FEMA01 | LBR15 | 38 18 41.41118 | 95 57 48.93125 | 765485.928 | 4244741.255 | 326.810 | 295.861 |  |
| FEMA01 | LBR17 | 38 11 08.32194 | 96 00 29.24906 | 762043.410 | 4230644.828 | 375.453 | 344.677 |  |
| FEMA01 | LBR19 | 38 11 28.07747 | 96 12 39.97309 | 744243.702 | 4230698.866 | 367.280 | 336.794 |  |
| FEMA01 | LHG02 | 38 41 43.78286 | 96 05 45.66179 | 752559.058 | 4286989.547 | 376.797 | 346.145 |  |
| FEMA01 | LHG03 | 38 36 28.90230 | 95 59 28.05513 | 762001.947 | 4277575.472 | 344.801 | 313.922 |  |
| FEMA01 | LHG09 | 38 30 10.48613 | 95 58 07.45662 | 764336.955 | 4265972.090 | 336.902 | 305.944 |  |
| FEMA01 | LHG10 | 38 24 42.49904 | 96 02 34.66735 | 758187.810 | 4255648.841 | 354.831 | 323.959 |  |
| FEMA01 | LHG14 | 38 21 41.03029 | 96 01 04.81050 | 760548.600 | 4250124.107 | 326.247 | 295.343 |  |
| FEMA01 | LHG15 | 38 18 44.15875 | 95 57 52.10574 | 765406.027 | 4244823.431 | 326.820 | 295.872 |  |
| FEMA01 | LHG16 | 38 15 34.90805 | 96 11 00.43179 | 746434.050 | 4238381.967 | 368.327 | 337.761 |  |
| FEMA01 | LHG17 | 38 16 02.69717 | 96 09 48.57816 | 748154.440 | 4239292.115 | 371.723 | 341.119 |  |
| FEMA01 | LHG21 | 38 43 27.80042 | 95 58 55.88114 | 762354.900 | 4290516.922 | 385.299 | 354.456 |  |
| FEMA01 | LBE19 | 38 13 50.63927 | 96 18 50.93551 | 735090.167 | 4234827.099 | 386.800 | 356.470 |  |
| FEMA01 | LBE20 | 38 19 05.71436 | 96 16 42.27261 | 737932.957 | 4244632.183 | 371.024 | 340.597 |  |
| FEMA01 | LBE25 | 38 19 55.59459 | 96 11 14.76212 | 745841.002 | 4246408.398 | 368.800 | 338.185 |  |
| FEMA01 | LBE26 | 38 11 12.73317 | 96 19 43.86252 | 733943.577 | 4229921.741 | 365.893 | 335.617 |  |
| FEMA01 | LBE23 | 38 22 49.59105 | 96 19 06.40412 | 734231.622 | 4251431.811 | 356.518 | 326.150 |  |
| FEMA01 | LBR22 | 38 19 04.74532 | 96 16 32.49593 | 738171.302 | 4244609.310 | 375.382 | 344.950 |  |
| FEMA01 | LBR23 | 38 22 51.04686 | 96 19 06.53639 | 734227.107 | 4251476.601 | 352.114 | 321.746 |  |
| FEMA01 | LHG12 | 38 19 05.02844 | 96 16 52.11210 | 737694.591 | 4244603.995 | 364.539 | 334.117 |  |
| FEMA01 | LHG13 | 38 20 31.95586 | 96 10 46.90929 | 746483.140 | 4247550.072 | 347.731 | 317.094 |  |
| FEMA01 | LHG18 | 38 11 17.43462 | 96 19 19.34357 | 734535.991 | 4230083.912 | 371.019 | 340.727 |  |
| FEMA01 | LHG23 | 38 22 52.61430 | 96 19 13.86618 | 734047.826 | 4251519.754 | 346.408 | 316.045 |  |
| FEMA01 | BR31 | 38 24 45.25481 | 96 05 35.90647 | 753788.551 | 4255593.902 | 347.426 | 316.620 |  |
| FEMA01 | BR32 | 38 24 50.92309 | 96 05 44.89396 | 753565.023 | 4255761.788 | 351.894 | 321.091 |  |
| FEMA02 | CBE03 | 38 24 09.57762 | 96 32 21.76414 | 714864.600 | 4253359.566 | 369.469 | 339.516 |  |
| FEMA02 | CBE04 | 38 29 00.09212 | 96 33 27.08776 | 713042.373 | 4262273.625 | 395.881 | 366.000 |  |
| FEMA02 | CBE05 | 38 26 58.61748 | 96 40 24.57620 | 703020.443 | 4258266.621 | 379.715 | 350.013 |  |
| FEMA02 | CBE06 | 38 30 10.61999 | 96 41 09.80028 | 701775.258 | 4264158.101 | 407.739 | 378.078 |  |
| FEMA02 | CBE08 | 38 24 49.34511 | 96 46 45.01948 | 693893.471 | 4254053.755 | 393.375 | 363.864 |  |
| FEMA02 | CBR03 | 38 24 01.48539 | 96 32 36.43774 | 714515.276 | 4253100.599 | 360.160 | 330.214 |  |
| FEMA02 | CBR04 | 38 29 08.57605 | 96 33 27.46282 | 713026.341 | 4262534.935 | 404.584 | 374.705 |  |
| FEMA02 | CBR05 | 38 26 29.56176 | 96 40 14.70968 | 703282.292 | 4257376.932 | 376.655 | 346.946 |  |
| FEMA02 | CBR06 | 38 30 11.37610 | 96 41 23.75118 | 701436.710 | 4264172.916 | 422.676 | 393.022 |  |
| FEMA02 | CBR08 | 38 24 50.12603 | 96 47 13.84890 | 693193.644 | 4254061.011 | 394.710 | 365.212 |  |
| FEMA02 | CHG03 | 38 23 59.48987 | 96 32 35.57601 | 714537.821 | 4253039.637 | 361.457 | 331.510 |  |
| FEMA02 | CHG04 | 38 28 57.19681 | 96 33 25.57257 | 713081.458 | 4262185.340 | 391.627 | 361.745 |  |
| FEMA02 | CHG05 | 38 26 51.64715 | 96 40 16.90254 | 703211.909 | 4258056.440 | 377.184 | 347.478 |  |
| FEMA02 | CHG06 | 38 29 36.41134 | 96 41 16.69625 | 701634.707 | 4263099.304 | 405.926 | 376.263 |  |
| FEMA02 | CHG08 | 38 24 49.54525 | 96 46 29.11395 | 694279.106 | 4254069.230 | 394.447 | 364.928 |  |
| FEMA02 | BE10 | 38 45 49.54453 | 96 39 41.67852 | 703172.031 | 4293158.262 | 411.989 | 382.332 |  |
| FEMA02 | BE11 | 38 50 30.75034 | 96 40 03.71234 | 702418.924 | 4301814.198 | 441.065 | 411.352 |  |
| FEMA02 | BE12 | 38 44 21.08911 | 96 42 17.35683 | 699482.785 | 4290336.049 | 443.639 | 414.056 |  |
| FEMA02 | BE13 | 38 42 37.64196 | 96 38 53.60045 | 704484.557 | 4287271.739 | 450.758 | 421.102 |  |
| FEMA02 | BE14 | 38 35 50.75953 | 96 29 55.30747 | 717829.706 | 4275072.159 | 383.015 | 353.071 |  |
| FEMA02 | BR09 | 38 45 55.15755 | 96 39 53.62855 | 702879.170 | 4293323.939 | 411.756 | 382.103 |  |
| FEMA02 | BR10 | 38 50 22.14044 | 96 40 04.18974 | 702414.194 | 4301548.461 | 441.997 | 412.287 |  |
| FEMA02 | BR11 | 38 43 59.95606 | 96 42 18.71000 | 699466.449 | 4289683.712 | 439.020 | 409.440 |  |
| FEMA02 | BR12 | 38 42 38.02604 | 96 38 58.85593 | 704357.305 | 4287280.320 | 453.297 | 423.644 |  |
| FEMA02 | BR13 | 38 35 50.08317 | 96 29 48.29599 | 717999.911 | 4275055.932 | 381.847 | 351.900 |  |
| FEMA02 | HG09 | 38 45 59.19918 | 96 39 51.35521 | 702930.859 | 4293449.942 | 417.517 | 387.862 |  |
| FEMA02 | HG10 | 38 51 25.60066 | 96 40 04.08584 | 702366.717 | 4303505.003 | 438.908 | 409.181 |  |
| FEMA02 | HG11 | 38 43 48.11070 | 96 42 16.99825 | 699516.942 | 4289319.564 | 444.451 | 414.871 |  |
| FEMA02 | HG12 | 38 42 36.45037 | 96 37 54.54319 | 705912.065 | 4287271.771 | 441.655 | 411.971 |  |
| FEMA02 | HG13 | 38 35 53.78741 | 96 30 02.86080 | 717644.418 | 4275160.530 | 381.180 | 351.240 |  |
| FEMA02 | MBE01 | 38 29 34.83300 | 96 24 35.77311 | 725887.314 | 4263696.771 | 391.818 | 361.682 |  |
| FEMA02 | MBE13 | 38 33 30.93404 | 96 44 42.46687 | 696471.936 | 4270205.480 | 409.514 | 379.967 |  |
| FEMA02 | MBE14 | 38 33 04.63779 | 96 36 56.28215 | 707777.659 | 4269679.719 | 447.593 | 417.851 |  |
| FEMA02 | MBE15 | 38 35 42.47503 | 96 41 24.30085 | 701166.805 | 4274379.885 | 463.610 | 434.003 |  |
| FEMA02 | MBE16 | 38 35 40.46474 | 96 33 57.47933 | 711979.019 | 4274597.254 | 426.664 | 396.848 |  |
| FEMA02 | MBR01 | 38 35 59.13320 | 96 24 08.77749 | 726206.459 | 4275563.206 | 366.623 | 336.499 |  |
| FEMA02 | MBR13 | 38 33 33.26891 | 96 44 45.39390 | 696399.318 | 4270275.722 | 408.425 | 378.880 |  |
| FEMA02 | MBR14 | 38 33 05.51929 | 96 36 57.29882 | 707752.342 | 4269706.256 | 446.216 | 416.475 |  |
| FEMA02 | MBR15 | 38 35 45.05332 | 96 41 22.97049 | 701196.992 | 4274460.181 | 462.566 | 432.958 |  |
| FEMA02 | MBR16 | 38 35 31.17531 | 96 33 57.78702 | 711979.168 | 4274310.670 | 429.093 | 399.277 |  |
| FEMA02 | MHG02 | 38 31 45.71796 | 96 25 12.45133 | 724885.406 | 4267706.955 | 390.829 | 360.719 |  |
| FEMA02 | MHG13 | 38 33 36.30863 | 96 44 47.13086 | 696354.975 | 4270368.400 | 409.253 | 379.709 |  |
| FEMA02 | MHG14 | 38 32 57.68180 | 96 36 56.15115 | 707786.396 | 4269465.355 | 452.474 | 422.731 |  |
| FEMA02 | MHG15 | 38 35 46.69873 | 96 41 22.85180 | 701198.586 | 4274510.979 | 462.183 | 432.575 |  |
| FEMA02 | MHG16 | 38 35 26.30417 | 96 33 18.35185 | 712937.329 | 4274185.851 | 409.466 | 379.630 |  |
| FEMA02 | BE18 | 38 27 33.54644 | 96 33 05.38139 | 713639.344 | 4259619.494 | 388.238 | 358.332 |  |
| FEMA02 | HG18 | 38 27 34.08904 | 96 33 05.86617 | 713627.148 | 4259635.909 | 387.857 | 357.952 |  |
| FEMA02 | HG19 | 38 23 54.86819 | 96 37 15.88615 | 707741.143 | 4252718.801 | 366.358 | 336.559 |  |
| FEMA02 | MBE03 | 38 50 25.56574 | 96 30 06.60022 | 716820.463 | 4302035.208 | 453.803 | 423.879 |  |
| FEMA02 | MBE04 | 38 50 25.54828 | 96 35 49.38700 | 708555.220 | 4301812.830 | 452.605 | 422.814 |  |
| FEMA02 | MBE05 | 38 47 26.95756 | 96 44 17.54844 | 696438.992 | 4295993.984 | 448.706 | 419.123 |  |
| FEMA02 | MBE06 | 38 52 05.52417 | 96 47 29.44345 | 691601.306 | 4304468.796 | 373.751 | 344.137 |  |
| FEMA02 | MBE07 | 38 47 52.51026 | 96 54 30.94605 | 681621.089 | 4296429.344 | 408.382 | 378.929 |  |
| FEMA02 | MBE08 | 38 44 20.70407 | 96 50 46.94690 | 687178.857 | 4290025.101 | 430.431 | 400.963 |  |
| FEMA02 | MBE09 | 38 41 58.58330 | 96 48 45.41556 | 690217.929 | 4285713.292 | 453.105 | 423.626 |  |
| FEMA02 | MBR02 | 38 47 51.59437 | 96 30 03.93168 | 717014.675 | 4297289.927 | 437.267 | 407.349 |  |
| FEMA02 | MBR03 | 38 50 28.52670 | 96 35 32.29077 | 708965.019 | 4301915.516 | 451.475 | 421.677 |  |
| FEMA02 | MBR04 | 38 47 30.24438 | 96 44 07.39965 | 696681.352 | 4296101.378 | 448.034 | 418.447 |  |
| FEMA02 | MBR05 | 38 52 05.08322 | 96 47 50.06289 | 691104.671 | 4304443.191 | 392.876 | 363.269 |  |
| FEMA02 | MBR06 | 38 47 52.15545 | 96 54 13.12389 | 682051.300 | 4296428.255 | 402.985 | 373.528 |  |
| FEMA02 | MBR07 | 38 44 15.27471 | 96 50 47.15060 | 687177.878 | 4289857.606 | 427.688 | 398.221 |  |
| FEMA02 | MBR08 | 38 41 58.36364 | 96 48 58.05606 | 689912.715 | 4285699.233 | 450.377 | 420.901 |  |
| FEMA02 | MHG03 | 38 49 35.72845 | 96 30 04.53047 | 716912.415 | 4300500.054 | 441.160 | 411.239 |  |
| FEMA02 | MHG04 | 38 47 49.74053 | 96 43 25.06242 | 697687.884 | 4296727.808 | 440.143 | 410.538 |  |
| FEMA02 | MHG05 | 38 52 03.87150 | 96 47 30.49286 | 691577.246 | 4304417.233 | 373.834 | 344.221 |  |
| FEMA02 | MHG06 | 38 47 50.77446 | 96 55 12.23969 | 680626.097 | 4296353.099 | 394.413 | 364.966 |  |
| FEMA02 | MHG07 | 38 44 21.71609 | 96 50 46.10434 | 687198.467 | 4290056.779 | 431.220 | 401.752 |  |
| FEMA02 | MHG08 | 38 42 08.92853 | 96 48 58.99113 | 689882.360 | 4286024.395 | 447.328 | 417.852 |  |
| FEMA03 | MBE08 | 38 28 41.48585 | 97 03 31.96733 | 669319.966 | 4260658.436 | 429.211 | 399.997 |  |
| FEMA03 | MBE09 | 38 29 50.99990 | 96 57 27.62525 | 678101.243 | 4262992.390 | 432.330 | 403.030 |  |
| FEMA03 | MBE12 | 38 20 46.12342 | 97 01 11.49390 | 673038.721 | 4246077.215 | 398.560 | 369.346 |  |
| FEMA03 | MBE13 | 38 23 34.17325 | 97 05 18.75466 | 666928.824 | 4251130.988 | 414.333 | 385.170 |  |
| FEMA03 | MBE14 | 38 16 31.26970 | 97 05 42.97057 | 666610.232 | 4238082.515 | 428.390 | 399.272 |  |
| FEMA03 | MBE15 | 38 09 53.12943 | 97 06 19.75483 | 665967.538 | 4225791.410 | 414.156 | 385.061 |  |
| FEMA03 | MBE17 | 38 07 47.68604 | 97 00 39.98261 | 674319.457 | 4222097.814 | 440.665 | 411.488 |  |
| FEMA03 | MBE18 | 38 05 29.20712 | 96 52 41.02917 | 686079.023 | 4218087.532 | 453.627 | 424.298 |  |
| FEMA03 | MBE20 | 38 14 18.97131 | 96 55 34.95774 | 681476.446 | 4234322.138 | 387.286 | 357.994 |  |
| FEMA03 | MBR08 | 38 29 42.54959 | 97 03 32.70012 | 669262.509 | 4262540.449 | 428.160 | 398.942 |  |
| FEMA03 | MBR09 | 38 29 27.41370 | 96 55 47.26016 | 680549.049 | 4262319.636 | 448.264 | 418.944 |  |
| FEMA03 | MBR12 | 38 20 37.90379 | 97 01 05.06027 | 673200.346 | 4245827.187 | 398.760 | 369.545 |  |
| FEMA03 | MBR13 | 38 23 29.88568 | 97 05 20.19954 | 666896.509 | 4250998.093 | 414.768 | 385.606 |  |
| FEMA03 | MBR14 | 38 16 29.92088 | 97 05 28.22427 | 666969.423 | 4238048.327 | 429.214 | 400.093 |  |
| FEMA03 | MBR15 | 38 09 41.02704 | 97 06 11.59393 | 666173.781 | 4225422.417 | 416.298 | 387.201 |  |
| FEMA03 | MBR17 | 38 07 50.80738 | 97 00 39.59386 | 674326.859 | 4222194.233 | 440.261 | 411.084 |  |
| FEMA03 | MBR18 | 38 05 23.82920 | 96 52 33.71677 | 686260.960 | 4217925.828 | 456.332 | 427.000 |  |
| FEMA03 | MHG08 | 38 29 37.96062 | 97 03 32.99663 | 669258.310 | 4262398.835 | 426.901 | 397.683 |  |
| FEMA03 | MHG09 | 38 29 52.61569 | 96 57 26.80010 | 678120.128 | 4263042.644 | 433.039 | 403.739 |  |
| FEMA03 | MHG12 | 38 20 39.18698 | 97 01 04.97455 | 673201.578 | 4245866.788 | 398.402 | 369.187 |  |
| FEMA03 | MHG13 | 38 23 40.69058 | 97 05 16.86552 | 666970.490 | 4251332.842 | 414.195 | 385.031 |  |
| FEMA03 | MHG14 | 38 16 32.73650 | 97 06 16.14886 | 665803.078 | 4238111.162 | 427.874 | 398.764 |  |
| FEMA03 | MHG15 | 38 09 38.95660 | 97 06 11.64425 | 666173.862 | 4225358.571 | 418.212 | 389.116 |  |
| FEMA03 | MHG17 | 38 07 49.29517 | 97 00 39.62955 | 674326.990 | 4222147.600 | 440.384 | 411.207 |  |
| FEMA03 | MHG18 | 38 05 23.04839 | 96 52 39.58680 | 686118.502 | 4217898.489 | 454.078 | 424.748 |  |
| FEMA03 | MHG20 | 38 14 20.08130 | 96 55 32.33386 | 681539.473 | 4234357.784 | 387.519 | 358.227 |  |
| FEMA03 | CBE20 | 38 21 44.23744 | 96 27 48.93434 | 721606.558 | 4249058.275 | 365.643 | 335.541 |  |
| FEMA03 | CBR20 | 38 22 36.56233 | 96 27 52.78665 | 721468.712 | 4250668.827 | 353.308 | 323.208 |  |
| FEMA03 | CHG20 | 38 21 44.14914 | 96 27 53.21752 | 721502.666 | 4249052.695 | 365.705 | 335.606 |  |
| FEMA03 | CBE09 | 38 21 47.02928 | 96 48 29.33722 | 691496.863 | 4248372.887 | 445.357 | 415.906 |  |
| FEMA03 | CBE07 | 38 22 28.33427 | 96 38 57.11636 | 705353.270 | 4249988.132 | 367.505 | 337.768 |  |
| FEMA03 | CBE10 | 38 17 55.22487 | 96 44 12.15368 | 697914.298 | 4241377.668 | 374.118 | 344.553 |  |
| FEMA03 | CBE11 | 38 12 00.04378 | 96 43 50.99749 | 698696.852 | 4230441.094 | 419.390 | 389.825 |  |
| FEMA03 | CBE12 | 38 07 45.66579 | 96 48 21.54272 | 692301.169 | 4222440.968 | 417.244 | 387.791 |  |
| FEMA03 | CBE13 | 38 09 30.60399 | 96 39 27.53968 | 705222.320 | 4225993.885 | 447.581 | 417.918 |  |
| FEMA03 | CBE18 | 38 09 06.28956 | 96 33 32.37911 | 713886.891 | 4225467.414 | 401.569 | 371.749 |  |
| FEMA03 | CBE19 | 38 16 19.37918 | 96 32 48.48352 | 714601.622 | 4238846.838 | 389.973 | 360.083 |  |
| FEMA03 | CBE21 | 38 15 39.36625 | 96 39 17.43925 | 705180.201 | 4237368.026 | 432.073 | 402.399 |  |
| FEMA03 | CBR07 | 38 22 20.57644 | 96 38 53.30537 | 705451.854 | 4249751.331 | 365.916 | 336.177 |  |
| FEMA03 | CBR09 | 38 21 43.29978 | 96 48 04.30482 | 692107.178 | 4248272.372 | 444.632 | 415.170 |  |
| FEMA03 | CBR10 | 38 18 03.57492 | 96 44 28.10728 | 697520.439 | 4241625.593 | 373.125 | 343.567 |  |
| FEMA03 | CBR11 | 38 12 05.11704 | 96 43 51.37547 | 698683.825 | 4230597.262 | 415.125 | 385.560 |  |
| FEMA03 | CBR12 | 38 07 44.70549 | 96 49 01.40858 | 691331.151 | 4222388.461 | 426.033 | 396.598 |  |
| FEMA03 | CBR13 | 38 09 30.43739 | 96 39 41.86988 | 704873.647 | 4225979.942 | 442.778 | 413.121 |  |
| FEMA03 | CBR14 | 38 15 39.60009 | 96 39 32.28069 | 704819.279 | 4237366.095 | 443.459 | 413.792 |  |
| FEMA03 | CBR18 | 38 09 03.01761 | 96 33 33.70521 | 713857.266 | 4225365.698 | 399.531 | 369.712 |  |
| FEMA03 | CBR19 | 38 16 17.80098 | 96 32 00.62357 | 715766.062 | 4238829.134 | 371.056 | 341.139 |  |
| FEMA03 | CHG07 | 38 22 29.79290 | 96 38 55.96483 | 705380.069 | 4250033.811 | 366.041 | 336.302 |  |
| FEMA03 | CHG09 | 38 21 46.98192 | 96 48 26.59394 | 691563.481 | 4248373.009 | 444.320 | 414.868 |  |
| FEMA03 | CHG10 | 38 18 02.04087 | 96 44 30.00238 | 697475.558 | 4241577.177 | 373.979 | 344.422 |  |
| FEMA03 | CHG11 | 38 12 27.32233 | 96 43 51.08334 | 698674.156 | 4231281.955 | 434.658 | 405.093 |  |
| FEMA03 | CHG12 | 38 07 46.44643 | 96 48 15.30103 | 692452.581 | 4222468.629 | 414.983 | 385.526 |  |
| FEMA03 | CHG13 | 38 09 31.29401 | 96 38 52.22422 | 706081.374 | 4226036.926 | 453.839 | 424.162 |  |
| FEMA03 | CHG14 | 38 15 39.36092 | 96 39 30.08748 | 704872.774 | 4237360.072 | 441.697 | 412.029 |  |
| FEMA03 | CHG18 | 38 09 12.34837 | 96 33 50.21693 | 713447.755 | 4225642.769 | 411.473 | 381.661 |  |
| FEMA03 | CHG19 | 38 16 29.31878 | 96 32 02.10459 | 715720.596 | 4239183.247 | 382.389 | 352.471 |  |
| FEMA03 | MBE19 | 38 10 00.99593 | 96 52 38.53708 | 685948.005 | 4226467.021 | 445.699 | 416.350 |  |
| FEMA03 | MBR19 | 38 09 57.28567 | 96 52 40.12225 | 685912.046 | 4226351.765 | 445.196 | 415.848 |  |
| FEMA03 | MHG19 | 38 09 55.50756 | 96 52 39.50972 | 685928.209 | 4226297.294 | 444.785 | 415.437 |  |
| FEMA03 | MBR20 | 38 14 20.95452 | 96 55 28.12207 | 681641.268 | 4234386.998 | 386.566 | 357.272 |  |
| FEMA04 | MBE18 | 38 12 14.326586 | 97 24 31.292006 | 639328.951 | 4229644.169 | 460.523 | 431.814 |  |
| FEMA04 | MBE19 | 38 16 31.591952 | 97 26 44.427981 | 635957.551 | 4237519.117 | 467.159 | 438.556 |  |
| FEMA04 | MBR18 | 38 12 07.174165 | 97 24 30.064818 | 639362.588 | 4229424.217 | 458.093 | 429.382 |  |
| FEMA04 | MBR19 | 38 16 34.622864 | 97 26 50.061766 | 635819.088 | 4237610.242 | 467.555 | 438.957 |  |
| FEMA04 | MHG18 | 38 12 14.039449 | 97 24 29.728364 | 639367.135 | 4229635.972 | 460.353 | 431.643 |  |
| FEMA04 | MHG19 | 38 16 35.898066 | 97 26 44.071968 | 635963.970 | 4237651.995 | 467.122 | 438.520 |  |
| FEMA04 | HBE01 | 38 04 31.76837 | 97 39 57.29116 | 617011.553 | 4215031.171 | 453.492 | 425.146 |  |
| FEMA04 | HBE02 | 38 01 27.99507 | 97 40 00.39568 | 617017.164 | 4209365.767 | 441.925 | 413.550 |  |
| FEMA04 | HBE03 | 37 56 29.41913 | 97 39 55.83643 | 617260.262 | 4200164.660 | 438.955 | 410.535 |  |
| FEMA04 | HBE04 | 37 56 27.34843 | 97 29 38.29913 | 632334.982 | 4200330.625 | 422.429 | 393.682 |  |
| FEMA04 | HBE05 | 37 55 07.65825 | 97 25 52.12153 | 637897.298 | 4197965.483 | 418.859 | 390.041 |  |
| FEMA04 | HBE06 | 38 01 41.96616 | 97 25 21.07944 | 638449.321 | 4210131.929 | 431.244 | 402.475 |  |
| FEMA04 | HBE07 | 38 01 44.98071 | 97 19 07.09866 | 647565.727 | 4210384.638 | 450.159 | 421.309 |  |
| FEMA04 | HBE08 | 37 56 28.92188 | 97 16 53.61491 | 651000.090 | 4200702.152 | 450.283 | 421.385 |  |
| FEMA04 | HBE09 | 37 57 06.10357 | 97 10 14.43330 | 660721.777 | 4202033.776 | 418.162 | 389.164 |  |
| FEMA04 | HBE10 | 38 01 18.02515 | 97 12 32.02253 | 657214.167 | 4209733.644 | 434.471 | 405.518 |  |
| FEMA04 | HBE11 | 38 05 11.49461 | 97 25 53.96890 | 637538.438 | 4216576.686 | 441.862 | 413.140 |  |
| FEMA04 | HBE12 | 38 00 27.56537 | 97 31 12.17918 | 629925.692 | 4207697.966 | 423.996 | 395.323 |  |
| FEMA04 | HBE13 | 38 06 04.75170 | 97 30 30.79889 | 630768.058 | 4218107.124 | 445.426 | 416.809 |  |
| FEMA04 | HBE15 | 38 08 42.86255 | 97 39 56.06654 | 616930.123 | 4222770.978 | 442.852 | 414.558 |  |
| FEMA04 | HBE16 | 38 04 28.67966 | 97 21 25.85848 | 644093.428 | 4215369.913 | 442.909 | 414.103 |  |
| FEMA04 | HBE17 | 38 02 45.38872 | 97 12 10.32897 | 657691.082 | 4212436.734 | 433.230 | 404.268 |  |
| FEMA04 | HBE18 | 38 09 37.94306 | 97 11 56.66480 | 657777.421 | 4225159.852 | 445.191 | 416.195 |  |
| FEMA04 | HBE19 | 38 06 57.62195 | 97 15 30.89529 | 652656.337 | 4220118.472 | 467.182 | 438.274 |  |
| FEMA04 | HBR03 | 37 56 28.24423 | 97 39 55.79719 | 617261.739 | 4200128.461 | 439.107 | 410.687 |  |
| FEMA04 | HBR04 | 37 56 28.65058 | 97 29 41.95118 | 632245.187 | 4200369.320 | 422.423 | 393.677 |  |
| FEMA04 | HBR05 | 37 54 57.94791 | 97 25 36.65988 | 638279.881 | 4197672.550 | 418.478 | 389.656 |  |
| FEMA04 | HBR06 | 38 01 45.30788 | 97 25 37.57659 | 638045.361 | 4210228.118 | 430.328 | 401.563 |  |
| FEMA04 | HBR07 | 38 02 02.39233 | 97 19 20.95058 | 647218.319 | 4210915.223 | 448.091 | 419.246 |  |
| FEMA04 | HBR09 | 37 57 17.30311 | 97 10 09.94717 | 660824.486 | 4202381.138 | 424.494 | 395.495 |  |
| FEMA04 | HBR10 | 38 01 22.76311 | 97 12 30.27701 | 657253.915 | 4209880.506 | 430.912 | 401.959 |  |
| FEMA04 | HBR11 | 38 05 38.60184 | 97 25 36.16147 | 637958.099 | 4217419.558 | 440.011 | 411.287 |  |
| FEMA04 | HBR12 | 38 00 29.25454 | 97 31 29.27423 | 629507.959 | 4207743.409 | 423.836 | 395.170 |  |
| FEMA04 | HBR13 | 38 06 04.32405 | 97 31 25.04808 | 629446.947 | 4218072.822 | 440.720 | 412.123 |  |
| FEMA04 | HBR15 | 38 08 15.28025 | 97 39 54.44500 | 616981.829 | 4221921.384 | 439.015 | 410.714 |  |
| FEMA04 | HBR16 | 38 04 25.64598 | 97 21 22.17609 | 644184.807 | 4215277.992 | 442.472 | 413.665 |  |
| FEMA04 | HBR17 | 38 02 41.86251 | 97 12 14.24282 | 657597.779 | 4212326.198 | 435.983 | 407.023 |  |
| FEMA04 | HBR19 | 38 07 04.99744 | 97 15 42.55869 | 652368.047 | 4220340.489 | 463.607 | 434.703 |  |
| FEMA04 | HBR21 | 38 08 56.62366 | 97 25 58.76828 | 637304.337 | 4223513.957 | 452.822 | 424.128 |  |
| FEMA04 | HHG01 | 38 04 53.09096 | 97 39 55.93821 | 617035.075 | 4215688.859 | 451.587 | 423.243 |  |
| FEMA04 | HHG02 | 38 00 10.17604 | 97 39 54.88431 | 617185.958 | 4206969.146 | 439.008 | 410.618 |  |
| FEMA04 | HHG03 | 37 55 36.33839 | 97 39 54.60952 | 617313.646 | 4198529.048 | 436.882 | 408.453 |  |
| FEMA04 | HHG04 | 37 56 27.91293 | 97 29 36.56042 | 632377.142 | 4200348.710 | 422.180 | 393.433 |  |
| FEMA04 | HHG05 | 37 56 01.45969 | 97 25 49.50614 | 637933.230 | 4199624.853 | 420.549 | 391.735 |  |
| FEMA04 | HHG06 | 38 01 42.75647 | 97 25 48.20755 | 637787.499 | 4210145.095 | 429.651 | 400.889 |  |
| FEMA04 | HHG07 | 38 02 03.19073 | 97 19 23.08083 | 647165.941 | 4210938.895 | 447.546 | 418.701 |  |
| FEMA04 | HHG08 | 37 56 37.68205 | 97 16 53.22559 | 651004.612 | 4200972.343 | 449.901 | 421.002 |  |
| FEMA04 | HHG09 | 37 57 16.13387 | 97 10 11.74780 | 660781.248 | 4202344.234 | 423.987 | 394.989 |  |
| FEMA04 | HHG11 | 38 05 12.18922 | 97 25 19.47008 | 638378.523 | 4216612.334 | 440.494 | 411.761 |  |
| FEMA04 | HHG12 | 38 00 28.20245 | 97 31 30.03832 | 629489.839 | 4207710.686 | 424.351 | 395.685 |  |
| FEMA04 | HHG13 | 38 06 08.95398 | 97 30 34.36089 | 630679.221 | 4218235.258 | 443.827 | 415.211 |  |
| FEMA04 | HHG15 | 38 08 13.31318 | 97 39 52.33826 | 617033.989 | 4221861.491 | 438.469 | 410.166 |  |
| FEMA04 | HHG16 | 38 04 23.39444 | 97 21 19.66939 | 644247.114 | 4215209.673 | 442.389 | 413.581 |  |
| FEMA04 | HHG17 | 38 02 44.98788 | 97 12 14.70764 | 657584.587 | 4212422.316 | 435.751 | 406.790 |  |
| FEMA04 | HHG19 | 38 06 55.42021 | 97 15 33.51378 | 652593.844 | 4220049.409 | 467.075 | 438.168 |  |
| FEMA04 | MBE01 | 38 26 06.89378 | 97 19 33.24173 | 646113.284 | 4255435.663 | 476.585 | 447.758 |  |
| FEMA04 | MBE03 | 38 30 04.71897 | 97 13 32.90634 | 654708.645 | 4262930.307 | 425.709 | 396.658 |  |
| FEMA04 | MBE07 | 38 21 10.95381 | 97 11 39.03272 | 657789.522 | 4246530.242 | 430.308 | 401.275 |  |
| FEMA04 | MBE11 | 38 18 16.20943 | 97 17 58.52965 | 648677.008 | 4240968.813 | 449.733 | 420.870 |  |
| FEMA04 | MBE16 | 38 13 18.59662 | 97 16 48.38632 | 650551.340 | 4231826.525 | 468.067 | 439.169 |  |
| FEMA04 | MBR01 | 38 26 05.73755 | 97 19 51.68794 | 645666.689 | 4255391.908 | 477.997 | 449.183 |  |
| FEMA04 | MBR03 | 38 29 17.46479 | 97 13 33.45793 | 654723.366 | 4261473.393 | 424.636 | 395.586 |  |
| FEMA04 | MBR07 | 38 20 51.47308 | 97 10 40.09234 | 659232.070 | 4245957.855 | 424.625 | 395.573 |  |
| FEMA04 | MBR11 | 38 17 57.06897 | 97 17 52.21789 | 648841.184 | 4240381.635 | 438.015 | 409.148 |  |
| FEMA04 | MBR16 | 38 13 19.14209 | 97 16 47.00694 | 650584.570 | 4231843.962 | 467.324 | 438.426 |  |
| FEMA04 | MHG01 | 38 25 29.82981 | 97 20 05.37082 | 645354.937 | 4254279.040 | 477.016 | 448.214 |  |
| FEMA04 | MHG03 | 38 30 06.88009 | 97 13 34.00609 | 654680.720 | 4262996.412 | 426.442 | 397.391 |  |
| FEMA04 | MHG07 | 38 21 00.07808 | 97 11 35.16343 | 657890.005 | 4246196.831 | 433.230 | 404.197 |  |
| FEMA04 | MHG11 | 38 18 00.09410 | 97 17 53.33853 | 648812.246 | 4240474.382 | 440.988 | 412.121 |  |
| FEMA04 | MHG16 | 38 13 30.63887 | 97 16 48.73900 | 650535.868 | 4232197.562 | 469.752 | 440.854 |  |
| FEMA04 | MBE21 | 38 28 54.10698 | 97 28 00.90732 | 633718.890 | 4260375.800 | 445.301 | 416.875 |  |
| FEMA04 | MBE22 | 38 23 26.04683 | 97 27 31.91464 | 634590.504 | 4250275.116 | 478.912 | 450.418 |  |
| FEMA04 | MBR21 | 38 28 52.79273 | 97 28 21.07121 | 633230.997 | 4260327.166 | 451.708 | 423.300 |  |
| FEMA04 | MBR22 | 38 23 28.17608 | 97 27 43.63872 | 634304.981 | 4250336.002 | 477.317 | 448.831 |  |
| FEMA04 | MHG21 | 38 28 57.31266 | 97 28 21.76624 | 633211.845 | 4260466.214 | 457.624 | 429.216 |  |
| FEMA04 | MHG22 | 38 23 29.11595 | 97 26 52.89854 | 635535.454 | 4250385.591 | 480.084 | 451.561 |  |
| FEMA05 | MBE02 | 38 33 57.97359 | 97 18 56.27655 | 646743.701 | 4269973.354 | 460.880 | 432.011 |  |
| FEMA05 | MBE04 | 38 33 01.47909 | 97 09 05.11509 | 661086.066 | 4268506.903 | 432.579 | 403.435 |  |
| FEMA05 | MBE05 | 38 35 58.39974 | 97 03 34.39104 | 668976.924 | 4274125.878 | 433.597 | 404.342 |  |
| FEMA05 | MBE06 | 38 33 48.98787 | 96 57 57.23361 | 677221.636 | 4270313.005 | 454.120 | 424.804 |  |
| FEMA05 | MBR02 | 38 33 29.41985 | 97 18 57.31047 | 646734.810 | 4269092.698 | 450.427 | 421.556 |  |
| FEMA05 | MBR04 | 38 32 52.10817 | 97 09 23.84225 | 660638.520 | 4268208.925 | 435.386 | 406.247 |  |
| FEMA05 | MBR05 | 38 35 42.49036 | 97 03 44.15169 | 668751.153 | 4273630.450 | 434.093 | 404.843 |  |
| FEMA05 | MBR06 | 38 33 58.41803 | 96 57 49.08806 | 677412.345 | 4270608.080 | 452.632 | 423.312 |  |
| FEMA05 | MHG02 | 38 34 12.03121 | 97 18 58.74932 | 646675.914 | 4270405.597 | 460.091 | 431.224 |  |
| FEMA05 | MHG04 | 38 33 04.47320 | 97 08 20.66599 | 662160.203 | 4268620.916 | 431.255 | 402.099 |  |
| FEMA05 | MHG05 | 38 35 43.30247 | 97 03 45.43589 | 668719.556 | 4273654.830 | 434.012 | 404.762 |  |
| FEMA05 | MHG06 | 38 33 58.85284 | 96 57 46.92986 | 677464.282 | 4270622.642 | 451.582 | 422.261 |  |
| FEMA05 | BE01 | 38 41 14.39289 | 97 17 41.06230 | 648314.004 | 4283460.154 | 398.572 | 369.687 |  |
| FEMA05 | BE03 | 38 41 36.45598 | 97 08 51.69227 | 661090.543 | 4284388.583 | 394.295 | 365.093 |  |
| FEMA05 | BE07 | 38 42 02.59981 | 96 57 52.01635 | 677010.256 | 4285532.715 | 407.315 | 377.933 |  |
| FEMA05 | BE08 | 38 48 19.40654 | 97 16 46.19832 | 649393.269 | 4296586.827 | 369.366 | 340.572 |  |
| FEMA05 | BE09 | 38 47 51.84470 | 97 06 08.89845 | 664783.616 | 4296041.423 | 411.483 | 382.213 |  |
| FEMA05 | BE10 | 38 47 33.70630 | 96 57 49.01921 | 676855.502 | 4295741.748 | 386.895 | 357.483 |  |
| FEMA05 | BE12 | 38 54 26.20898 | 97 00 03.48922 | 673332.774 | 4308386.996 | 369.418 | 340.020 |  |
| FEMA05 | BE13 | 38 55 26.70912 | 97 22 03.81972 | 641496.304 | 4309618.882 | 372.076 | 343.952 |  |
| FEMA05 | BE14 | 38 54 45.37045 | 97 11 54.08212 | 656204.729 | 4308621.024 | 348.703 | 319.851 |  |
| FEMA05 | BE15 | 38 57 32.38661 | 97 06 25.38933 | 664014.514 | 4313930.271 | 358.841 | 329.783 |  |
| FEMA05 | BE16 | 38 59 14.30324 | 97 01 28.83950 | 671084.324 | 4317223.824 | 346.605 | 317.346 |  |
| FEMA05 | BE17 | 39 00 59.94522 | 97 15 34.34927 | 650678.885 | 4320065.420 | 368.016 | 339.889 |  |
| FEMA05 | BE18 | 39 06 14.21284 | 97 18 55.85682 | 645653.115 | 4329662.570 | 408.420 | 380.888 |  |
| FEMA05 | BE19 | 39 04 34.34615 | 97 08 54.12455 | 660169.721 | 4326865.180 | 379.217 | 350.843 |  |
| FEMA05 | BE20 | 39 05 24.65097 | 97 01 41.57985 | 670530.203 | 4328634.746 | 383.366 | 354.444 |  |
| FEMA05 | BR01 | 38 41 04.29971 | 97 17 56.67345 | 647942.596 | 4283142.005 | 397.687 | 368.812 |  |
| FEMA05 | BR03 | 38 41 25.22698 | 97 08 51.05210 | 661113.008 | 4284042.736 | 385.438 | 356.237 |  |
| FEMA05 | BR07 | 38 42 01.09944 | 96 57 51.81179 | 677016.226 | 4285486.572 | 408.240 | 378.859 |  |
| FEMA05 | BR08 | 38 48 19.68005 | 97 16 25.32169 | 649896.683 | 4296604.754 | 362.181 | 333.365 |  |
| FEMA05 | BR09 | 38 47 43.38048 | 97 06 15.39995 | 664632.183 | 4295777.235 | 412.689 | 383.421 |  |
| FEMA05 | BR10 | 38 47 25.67521 | 96 57 37.46193 | 677139.864 | 4295500.378 | 377.902 | 348.488 |  |
| FEMA05 | BR11 | 38 53 09.92105 | 97 06 43.86725 | 663737.310 | 4305829.635 | 368.929 | 339.728 |  |
| FEMA05 | BR12 | 38 54 24.70336 | 97 00 04.02674 | 673320.844 | 4308340.296 | 370.226 | 340.829 |  |
| FEMA05 | BR13 | 38 55 34.89093 | 97 22 17.17884 | 641170.097 | 4309865.354 | 367.717 | 339.616 |  |
| FEMA05 | BR14 | 38 54 34.73871 | 97 11 42.70020 | 656485.354 | 4308298.688 | 345.058 | 316.183 |  |
| FEMA05 | BR15 | 38 57 32.79640 | 97 06 28.95507 | 663928.425 | 4313941.122 | 358.852 | 329.798 |  |
| FEMA05 | BR16 | 38 59 17.26624 | 97 01 30.63127 | 671039.231 | 4317314.236 | 348.129 | 318.873 |  |
| FEMA05 | BR17 | 39 00 59.57666 | 97 15 27.07058 | 650854.154 | 4320057.408 | 371.947 | 343.811 |  |
| FEMA05 | BR18 | 39 06 06.52652 | 97 18 55.10937 | 645675.464 | 4329425.945 | 413.395 | 385.855 |  |
| FEMA05 | BR19 | 39 04 32.50729 | 97 09 09.52461 | 659800.810 | 4326800.957 | 384.241 | 355.887 |  |
| FEMA05 | BR20 | 39 05 22.22255 | 97 01 30.38699 | 670800.746 | 4328565.721 | 381.746 | 352.807 |  |
| FEMA05 | HG01 | 38 41 14.15875 | 97 17 44.61812 | 648228.225 | 4283451.338 | 396.648 | 367.765 |  |
| FEMA05 | HG03 | 38 41 30.31829 | 97 08 50.13298 | 661132.041 | 4284200.136 | 388.516 | 359.314 |  |
| FEMA05 | HG07 | 38 41 59.27090 | 96 57 53.25316 | 676982.658 | 4285429.428 | 410.005 | 380.624 |  |
| FEMA05 | HG08 | 38 48 19.30165 | 97 16 28.03674 | 649831.412 | 4296591.852 | 362.626 | 333.813 |  |
| FEMA05 | HG09 | 38 47 44.41005 | 97 06 12.00472 | 664713.435 | 4295810.673 | 412.583 | 383.314 |  |
| FEMA05 | HG10 | 38 47 26.58385 | 96 57 29.85209 | 677322.844 | 4295532.489 | 377.194 | 347.779 |  |
| FEMA05 | HG11 | 38 53 25.93783 | 97 06 42.16868 | 663768.016 | 4306324.252 | 359.188 | 329.991 |  |
| FEMA05 | HG12 | 38 54 26.83060 | 97 00 02.50730 | 673356.006 | 4308406.679 | 369.127 | 339.729 |  |
| FEMA05 | HG13 | 38 55 34.63239 | 97 22 13.30267 | 641263.579 | 4309859.051 | 370.464 | 342.358 |  |
| FEMA05 | HG14 | 38 54 39.57245 | 97 11 53.29621 | 656227.190 | 4308442.657 | 349.342 | 320.483 |  |
| FEMA05 | HG15 | 38 57 33.44400 | 97 06 26.43983 | 663988.551 | 4313962.344 | 359.568 | 330.512 |  |
| FEMA05 | HG16 | 38 59 15.65320 | 97 01 30.13111 | 671052.344 | 4317264.768 | 348.203 | 318.946 |  |
| FEMA05 | HG17 | 39 01 00.71367 | 97 15 32.06916 | 650733.268 | 4320090.159 | 368.479 | 340.351 |  |
| FEMA05 | HG18 | 39 05 31.89472 | 97 18 55.58772 | 645683.775 | 4328358.083 | 403.013 | 375.439 |  |
| FEMA05 | HG19 | 39 04 29.80746 | 97 09 08.19867 | 659834.365 | 4326718.371 | 381.771 | 353.411 |  |
| FEMA05 | HG20 | 39 05 24.58014 | 97 01 29.79289 | 670813.440 | 4328638.715 | 381.800 | 352.862 |  |
| FEMA05 | BE22 | 38 47 52.39348 | 97 17 53.38638 | 647788.106 | 4295723.748 | 368.478 | 339.744 |  |
| FEMA05 | BE21 | 38 47 52.39348 | 97 17 53.38638 | 647788.106 | 4295723.748 | 368.478 | 339.744 |  |
| FEMA06 | BE01 | 38 49 10.22302 | 97 32 21.44362 | 626809.147 | 4297760.749 | 385.439 | 357.498 |  |
| FEMA06 | BE02 | 38 52 14.70831 | 97 23 59.83658 | 638806.322 | 4303650.484 | 357.786 | 329.579 |  |
| FEMA06 | BE03 | 38 58 07.80045 | 97 27 56.06210 | 632930.370 | 4314437.589 | 362.717 | 335.060 |  |
| FEMA06 | BE04 | 38 52 54.27411 | 97 36 43.01941 | 620396.039 | 4304569.123 | 370.802 | 343.075 |  |
| FEMA06 | BE06 | 38 51 50.85841 | 97 45 40.50275 | 607472.117 | 4302427.880 | 388.109 | 360.492 |  |
| FEMA06 | BE07 | 38 53 33.57036 | 97 52 22.62924 | 597741.649 | 4305468.507 | 411.074 | 383.623 |  |
| FEMA06 | BE08 | 38 57 59.71402 | 97 45 34.27348 | 607467.579 | 4313800.406 | 384.244 | 356.751 |  |
| FEMA06 | BE09 | 38 58 01.32179 | 97 37 22.66486 | 619297.835 | 4314019.947 | 428.826 | 401.273 |  |
| FEMA06 | BE10 | 39 00 23.10159 | 97 52 25.70828 | 597511.462 | 4318091.921 | 394.149 | 366.786 |  |
| FEMA06 | BE11 | 39 02 08.88637 | 97 43 27.77498 | 610404.225 | 4321523.662 | 401.159 | 373.759 |  |
| FEMA06 | BE12 | 39 01 45.35579 | 97 35 35.64485 | 621766.640 | 4320965.652 | 371.520 | 344.086 |  |
| FEMA06 | BE13 | 39 05 19.76087 | 97 27 51.22315 | 632822.001 | 4327755.927 | 393.970 | 366.676 |  |
| FEMA06 | BE14 | 39 08 47.66756 | 97 31 59.58637 | 626751.563 | 4334066.673 | 417.092 | 389.924 |  |
| FEMA06 | BE15 | 39 07 14.19858 | 97 42 44.97784 | 611299.823 | 4330950.119 | 380.805 | 353.536 |  |
| FEMA06 | BE16 | 39 08 57.28213 | 97 53 23.32120 | 595932.123 | 4333925.496 | 390.704 | 363.476 |  |
| FEMA06 | BE17 | 39 15 46.48631 | 97 52 41.93752 | 596769.353 | 4346552.492 | 496.007 | 468.938 |  |
| FEMA06 | BE18 | 39 16 39.12874 | 97 44 35.03259 | 608415.508 | 4348328.677 | 408.703 | 381.657 |  |
| FEMA06 | BE19 | 39 15 47.41710 | 97 32 57.45735 | 625155.217 | 4346984.544 | 413.023 | 386.032 |  |
| FEMA06 | BE20 | 39 14 53.46581 | 97 24 28.49962 | 637381.951 | 4345526.306 | 429.953 | 402.939 |  |
| FEMA06 | BR01 | 38 49 03.20101 | 97 32 21.44659 | 626812.536 | 4297544.283 | 382.532 | 354.586 |  |
| FEMA06 | BR02 | 38 52 13.66770 | 97 24 02.96803 | 638731.419 | 4303617.083 | 358.008 | 329.803 |  |
| FEMA06 | BR03 | 38 58 09.62190 | 97 28 02.72538 | 632769.069 | 4314491.040 | 362.816 | 335.164 |  |
| FEMA06 | BR04 | 38 53 06.08948 | 97 36 47.96272 | 620271.397 | 4304931.540 | 372.270 | 344.550 |  |
| FEMA06 | BR05 | 38 48 13.13911 | 97 36 42.12030 | 620549.431 | 4295903.068 | 376.914 | 349.057 |  |
| FEMA06 | BR06 | 38 52 15.07028 | 97 45 23.60008 | 607869.312 | 4303179.777 | 401.078 | 373.465 |  |
| FEMA06 | BR07 | 38 53 47.27672 | 97 52 36.22063 | 597409.023 | 4305886.981 | 418.812 | 391.370 |  |
| FEMA06 | BR08 | 38 57 58.92224 | 97 45 37.05062 | 607401.078 | 4313775.088 | 384.288 | 356.796 |  |
| FEMA06 | BR09 | 38 58 08.91452 | 97 37 24.36982 | 619253.266 | 4314253.388 | 418.705 | 391.156 |  |
| FEMA06 | BR10 | 39 00 17.13354 | 97 52 27.42196 | 597472.520 | 4317907.436 | 390.701 | 363.336 |  |
| FEMA06 | BR11 | 39 02 11.67818 | 97 43 31.00653 | 610325.324 | 4321608.636 | 402.106 | 374.707 |  |
| FEMA06 | BR12 | 39 01 39.17392 | 97 35 37.91305 | 621715.046 | 4320774.238 | 370.812 | 343.374 |  |
| FEMA06 | BR13 | 39 05 19.18826 | 97 27 35.02748 | 633211.401 | 4327744.862 | 381.361 | 354.064 |  |
| FEMA06 | BR14 | 39 08 49.69276 | 97 32 15.98910 | 626356.807 | 4334122.751 | 422.213 | 395.045 |  |
| FEMA06 | BR15 | 39 07 07.47950 | 97 42 53.21515 | 611104.948 | 4330740.184 | 381.265 | 353.992 |  |
| FEMA06 | BR16 | 39 08 52.41340 | 97 53 12.16696 | 596201.706 | 4333778.686 | 389.497 | 362.268 |  |
| FEMA06 | BR17 | 39 15 48.93304 | 97 52 38.70152 | 596845.968 | 4346628.881 | 492.988 | 465.920 |  |
| FEMA06 | BR18 | 39 16 31.60976 | 97 44 34.72558 | 608426.083 | 4348096.982 | 406.497 | 379.450 |  |
| FEMA06 | BR19 | 39 15 51.90236 | 97 32 54.44972 | 625225.078 | 4347123.974 | 413.584 | 386.595 |  |
| FEMA06 | BR20 | 39 14 53.70734 | 97 24 29.05740 | 637368.450 | 4345533.517 | 429.701 | 402.687 |  |
| FEMA06 | HG01 | 38 49 01.27980 | 97 32 20.23525 | 626842.696 | 4297485.526 | 383.034 | 355.087 |  |
| FEMA06 | HG02 | 38 52 14.17185 | 97 24 02.07955 | 638752.558 | 4303632.999 | 358.054 | 329.849 |  |
| FEMA06 | HG03 | 38 58 09.41431 | 97 28 04.55396 | 632725.171 | 4314483.900 | 362.819 | 335.167 |  |
| FEMA06 | HG04 | 38 53 05.86806 | 97 36 49.69760 | 620229.701 | 4304924.080 | 371.860 | 344.140 |  |
| FEMA06 | HG05 | 38 48 12.37775 | 97 36 41.87601 | 620555.680 | 4295879.688 | 376.912 | 349.055 |  |
| FEMA06 | HG06 | 38 52 14.96564 | 97 45 53.22349 | 607155.487 | 4303166.859 | 392.795 | 365.190 |  |
| FEMA06 | HG07 | 38 53 33.44214 | 97 52 24.11977 | 597705.790 | 4305464.111 | 410.470 | 383.020 |  |
| FEMA06 | HG08 | 38 57 57.34764 | 97 45 33.66399 | 607483.241 | 4313727.658 | 383.450 | 355.957 |  |
| FEMA06 | HG09 | 38 58 11.92291 | 97 37 23.52537 | 619272.186 | 4314346.435 | 415.488 | 387.941 |  |
| FEMA06 | HG10 | 39 00 18.63197 | 97 52 25.13245 | 597527.017 | 4317954.309 | 394.599 | 367.235 |  |
| FEMA06 | HG11 | 39 02 09.96541 | 97 43 29.72022 | 610356.990 | 4321556.270 | 400.264 | 372.865 |  |
| FEMA06 | HG12 | 39 01 49.40918 | 97 35 16.43237 | 622226.661 | 4321097.765 | 371.135 | 343.703 |  |
| FEMA06 | HG13 | 39 05 20.47345 | 97 27 34.41380 | 633225.473 | 4327784.732 | 381.534 | 354.238 |  |
| FEMA06 | HG14 | 39 08 48.49795 | 97 32 17.99896 | 626309.154 | 4334085.140 | 423.686 | 396.517 |  |
| FEMA06 | HG15 | 39 07 06.08654 | 97 42 52.93720 | 611112.230 | 4330697.337 | 380.645 | 353.372 |  |
| FEMA06 | HG16 | 39 08 52.52309 | 97 53 06.92675 | 596327.451 | 4333783.612 | 389.464 | 362.234 |  |
| FEMA06 | HG17 | 39 15 47.41365 | 97 52 40.45835 | 596804.447 | 4346581.519 | 494.947 | 467.878 |  |
| FEMA06 | HG18 | 39 16 30.95884 | 97 44 31.49722 | 608503.717 | 4348077.990 | 403.194 | 376.147 |  |
| FEMA06 | HG19 | 39 15 49.77272 | 97 32 53.46746 | 625249.671 | 4347058.697 | 411.783 | 384.794 |  |
| FEMA06 | HG20 | 39 14 54.19373 | 97 24 48.46672 | 636902.929 | 4345540.345 | 438.322 | 411.310 |  |
| FEMA06 | UR05 | 38 49 41.94883 | 97 38 01.43797 | 618595.273 | 4298611.911 | 374.552 | 346.756 |  |
| FEMA06 | UR06 | 38 48 35.19390 | 97 35 43.75936 | 621946.718 | 4296604.440 | 375.919 | 348.052 |  |
| FEMA06 | UR07 | 38 48 47.12554 | 97 37 11.79798 | 619817.754 | 4296939.902 | 377.053 | 349.220 |  |
| FEMA06 | UR08 | 38 48 41.51548 | 97 36 20.45417 | 621058.700 | 4296785.760 | 375.882 | 348.030 |  |
| FEMA06 | UR09 | 38 49 39.29232 | 97 35 35.68978 | 622110.932 | 4298583.353 | 373.676 | 345.836 |  |
| FEMA06 | UR10 | 38 49 34.77683 | 97 36 46.63572 | 620402.320 | 4298418.002 | 375.126 | 347.306 |  |
| FEMA06 | UR11 | 38 49 33.13981 | 97 37 19.06802 | 619621.026 | 4298355.705 | 375.070 | 347.259 |  |
| FEMA06 | UR12 | 38 50 06.85429 | 97 35 51.74766 | 621710.685 | 4299427.037 | 372.809 | 344.987 |  |
| FEMA06 | UR13 | 38 50 15.54122 | 97 37 31.45945 | 619302.557 | 4299658.283 | 374.285 | 346.494 |  |
| FEMA06 | UR14 | 38 50 22.37226 | 97 36 36.08132 | 620634.500 | 4299889.064 | 373.715 | 345.913 |  |
| FEMA06 | UR15 | 38 50 57.25338 | 97 37 16.07681 | 619654.015 | 4300949.708 | 373.442 | 345.665 |  |
| FEMA06 | UR16 | 38 50 55.17320 | 97 36 34.84797 | 620648.843 | 4300900.652 | 372.537 | 344.749 |  |
| FEMA06 | UR17 | 38 51 24.66998 | 97 36 27.39358 | 620814.673 | 4301812.672 | 371.251 | 343.475 |  |
| FEMA06 | UR18 | 38 51 03.95414 | 97 35 40.47871 | 621955.300 | 4301191.399 | 371.300 | 343.502 |  |
| FEMA06 | UR19 | 38 50 26.77889 | 97 34 42.56091 | 623369.256 | 4300067.025 | 380.550 | 352.715 |  |
| FEMA06 | UR20 | 38 49 28.71198 | 97 33 49.92873 | 624666.289 | 4298296.874 | 390.360 | 362.474 |  |
| FEMA06 | BE23 | 38 50 25.16485 | 97 26 46.10980 | 634856.784 | 4300204.362 | 376.134 | 347.989 |  |
| FEMA06 | BE21 | 38 50 25.16484 | 97 26 46.10990 | 634856.782 | 4300204.362 | 376.139 | 347.994 |  |
| FEMA06 | BE22 | 38 50 25.16505 | 97 26 46.11002 | 634856.779 | 4300204.368 | 376.136 | 347.991 |  |
| FEMA06 | UR21 | 38 49 23.16201 | 97 35 36.10351 | 622108.611 | 4298085.960 | 375.787 | 347.940 |  |
| FEMA06 | UR23 | 38 48 47.67339 | 97 36 31.47086 | 620790.104 | 4296971.536 | 376.408 | 348.563 |  |
| FEMA06 | UR24 | 38 48 39.89425 | 97 36 48.91758 | 620372.964 | 4296725.340 | 376.566 | 348.723 |  |
| FEMA06 | UR25 | 38 49 26.71949 | 97 36 48.46746 | 620361.919 | 4298168.953 | 375.752 | 347.929 |  |
| FEMA07 | MBE03 | 38 14 50.386300 | 97 46 49.525823 | 606716.814 | 4233960.488 | 464.552 | 436.519 |  |
| FEMA07 | MBE04 | 38 22 07.560786 | 97 47 09.888054 | 606044.860 | 4247429.124 | 469.864 | 441.941 |  |
| FEMA07 | MBE05 | 38 28 56.501719 | 97 49 55.594726 | 601863.993 | 4259982.323 | 447.237 | 419.435 |  |
| FEMA07 | MBE06 | 38 33 31.416704 | 97 50 35.958641 | 600779.472 | 4268444.007 | 425.381 | 397.632 |  |
| FEMA07 | MBE16 | 38 13 47.130498 | 97 39 58.186140 | 616743.525 | 4232148.686 | 444.701 | 416.510 |  |
| FEMA07 | MBE17 | 38 12 19.903604 | 97 30 35.329746 | 630471.867 | 4229668.780 | 451.347 | 422.812 |  |
| FEMA07 | MBE20 | 38 17 33.550146 | 97 35 33.899329 | 623063.095 | 4239222.886 | 448.683 | 420.443 |  |
| FEMA07 | MBR03 | 38 14 50.847975 | 97 46 43.934788 | 606852.529 | 4233976.511 | 463.756 | 435.723 |  |
| FEMA07 | MBR04 | 38 22 09.266186 | 97 47 08.602657 | 606075.361 | 4247482.101 | 469.040 | 441.117 |  |
| FEMA07 | MBR05 | 38 28 44.059722 | 97 49 40.137813 | 602243.377 | 4259603.570 | 454.038 | 426.229 |  |
| FEMA07 | MBR06 | 38 33 30.322785 | 97 50 32.262665 | 600869.350 | 4268411.414 | 424.965 | 397.213 |  |
| FEMA07 | MBR16 | 38 13 43.778716 | 97 39 53.508524 | 616858.744 | 4232047.013 | 445.523 | 417.329 |  |
| FEMA07 | MBR17 | 38 12 21.540045 | 97 30 36.659153 | 630438.722 | 4229718.700 | 450.688 | 422.154 |  |
| FEMA07 | MBR20 | 38 17 27.370290 | 97 35 28.498029 | 623197.210 | 4239034.398 | 448.743 | 420.496 |  |
| FEMA07 | MHG03 | 38 14 47.530291 | 97 46 51.813781 | 606662.359 | 4233871.725 | 462.915 | 434.882 |  |
| FEMA07 | MHG04 | 38 22 08.246901 | 97 47 06.159087 | 606135.072 | 4247451.463 | 469.173 | 441.249 |  |
| FEMA07 | MHG05 | 38 28 44.375575 | 97 49 42.246022 | 602192.172 | 4259612.656 | 453.067 | 425.259 |  |
| FEMA07 | MHG06 | 38 33 30.773629 | 97 50 54.847092 | 600322.562 | 4268418.444 | 426.328 | 398.586 |  |
| FEMA07 | MHG16 | 38 13 51.566596 | 97 39 56.645482 | 616779.013 | 4232285.960 | 444.795 | 416.606 |  |
| FEMA07 | MHG17 | 38 12 01.756270 | 97 30 32.777819 | 630542.938 | 4229110.413 | 450.184 | 421.643 |  |
| FEMA07 | MHG20 | 38 17 28.479930 | 97 35 31.869021 | 623114.797 | 4239067.354 | 448.516 | 420.273 |  |
| FEMA07 | BE07 | 38 40 56.64838 | 97 53 19.61015 | 596652.457 | 4282119.221 | 430.432 | 402.840 |  |
| FEMA07 | BE08 | 38 46 56.53541 | 97 51 24.14979 | 599303.625 | 4293247.129 | 441.599 | 414.033 |  |
| FEMA07 | BE09 | 38 40 54.57629 | 97 44 05.04327 | 610052.548 | 4282229.051 | 408.653 | 380.828 |  |
| FEMA07 | BE10 | 38 46 00.67706 | 97 41 14.23496 | 614044.350 | 4291722.835 | 393.076 | 365.265 |  |
| FEMA07 | BE11 | 38 42 39.83457 | 97 33 47.83524 | 624914.716 | 4285693.538 | 390.445 | 362.356 |  |
| FEMA07 | BE12 | 38 47 01.13080 | 97 26 48.10611 | 634915.555 | 4293913.835 | 368.032 | 339.765 |  |
| FEMA07 | BE13 | 38 42 18.95285 | 97 25 03.48573 | 637590.256 | 4285258.549 | 374.983 | 346.521 |  |
| FEMA07 | BE14 | 38 36 25.62363 | 97 25 40.33412 | 636886.919 | 4274351.478 | 394.494 | 365.977 |  |
| FEMA07 | BE15 | 38 33 04.23260 | 97 32 31.76816 | 627033.835 | 4267979.307 | 466.759 | 438.562 |  |
| FEMA07 | BE17 | 38 34 06.04752 | 97 40 29.00868 | 615454.228 | 4269709.852 | 407.055 | 379.111 |  |
| FEMA07 | BR07 | 38 40 36.61515 | 97 53 17.98712 | 596699.160 | 4281502.174 | 434.494 | 406.898 |  |
| FEMA07 | BR08 | 38 47 00.94851 | 97 51 20.20618 | 599397.074 | 4293384.355 | 437.703 | 410.136 |  |
| FEMA07 | BR09 | 38 40 52.67751 | 97 44 06.22534 | 610024.794 | 4282170.126 | 410.027 | 382.202 |  |
| FEMA07 | BR10 | 38 46 07.13953 | 97 41 14.12176 | 614044.223 | 4291922.085 | 392.706 | 364.898 |  |
| FEMA07 | BR11 | 38 42 40.70446 | 97 33 25.02152 | 625465.301 | 4285729.015 | 398.365 | 370.263 |  |
| FEMA07 | BR12 | 38 47 03.25893 | 97 26 09.54829 | 635844.767 | 4293995.296 | 366.075 | 337.779 |  |
| FEMA07 | BR13 | 38 42 17.72065 | 97 25 24.70051 | 637078.472 | 4285211.730 | 374.447 | 346.002 |  |
| FEMA07 | BR14 | 38 36 22.92273 | 97 25 39.89239 | 636899.029 | 4274268.404 | 393.675 | 365.157 |  |
| FEMA07 | BR15 | 38 33 05.62094 | 97 32 49.80419 | 626596.574 | 4268015.191 | 462.207 | 434.024 |  |
| FEMA07 | HG07 | 38 40 32.27336 | 97 53 17.98399 | 596700.859 | 4281368.340 | 443.827 | 416.230 |  |
| FEMA07 | HG08 | 38 46 59.22574 | 97 51 23.74018 | 599312.471 | 4293330.183 | 439.334 | 411.768 |  |
| FEMA07 | HG09 | 38 40 53.70120 | 97 44 06.79422 | 610010.613 | 4282201.492 | 410.367 | 382.542 |  |
| FEMA07 | HG10 | 38 45 27.12423 | 97 41 12.45360 | 614102.185 | 4290689.159 | 403.528 | 375.707 |  |
| FEMA07 | HG11 | 38 42 38.13172 | 97 33 28.52302 | 625381.980 | 4285648.376 | 393.358 | 365.257 |  |
| FEMA07 | HG12 | 38 47 03.21276 | 97 26 10.96315 | 635810.654 | 4293993.289 | 366.464 | 338.169 |  |
| FEMA07 | HG13 | 38 42 23.70916 | 97 25 38.46074 | 636742.926 | 4285390.621 | 374.869 | 346.438 |  |
| FEMA07 | HG14 | 38 36 33.44217 | 97 25 50.31378 | 636641.413 | 4274588.361 | 394.003 | 365.494 |  |
| FEMA07 | HG15 | 38 33 04.24476 | 97 32 55.34590 | 626463.100 | 4267970.651 | 466.903 | 438.723 |  |
| FEMA07 | HG17 | 38 33 47.00822 | 97 40 06.21837 | 616014.262 | 4269130.942 | 403.855 | 375.903 |  |
| FEMA07 | MBE01 | 38 20 00.40110 | 97 39 58.34335 | 616573.740 | 4243654.124 | 454.854 | 426.803 |  |
| FEMA07 | MBE02 | 38 23 45.14474 | 97 39 57.60113 | 616491.635 | 4250581.855 | 460.418 | 432.411 |  |
| FEMA07 | MBE18 | 38 27 09.77814 | 97 40 01.62131 | 616302.904 | 4256888.100 | 462.637 | 434.665 |  |
| FEMA07 | MBE19 | 38 26 53.55818 | 97 32 15.42733 | 627610.968 | 4256559.576 | 476.296 | 448.064 |  |
| FEMA07 | MBR01 | 38 19 58.52497 | 97 40 01.13280 | 616506.850 | 4243595.317 | 454.197 | 426.147 |  |
| FEMA07 | MBR02 | 38 23 42.35362 | 97 40 01.89275 | 616388.773 | 4250494.316 | 459.864 | 431.857 |  |
| FEMA07 | MBR18 | 38 26 57.02200 | 97 40 03.19387 | 616270.474 | 4256494.350 | 456.623 | 428.650 |  |
| FEMA07 | MBR19 | 38 26 55.99507 | 97 32 19.99330 | 627499.094 | 4256632.936 | 477.535 | 449.307 |  |
| FEMA07 | MHG01 | 38 19 58.36206 | 97 40 03.52403 | 616448.866 | 4243589.458 | 454.538 | 426.489 |  |
| FEMA07 | MHG02 | 38 24 10.24661 | 97 40 00.49277 | 616410.305 | 4251354.582 | 460.213 | 432.212 |  |
| FEMA07 | MHG18 | 38 27 06.68237 | 97 39 58.86893 | 616371.000 | 4256793.641 | 462.583 | 434.609 |  |
| FEMA07 | MHG19 | 38 26 52.24031 | 97 32 15.43383 | 627611.455 | 4256518.950 | 476.492 | 448.260 |  |
| FEMA08 | BE14 | 39 32 48.60132 | 97 56 44.84430 | 590580.047 | 4377992.772 | 473.377 | 446.598 |  |
| FEMA08 | BE18 | 39 31 29.07382 | 98 25 27.31267 | 549484.719 | 4375168.710 | 466.843 | 440.451 |  |
| FEMA08 | BR14 | 39 32 48.79325 | 97 56 47.52882 | 590515.904 | 4377997.938 | 473.229 | 446.451 |  |
| FEMA08 | BR18 | 39 31 27.78513 | 98 25 32.61930 | 549358.277 | 4375128.172 | 467.750 | 441.357 |  |
| FEMA08 | HG14 | 39 32 49.47935 | 97 56 42.25421 | 590641.549 | 4378020.565 | 473.641 | 446.861 |  |
| FEMA08 | HG18 | 39 31 25.97678 | 98 25 25.41991 | 549530.519 | 4375073.522 | 465.523 | 439.129 |  |
| FEMA08 | BE01 | 39 34 54.31495 | 98 12 20.27530 | 568219.307 | 4381639.037 | 462.243 | 435.841 |  |
| FEMA08 | BE02 | 39 36 39.25289 | 98 18 23.33162 | 559533.479 | 4384802.499 | 476.337 | 450.000 |  |
| FEMA08 | BE03 | 39 35 46.62872 | 98 26 52.94735 | 547391.489 | 4383095.948 | 492.299 | 466.002 |  |
| FEMA08 | BE04 | 39 41 07.29180 | 98 28 01.75816 | 545691.678 | 4392971.753 | 525.395 | 499.231 |  |
| FEMA08 | BE05 | 39 39 48.30608 | 98 20 48.11142 | 556038.755 | 4390604.960 | 468.754 | 442.500 |  |
| FEMA08 | BE06 | 39 41 55.75469 | 98 16 55.19998 | 561556.978 | 4394576.512 | 511.989 | 485.764 |  |
| FEMA08 | BE07 | 39 40 00.14850 | 98 09 07.84177 | 572720.925 | 4391109.577 | 473.217 | 446.862 |  |
| FEMA08 | BE08 | 39 38 01.33215 | 98 01 29.97845 | 583669.971 | 4387557.298 | 437.437 | 410.894 |  |
| FEMA08 | BE09 | 39 40 56.44181 | 97 57 01.24918 | 590012.796 | 4393028.092 | 448.404 | 421.839 |  |
| FEMA08 | BE10 | 39 43 33.88984 | 97 57 03.76003 | 589896.207 | 4397881.516 | 449.429 | 422.941 |  |
| FEMA08 | BE11 | 39 46 53.37910 | 97 59 49.56274 | 585880.223 | 4403986.629 | 462.737 | 436.406 |  |
| FEMA08 | BE12 | 39 47 03.96904 | 98 05 42.93746 | 577471.186 | 4404223.573 | 502.067 | 475.850 |  |
| FEMA08 | BE13 | 39 47 03.56134 | 98 12 02.12516 | 568451.977 | 4404125.166 | 539.648 | 513.495 |  |
| FEMA08 | BE22 | 39 48 49.71291 | 98 28 01.98472 | 545601.512 | 4407227.945 | 558.585 | 532.621 |  |
| FEMA08 | BE15 | 39 59 15.94662 | 98 29 10.59341 | 543859.261 | 4426525.385 | 592.956 | 567.052 |  |
| FEMA08 | BE16 | 39 58 23.88781 | 98 22 29.55480 | 553381.331 | 4424981.112 | 562.218 | 536.278 |  |
| FEMA08 | BE17 | 39 51 55.82182 | 98 16 58.56919 | 561328.582 | 4413075.878 | 532.404 | 506.356 |  |
| FEMA08 | BE23 | 39 54 23.29997 | 98 06 56.52741 | 575586.827 | 4417750.858 | 488.024 | 461.945 |  |
| FEMA08 | BE19 | 39 56 09.61266 | 98 02 07.90291 | 582404.487 | 4421099.519 | 508.466 | 482.342 |  |
| FEMA08 | BE20 | 39 51 56.13069 | 97 58 53.39904 | 587109.922 | 4413335.712 | 474.412 | 448.204 |  |
| FEMA08 | BE21_2 | 39 32 19.90780 | 98 09 12.26771 | 572749.125 | 4376919.757 | 462.405 | 435.890 |  |
| FEMA08 | BR01 | 39 34 55.18260 | 98 12 00.41092 | 568692.943 | 4381669.986 | 462.289 | 435.882 |  |
| FEMA08 | BR02 | 39 36 38.37654 | 98 18 06.46518 | 559935.876 | 4384778.597 | 477.914 | 451.577 |  |
| FEMA08 | BR03 | 39 35 46.39055 | 98 26 37.86978 | 547751.139 | 4383090.822 | 491.803 | 465.504 |  |
| FEMA08 | BR04 | 39 41 01.70024 | 98 28 05.89424 | 545594.179 | 4392798.785 | 527.299 | 501.134 |  |
| FEMA08 | BR05 | 39 39 48.18282 | 98 20 49.68072 | 556001.391 | 4390600.888 | 468.512 | 442.257 |  |
| FEMA08 | BR06 | 39 41 49.87864 | 98 16 44.85123 | 561804.891 | 4394397.333 | 510.164 | 483.935 |  |
| FEMA08 | BR07 | 39 40 06.90260 | 98 09 42.46144 | 571894.119 | 4391310.055 | 470.701 | 444.358 |  |
| FEMA08 | BR08 | 39 37 54.45795 | 98 01 29.70931 | 583678.688 | 4387345.440 | 437.231 | 410.686 |  |
| FEMA08 | BR09 | 39 41 03.29840 | 97 57 01.73790 | 589998.681 | 4393239.344 | 449.120 | 422.558 |  |
| FEMA08 | BR10 | 39 43 35.33372 | 97 56 59.74146 | 589991.352 | 4397927.151 | 447.829 | 421.339 |  |
| FEMA08 | BR11 | 39 46 42.56654 | 97 59 40.66426 | 586095.635 | 4403655.648 | 461.969 | 435.629 |  |
| FEMA08 | BR12 | 39 47 19.52665 | 98 05 19.28125 | 578028.989 | 4404708.924 | 508.729 | 482.513 |  |
| FEMA08 | BR13 | 39 47 03.49606 | 98 11 58.90389 | 568528.616 | 4404123.838 | 539.701 | 513.548 |  |
| FEMA08 | BR21 | 39 48 50.19460 | 98 26 06.96219 | 548336.136 | 4407259.566 | 557.182 | 531.185 |  |
| FEMA08 | BR15 | 39 59 15.55613 | 98 29 06.40980 | 543958.547 | 4426513.918 | 589.883 | 563.979 |  |
| FEMA08 | BR16 | 39 58 24.67076 | 98 22 28.68685 | 553401.750 | 4425005.396 | 563.663 | 537.724 |  |
| FEMA08 | BR17 | 39 51 52.69849 | 98 16 58.70262 | 561326.185 | 4412979.559 | 534.019 | 507.972 |  |
| FEMA08 | BR18 | 39 54 22.55322 | 98 07 07.00182 | 575338.351 | 4417725.376 | 487.690 | 461.612 |  |
| FEMA08 | BR19 | 39 56 03.77945 | 98 02 05.74938 | 582457.543 | 4420920.228 | 507.783 | 481.658 |  |
| FEMA08 | BR20 | 39 51 53.15160 | 97 58 48.57731 | 587225.526 | 4413245.170 | 475.651 | 449.440 |  |
| FEMA08 | HG01 | 39 34 54.16969 | 98 12 05.26131 | 568577.513 | 4381637.731 | 460.165 | 433.759 |  |
| FEMA08 | HG02 | 39 36 38.87822 | 98 18 31.82438 | 559331.055 | 4384789.388 | 472.787 | 446.451 |  |
| FEMA08 | HG03 | 39 35 46.30894 | 98 27 00.37030 | 547214.510 | 4383085.005 | 485.646 | 459.350 |  |
| FEMA08 | HG03_2 | 39 32 22.27985 | 98 08 12.08348 | 574185.037 | 4377006.533 | 456.417 | 429.880 |  |
| FEMA08 | HG04 | 39 41 23.21340 | 98 28 02.36529 | 545674.302 | 4393462.517 | 523.051 | 496.893 |  |
| FEMA08 | HG05 | 39 40 07.58810 | 98 20 02.03978 | 557132.108 | 4391207.478 | 484.081 | 457.830 |  |
| FEMA08 | HG06 | 39 41 50.43668 | 98 16 43.02316 | 561848.290 | 4394414.887 | 513.105 | 486.876 |  |
| FEMA08 | HG07 | 39 39 35.70738 | 98 09 23.77585 | 572348.354 | 4390352.494 | 473.342 | 446.982 |  |
| FEMA08 | HG08 | 39 38 09.39551 | 98 01 33.10221 | 583592.811 | 4387805.080 | 441.671 | 415.133 |  |
| FEMA08 | HG09 | 39 40 58.27695 | 97 56 36.15872 | 590609.817 | 4393091.685 | 443.084 | 416.511 |  |
| FEMA08 | HG10 | 39 43 37.44728 | 97 57 00.91187 | 589962.726 | 4397991.986 | 445.377 | 418.889 |  |
| FEMA08 | HG11 | 39 46 36.25970 | 97 59 49.09955 | 585897.153 | 4403458.956 | 456.384 | 430.044 |  |
| FEMA08 | HG12 | 39 47 03.29232 | 98 05 29.26065 | 577796.714 | 4404206.004 | 498.130 | 471.908 |  |
| FEMA08 | HG13 | 39 47 04.10587 | 98 11 53.92884 | 568646.783 | 4404143.698 | 536.278 | 510.125 |  |
| FEMA08 | HG21 | 39 48 49.09652 | 98 26 53.97189 | 547218.668 | 4407218.740 | 549.587 | 523.603 |  |
| FEMA08 | HG15 | 39 59 16.39085 | 98 29 26.54737 | 543480.828 | 4426536.910 | 592.803 | 566.902 |  |
| FEMA08 | HG16 | 39 58 22.91116 | 98 22 29.51207 | 553382.556 | 4424951.009 | 560.787 | 534.847 |  |
| FEMA08 | HG16_2 | 39 32 20.83116 | 98 09 29.93073 | 572327.239 | 4376944.269 | 454.151 | 427.642 |  |
| FEMA08 | HG17 | 39 51 51.84932 | 98 17 13.46124 | 560975.757 | 4412950.574 | 516.904 | 490.857 |  |
| FEMA08 | HG21 | 39 54 21.35995 | 98 06 59.01322 | 575528.396 | 4417690.461 | 485.794 | 459.715 |  |
| FEMA08 | HG19 | 39 56 00.84948 | 98 02 03.43472 | 582513.458 | 4420830.488 | 508.521 | 482.396 |  |
| FEMA08 | HG20 | 39 51 55.15583 | 97 58 52.85842 | 587123.109 | 4413305.802 | 474.465 | 448.257 |  |
| FEMA09 | BE03 | 39 05 18.61155 | 98 25 07.24021 | 550275.114 | 4326758.768 | 505.473 | 478.863 |  |
| FEMA09 | BE04 | 39 09 40.35510 | 98 21 33.17967 | 555360.666 | 4334861.950 | 451.685 | 424.998 |  |
| FEMA09 | BE05 | 39 14 01.86287 | 98 23 32.15635 | 552451.317 | 4342903.725 | 486.493 | 459.906 |  |
| FEMA09 | BE06 | 39 05 20.87564 | 98 16 41.65227 | 562420.599 | 4326915.653 | 444.314 | 417.451 |  |
| FEMA09 | BE07 | 39 02 32.38115 | 98 08 01.37778 | 574968.928 | 4321830.800 | 427.526 | 400.419 |  |
| FEMA09 | BE08 | 39 02 31.24987 | 97 56 15.62062 | 591935.484 | 4321975.808 | 415.164 | 387.879 |  |
| FEMA09 | BE09 | 39 06 20.70348 | 97 57 46.86564 | 589661.264 | 4329023.799 | 424.900 | 397.660 |  |
| FEMA09 | BE10 | 39 10 32.31590 | 97 58 02.44631 | 589198.791 | 4336776.010 | 402.402 | 375.218 |  |
| FEMA09 | BE11 | 39 11 12.91437 | 98 02 38.65504 | 582558.129 | 4337954.885 | 402.263 | 375.125 |  |
| FEMA09 | BE12 | 39 12 27.98342 | 98 09 18.76727 | 572937.689 | 4340173.725 | 418.013 | 391.024 |  |
| FEMA09 | BE13 | 39 15 44.99948 | 98 05 53.55923 | 577798.793 | 4346294.624 | 422.621 | 395.624 |  |
| FEMA09 | BE15 | 39 24 23.12103 | 97 58 40.22303 | 588002.980 | 4362377.657 | 410.113 | 383.190 |  |
| FEMA09 | BE16 | 39 28 13.88513 | 98 06 00.51710 | 577402.050 | 4369379.608 | 440.070 | 413.376 |  |
| FEMA09 | BE17 | 39 29 41.61530 | 98 17 02.95598 | 561552.509 | 4371942.307 | 432.210 | 405.739 |  |
| FEMA09 | BE19 | 39 20 14.77281 | 98 28 03.60833 | 545875.862 | 4354358.497 | 485.829 | 459.393 |  |
| FEMA09 | BE20 | 39 20 59.54721 | 98 14 31.86303 | 565296.580 | 4355877.476 | 461.656 | 434.971 |  |
| FEMA09 | BR03 | 39 05 22.25103 | 98 25 06.66908 | 550288.117 | 4326871.047 | 505.020 | 478.410 |  |
| FEMA09 | BR04 | 39 09 42.88662 | 98 21 44.26222 | 555094.149 | 4334938.114 | 456.614 | 429.932 |  |
| FEMA09 | BR05 | 39 14 01.23910 | 98 23 39.60251 | 552272.930 | 4342883.301 | 485.682 | 459.098 |  |
| FEMA09 | BR06 | 39 05 24.28493 | 98 16 42.48463 | 562399.769 | 4327020.590 | 444.531 | 417.669 |  |
| FEMA09 | BR08 | 39 02 26.62842 | 97 56 15.20297 | 591947.188 | 4321833.461 | 412.903 | 385.618 |  |
| FEMA09 | BR09 | 39 06 20.36439 | 97 57 46.11930 | 589679.309 | 4329013.551 | 424.075 | 396.835 |  |
| FEMA09 | BR10 | 39 10 34.79305 | 97 58 00.80457 | 589237.314 | 4336852.822 | 401.042 | 373.859 |  |
| FEMA09 | BR11 | 39 11 11.52951 | 98 02 36.92189 | 582600.158 | 4337912.632 | 401.900 | 374.761 |  |
| FEMA09 | BR12 | 39 12 29.47110 | 98 09 15.85455 | 573007.118 | 4340220.237 | 422.387 | 395.398 |  |
| FEMA09 | BR13 | 39 15 45.20810 | 98 06 00.07475 | 577642.587 | 4346299.501 | 421.387 | 394.392 |  |
| FEMA09 | BR15 | 39 24 26.08825 | 97 58 40.68636 | 587990.863 | 4362469.006 | 410.395 | 383.473 |  |
| FEMA09 | BR16 | 39 28 04.16303 | 98 06 04.26177 | 577315.565 | 4369078.997 | 440.541 | 413.843 |  |
| FEMA09 | BR17 | 39 29 41.15892 | 98 16 57.71342 | 561677.840 | 4371929.234 | 431.687 | 405.216 |  |
| FEMA09 | BR19 | 39 20 16.54816 | 98 28 03.65185 | 545874.498 | 4354413.220 | 486.148 | 459.712 |  |
| FEMA09 | BR20 | 39 21 00.52576 | 98 14 29.43192 | 565354.514 | 4355908.130 | 463.491 | 436.806 |  |
| FEMA09 | HG04 | 39 09 40.94770 | 98 21 42.21949 | 555143.592 | 4334878.689 | 455.477 | 428.794 |  |
| FEMA09 | HG05 | 39 14 04.18239 | 98 23 38.05592 | 552309.401 | 4342974.281 | 485.845 | 459.261 |  |
| FEMA09 | HG06 | 39 05 27.19343 | 98 16 45.80127 | 562319.382 | 4327109.616 | 440.466 | 413.606 |  |
| FEMA09 | HG08 | 39 02 43.02672 | 97 56 34.39136 | 591480.018 | 4322333.591 | 416.055 | 388.775 |  |
| FEMA09 | HG09 | 39 06 21.36319 | 97 57 46.48447 | 589670.187 | 4329044.241 | 423.002 | 395.762 |  |
| FEMA09 | HG10 | 39 10 32.58258 | 97 58 00.32638 | 589249.564 | 4336784.810 | 400.634 | 373.450 |  |
| FEMA09 | HG11 | 39 11 12.85423 | 98 02 40.28173 | 582519.123 | 4337952.620 | 402.403 | 375.265 |  |
| FEMA09 | HG12 | 39 12 28.42994 | 98 09 13.47543 | 573064.476 | 4340188.674 | 422.207 | 395.217 |  |
| FEMA09 | HG15 | 39 24 20.57855 | 97 58 35.91889 | 588106.805 | 4362300.443 | 409.862 | 382.937 |  |
| FEMA09 | HG17 | 39 29 42.34603 | 98 16 58.65788 | 561654.990 | 4371965.651 | 432.344 | 405.873 |  |
| FEMA09 | HG19 | 39 20 09.61145 | 98 27 08.14292 | 547204.602 | 4354207.319 | 472.316 | 445.862 |  |
| FEMA09 | HG20 | 39 20 59.07333 | 98 14 33.85990 | 565248.907 | 4355862.467 | 459.805 | 433.121 |  |
| FEMA09 | BE21 | 39 28 52.42419 | 98 04 45.57932 | 579180.455 | 4370585.797 | 447.673 | 420.968 |  |
| FEMA09 | BE22 | 39 27 07.09455 | 97 58 18.80883 | 588457.501 | 4367438.539 | 427.792 | 400.927 |  |
| FEMA09 | BE24 | 39 23 38.07414 | 98 16 59.75889 | 561717.947 | 4360735.621 | 484.188 | 457.614 |  |
| FEMA09 | BE25 | 39 20 59.45880 | 98 07 28.80538 | 575422.423 | 4355966.258 | 456.406 | 456.406 |  |
| FEMA09 | BR21 | 39 27 22.70946 | 97 58 18.76116 | 588453.150 | 4367919.939 | 429.409 | 402.550 |  |
| FEMA09 | BR22 | 39 28 49.55048 | 98 07 07.88278 | 575781.684 | 4370463.214 | 437.577 | 410.926 |  |
| FEMA09 | BR23 | 39 27 34.78498 | 98 08 06.54840 | 574402.262 | 4368144.728 | 420.298 | 393.637 |  |
| FEMA09 | BR24 | 39 25 20.59233 | 98 12 25.30197 | 568255.114 | 4363950.951 | 449.689 | 423.080 |  |
| FEMA09 | BR26 | 39 23 46.60739 | 98 17 01.18070 | 561681.850 | 4360998.413 | 473.812 | 447.242 |  |
| FEMA09 | HG21 | 39 28 52.05345 | 98 04 44.24009 | 579212.566 | 4370574.695 | 447.371 | 420.665 |  |
| FEMA09 | HG22 | 39 27 52.37246 | 98 08 07.26619 | 574379.908 | 4368686.759 | 420.095 | 393.442 |  |
| FEMA09 | HG23 | 39 26 14.34724 | 98 12 36.86116 | 567964.225 | 4365605.691 | 450.112 | 423.530 |  |
| FEMA09 | HG24 | 39 25 22.86369 | 98 16 59.37994 | 561701.352 | 4363966.139 | 460.616 | 434.078 |  |
| FEMA09 | HG25 | 39 24 55.27889 | 98 26 01.35187 | 548748.314 | 4363023.611 | 468.937 | 442.492 |  |
| FEMA09 | HG27 | 39 20 59.26734 | 98 15 23.59916 | 564058.359 | 4355858.561 | 449.151 | 422.483 |  |
| FEMA09 | F09HG30 | 39 27 06.02566 | 98 25 54.02424 | 548898.138 | 4367055.361 | 447.195 | 420.760 |  |
| FEMA09 | F09HG31 | 39 25 22.51013 | 98 23 53.96227 | 551788.887 | 4363882.806 | 461.077 | 434.608 |  |
| FEMA09 | F09HG32 | 39 27 07.26835 | 98 25 33.27150 | 549393.881 | 4367096.813 | 451.055 | 424.616 |  |
| FEMA09 | F09HG33 | 39 25 21.62684 | 98 18 07.05891 | 560083.460 | 4363915.321 | 456.714 | 430.187 |  |
| FEMA10 | BE01 | 38-35-14.58134 | 98-24-56.30413 | 550893.523 | 4271151.606 | 551.634 | 525.187 |  |
| FEMA10 | BE03 | 38-35-21.36060 | 98-10-58.61214 | 571157.737 | 4271515.161 | 526.347 | 499.314 |  |
| FEMA10 | BE04 | 38-42-57.93110 | 98-09-25.61716 | 573278.173 | 4285608.911 | 482.936 | 455.931 |  |
| FEMA10 | BE05 | 38-44-15.59991 | 98-01-52.18489 | 584203.124 | 4288111.331 | 477.115 | 449.860 |  |
| FEMA10 | BE06 | 38-36-26.99255 | 97-57-43.99007 | 590358.707 | 4273732.322 | 456.149 | 428.645 |  |
| FEMA10 | BE07 | 38-40-40.76722 | 97-58-49.22301 | 588694.011 | 4281537.120 | 494.708 | 467.303 |  |
| FEMA10 | BE08 | 38-44-30.59459 | 98-16-52.14409 | 562472.311 | 4288373.281 | 474.260 | 447.552 |  |
| FEMA10 | BE09 | 38-46-09.52910 | 98-24-58.65666 | 550708.073 | 4291339.322 | 493.901 | 467.495 |  |
| FEMA10 | BE10 | 38-49-42.99881 | 98-29-06.33693 | 544694.143 | 4297883.537 | 513.956 | 487.635 |  |
| FEMA10 | BE11 | 38-51-38.26956 | 98-22-28.27961 | 554267.550 | 4301496.632 | 512.617 | 486.091 |  |
| FEMA10 | BE12 | 38-57-52.99532 | 98-28-34.24088 | 545381.314 | 4312992.092 | 459.958 | 433.497 |  |
| FEMA10 | BE15 | 38-50-58.46630 | 98-07-09.52265 | 576422.190 | 4300452.302 | 469.240 | 442.227 |  |
| FEMA10 | BE16 | 38-55-39.35682 | 98-09-04.21023 | 573577.142 | 4309084.699 | 448.812 | 421.816 |  |
| FEMA10 | BE17 | 38-59-26.30348 | 98-04-38.29542 | 579909.093 | 4316142.741 | 413.181 | 386.015 |  |
| FEMA10 | BE18 | 38-55-45.66944 | 98-00-53.40472 | 585392.859 | 4309398.145 | 438.286 | 411.063 |  |
| FEMA10 | BE19 | 38-50-31.18555 | 97-58-33.90550 | 588860.482 | 4299740.952 | 446.558 | 419.252 |  |
| FEMA10 | BE20 | 38-41-42.53720 | 98-22-09.96739 | 554835.465 | 4283136.498 | 516.992 | 490.494 |  |
| FEMA10 | BE21 | 38-49-46.38696 | 98-15-45.99940 | 563990.639 | 4298120.190 | 546.407 | 519.686 |  |
| FEMA10 | BE22 | 38-52-26.40863 | 98-16-53.31163 | 562328.848 | 4303039.936 | 537.983 | 511.282 |  |
| FEMA10 | BE23 | 38-53-59.62384 | 98-18-55.00230 | 559375.000 | 4305890.773 | 529.882 | 503.225 |  |
| FEMA10 | BE24 | 38-55-46.79085 | 98-23-06.30873 | 553299.397 | 4309151.107 | 484.059 | 457.490 |  |
| FEMA10 | BE25 | 38-58-20.62699 | 98-24-29.39260 | 551268.163 | 4313879.928 | 447.230 | 420.648 |  |
| FEMA10 | BE26 | 38-58-41.70384 | 98-15-41.95234 | 563954.666 | 4314622.301 | 452.823 | 425.983 |  |
| FEMA10 | BR01 | 38-35-13.48246 | 98-25-08.73271 | 550593.057 | 4271115.828 | 553.343 | 526.903 |  |
| FEMA10 | BR03 | 38-35-22.12706 | 98-10-59.67904 | 571131.716 | 4271538.557 | 526.325 | 499.293 |  |
| FEMA10 | BR04 | 38-42-55.49976 | 98-09-23.49981 | 573329.997 | 4285534.437 | 483.118 | 456.111 |  |
| FEMA10 | BR05 | 38-44-13.40192 | 98-01-53.15603 | 584180.395 | 4288043.331 | 476.653 | 449.399 |  |
| FEMA10 | BR06 | 38-36-33.51747 | 97-57-28.93160 | 590720.636 | 4273937.574 | 440.719 | 413.207 |  |
| FEMA10 | BR07 | 38-40-40.36007 | 97-58-50.10672 | 588672.797 | 4281524.333 | 495.519 | 468.115 |  |
| FEMA10 | BR08 | 38-44-23.98859 | 98-16-50.30715 | 562518.257 | 4288170.003 | 473.403 | 446.694 |  |
| FEMA10 | BR09 | 38-46-09.16402 | 98-24-45.93353 | 551015.173 | 4291330.033 | 491.944 | 465.531 |  |
| FEMA10 | BR10 | 38-49-38.89657 | 98-29-05.61303 | 544712.310 | 4297757.186 | 512.505 | 486.184 |  |
| FEMA10 | BR11 | 38-51-52.42425 | 98-22-28.81322 | 554251.702 | 4301932.862 | 513.102 | 486.574 |  |
| FEMA10 | BR12 | 38-57-44.16353 | 98-28-34.35131 | 545380.221 | 4312719.833 | 464.008 | 437.549 |  |
| FEMA10 | BR15 | 38-50-57.53306 | 98-06-51.12201 | 576866.012 | 4300427.824 | 465.222 | 438.199 |  |
| FEMA10 | BR16 | 38-55-43.51562 | 98-09-09.51666 | 573448.180 | 4309211.708 | 451.267 | 424.272 |  |
| FEMA10 | BR17 | 38-59-46.02209 | 98-04-38.91562 | 579888.014 | 4316750.438 | 410.003 | 382.835 |  |
| FEMA10 | BR18 | 38-55-44.18769 | 98-00-38.10796 | 585761.668 | 4309356.457 | 429.381 | 402.151 |  |
| FEMA10 | BR19 | 38-50-30.99506 | 97-58-38.15363 | 588758.136 | 4299733.933 | 446.366 | 419.062 |  |
| FEMA10 | BR20 | 38-40-51.76064 | 98-22-10.16403 | 554841.481 | 4281571.325 | 545.280 | 518.781 |  |
| FEMA10 | BR21 | 38-49-15.25718 | 98-15-45.71190 | 564005.313 | 4297160.673 | 545.953 | 519.232 |  |
| FEMA10 | BR22 | 38-52-33.65494 | 98-16-52.51766 | 562346.221 | 4303263.455 | 542.219 | 515.517 |  |
| FEMA10 | BR23 | 38-53-59.52153 | 98-18-33.56164 | 559891.475 | 4305891.512 | 537.542 | 510.874 |  |
| FEMA10 | BR24 | 38-55-45.26033 | 98-23-25.63546 | 552834.376 | 4309100.804 | 506.930 | 480.369 |  |
| FEMA10 | BR25 | 38-57-40.59935 | 98-24-42.70992 | 550955.675 | 4312643.981 | 451.031 | 424.467 |  |
| FEMA10 | BR26 | 38-58-58.55441 | 98-15-39.71766 | 564004.221 | 4315142.168 | 450.601 | 423.754 |  |
| FEMA10 | HG01 | 38-35-12.38075 | 98-25-09.28968 | 550579.797 | 4271081.784 | 552.723 | 526.284 |  |
| FEMA10 | HG03 | 38-35-20.73359 | 98-11-00.27544 | 571117.669 | 4271495.476 | 526.519 | 499.487 |  |
| FEMA10 | HG04 | 38-42-55.52152 | 98-09-22.15732 | 573362.412 | 4285535.406 | 482.583 | 455.576 |  |
| FEMA10 | HG05 | 38-44-12.81797 | 98-01-54.61166 | 584145.442 | 4288024.958 | 478.062 | 450.808 |  |
| FEMA10 | HG06 | 38-36-25.46535 | 97-57-39.39707 | 590470.328 | 4273686.504 | 454.582 | 427.075 |  |
| FEMA10 | HG07 | 38-42-04.55911 | 97-59-01.06720 | 588379.179 | 4284116.804 | 509.204 | 481.825 |  |
| FEMA10 | HG08 | 38-44-24.71956 | 98-16-53.52988 | 562440.278 | 4288191.923 | 473.146 | 446.438 |  |
| FEMA10 | HG09 | 38-46-08.69258 | 98-25-44.80275 | 549594.662 | 4291306.511 | 489.549 | 463.170 |  |
| FEMA10 | HG10 | 38-50-44.04021 | 98-29-06.01140 | 544691.386 | 4299765.165 | 534.171 | 507.839 |  |
| FEMA10 | HG11 | 38-52-00.45126 | 98-22-28.69059 | 554252.962 | 4302180.315 | 512.734 | 486.204 |  |
| FEMA10 | HG12 | 38-57-47.55511 | 98-28-34.00246 | 545388.015 | 4312824.428 | 461.607 | 435.147 |  |
| FEMA10 | HG15 | 38-50-58.14271 | 98-07-15.49483 | 576278.329 | 4300440.940 | 472.044 | 445.034 |  |
| FEMA10 | HG16 | 38-55-42.95993 | 98-09-05.13087 | 573553.941 | 4309195.561 | 450.675 | 423.678 |  |
| FEMA10 | HG17 | 38-59-13.65013 | 98-04-34.07798 | 580014.510 | 4315753.717 | 414.052 | 386.887 |  |
| FEMA10 | HG18 | 38-55-45.14880 | 97-59-59.67410 | 586686.757 | 4309396.181 | 435.208 | 407.962 |  |
| FEMA10 | HG19 | 38-50-32.34156 | 97-58-30.57031 | 588940.485 | 4299777.488 | 446.311 | 419.004 |  |
| FEMA10 | HG20 | 38-41-21.35629 | 98-22-10.68665 | 554822.581 | 4282483.496 | 530.156 | 503.659 |  |
| FEMA10 | HG21 | 38-49-39.44418 | 98-15-56.07890 | 563749.329 | 4297904.222 | 539.715 | 512.999 |  |
| FEMA10 | HG22 | 38-53-07.88832 | 98-16-52.73819 | 562332.598 | 4304318.663 | 532.588 | 505.879 |  |
| FEMA10 | HG23 | 38-53-59.99261 | 98-19-11.73944 | 558971.760 | 4305899.125 | 530.397 | 503.747 |  |
| FEMA10 | HG24 | 38-55-43.74098 | 98-23-16.96014 | 553043.570 | 4309055.368 | 484.780 | 458.216 |  |
| FEMA10 | HG25 | 38-58-18.33537 | 98-24-42.60805 | 550950.618 | 4313807.228 | 448.001 | 421.426 |  |
| FEMA10 | HG26 | 38-58-21.17993 | 98-15-11.49551 | 564692.677 | 4313995.611 | 451.516 | 424.666 |  |
| FEMA10 | BE27 | 39 01 53.14076 | 98 16 47.31830 | 562335.205 | 4320510.911 | 425.194 | 398.344 |  |
| FEMA10 | BE02 | 39 01 56.21127 | 98 28 52.25810 | 544904.998 | 4320486.877 | 444.266 | 417.763 |  |
| FEMA10 | BR27 | 39 01 50.27084 | 98 16 48.12442 | 562316.524 | 4320422.290 | 425.254 | 398.404 |  |
| FEMA10 | BR02 | 39 01 49.46235 | 98 28 52.19936 | 544907.597 | 4320278.845 | 444.044 | 417.542 |  |
| FEMA10 | BR28 | 39 02 15.51940 | 98 07 58.85341 | 575034.567 | 4321311.594 | 434.463 | 407.356 |  |
| FEMA10 | HG27 | 39 01 55.21777 | 98 16 43.41181 | 562428.622 | 4320575.681 | 424.814 | 397.961 |  |
| FEMA10 | HG02 | 39 02 04.07008 | 98 28 53.27453 | 544879.180 | 4320728.992 | 446.768 | 420.264 |  |
| FEMA10 | HG28 | 39 02 00.36302 | 98 07 46.38634 | 575338.760 | 4320847.242 | 425.569 | 398.457 |  |
| FEMA11 | MBE01 | 38 12 28.721533 | 97 54 14.595858 | 595950.103 | 4229458.671 | 466.770 | 438.845 |  |
| FEMA11 | MBE02 | 38 17 37.675453 | 97 52 11.521286 | 598826.998 | 4239017.452 | 476.232 | 448.340 |  |
| FEMA11 | MBR01 | 38 12 31.887843 | 97 54 14.671477 | 595947.110 | 4229556.243 | 466.279 | 438.355 |  |
| FEMA11 | MBR02 | 38 17 25.221145 | 97 52 10.697270 | 598851.708 | 4238633.818 | 474.073 | 446.178 |  |
| FEMA11 | MHG01 | 38 12 24.509579 | 97 54 13.847479 | 595969.841 | 4229329.064 | 467.447 | 439.521 |  |
| FEMA11 | MHG02 | 38 17 30.084850 | 97 52 10.532837 | 598853.870 | 4238783.780 | 474.818 | 446.923 |  |
| FEMA11 | BE02 | 38 33 56.37041 | 98 19 02.51122 | 559470.630 | 4268799.895 | 542.457 | 515.761 |  |
| FEMA11 | BR02 | 38 34 00.49497 | 98 19 00.24447 | 559524.540 | 4268927.437 | 542.981 | 516.285 |  |
| FEMA11 | HG02 | 38 33 58.64618 | 98 18 53.53007 | 559687.451 | 4268871.660 | 542.893 | 516.192 |  |
| FEMA11 | BE03 | 38 13 00.34986 | 98 12 47.58993 | 568873.923 | 4230158.420 | 500.830 | 473.429 |  |
| FEMA11 | BE04 | 38 11 17.73052 | 98 05 25.07776 | 579665.471 | 4227094.044 | 488.006 | 460.343 |  |
| FEMA11 | BE06 | 38 16 58.03981 | 98 07 40.20280 | 576279.545 | 4237551.490 | 496.537 | 469.041 |  |
| FEMA11 | BE07 | 38 17 13.88310 | 97 58 49.34223 | 589171.405 | 4238171.748 | 494.379 | 466.636 |  |
| FEMA11 | BE08 | 38 22 51.21118 | 97 58 47.28898 | 589106.445 | 4248569.734 | 485.197 | 457.531 |  |
| FEMA11 | BE09 | 38 23 12.95943 | 98 05 59.09819 | 578623.239 | 4249131.054 | 531.277 | 503.835 |  |
| FEMA11 | BE10 | 38 14 43.00773 | 98 18 21.37617 | 560733.633 | 4233257.625 | 511.871 | 484.713 |  |
| FEMA11 | BE11 | 38 16 55.12237 | 98 24 56.80976 | 551096.095 | 4237263.259 | 532.795 | 505.954 |  |
| FEMA11 | BE12 | 38 21 06.51631 | 98 20 56.17192 | 556887.750 | 4245050.807 | 523.662 | 496.777 |  |
| FEMA11 | BE13 | 38 26 04.47872 | 98 25 28.74069 | 550214.820 | 4254190.804 | 528.030 | 501.448 |  |
| FEMA11 | BE14 | 38 31 00.80031 | 98 23 44.39603 | 552684.592 | 4263340.489 | 540.282 | 513.727 |  |
| FEMA11 | BE15 | 38 30 55.75595 | 98 15 56.56175 | 564015.072 | 4263267.426 | 537.289 | 510.408 |  |
| FEMA11 | BE16 | 38 21 32.51990 | 98 12 16.07181 | 569504.521 | 4245951.192 | 518.839 | 491.592 |  |
| FEMA11 | BE17 | 38 26 58.95958 | 98 12 20.30452 | 569315.132 | 4256012.103 | 528.555 | 501.433 |  |
| FEMA11 | BE18 | 38 30 46.86500 | 98 09 32.89274 | 573308.875 | 4263072.913 | 533.764 | 506.598 |  |
| FEMA11 | BE19 | 38 30 52.73951 | 98 02 09.97306 | 584033.586 | 4263359.193 | 516.499 | 489.066 |  |
| FEMA11 | BE20 | 38 26 05.58945 | 98 01 05.68620 | 585684.822 | 4254524.670 | 492.686 | 465.136 |  |
| FEMA11 | BE21 | 38 19 33.70639 | 98 14 18.88143 | 566554.212 | 4242263.934 | 506.589 | 479.380 |  |
| FEMA11 | BR03 | 38 13 11.57414 | 98 12 56.45504 | 568655.422 | 4230502.544 | 501.062 | 473.670 |  |
| FEMA11 | BR04 | 38 11 26.29018 | 98 05 14.56031 | 579918.725 | 4227360.386 | 488.299 | 460.635 |  |
| FEMA11 | BR06 | 38 16 55.70072 | 98 07 40.22527 | 576279.679 | 4237479.388 | 495.636 | 468.139 |  |
| FEMA11 | BR07 | 38 20 06.31499 | 98 12 31.34248 | 569156.680 | 4243290.953 | 512.660 | 485.391 |  |
| FEMA11 | BR08 | 38 22 49.27004 | 97 58 47.65901 | 589098.129 | 4248509.803 | 484.911 | 457.245 |  |
| FEMA11 | BR09 | 38 19 53.85870 | 98 14 19.55569 | 566532.722 | 4242884.942 | 510.903 | 483.702 |  |
| FEMA11 | BR10 | 38 14 43.74408 | 98 18 23.21798 | 560688.694 | 4233279.985 | 512.094 | 484.937 |  |
| FEMA11 | BR11 | 38 16 55.93224 | 98 24 55.47345 | 551128.403 | 4237288.425 | 534.583 | 507.741 |  |
| FEMA11 | BR12 | 38 21 06.90663 | 98 20 59.80510 | 556799.483 | 4245062.216 | 523.509 | 496.626 |  |
| FEMA11 | BR13 | 38 26 03.79845 | 98 25 33.13044 | 550108.526 | 4254169.173 | 528.861 | 502.282 |  |
| FEMA11 | BR14 | 38 31 03.41942 | 98 23 45.28995 | 552662.414 | 4263421.076 | 539.772 | 513.218 |  |
| FEMA11 | BR15 | 38 30 55.01507 | 98 16 00.52510 | 563919.274 | 4263243.824 | 536.107 | 509.229 |  |
| FEMA11 | BR16 | 38 21 37.62551 | 98 12 15.66851 | 569512.952 | 4246108.644 | 519.439 | 492.194 |  |
| FEMA11 | BR17 | 38 26 57.73765 | 98 12 21.38073 | 569289.371 | 4255974.215 | 529.475 | 502.353 |  |
| FEMA11 | BR18 | 38 30 50.32518 | 98 09 33.49926 | 573293.212 | 4263179.434 | 532.722 | 505.557 |  |
| FEMA11 | BR19 | 38 30 52.55389 | 98 02 04.67781 | 584161.884 | 4263354.816 | 516.189 | 488.752 |  |
| FEMA11 | BR20 | 38 26 05.89741 | 98 00 56.80229 | 585900.104 | 4254536.460 | 498.420 | 470.866 |  |
| FEMA11 | HG03 | 38 13 00.34640 | 98 12 48.79083 | 568844.722 | 4230158.065 | 500.951 | 473.551 |  |
| FEMA11 | HG04 | 38 11 25.46562 | 98 05 14.17734 | 579928.291 | 4227335.063 | 488.395 | 460.731 |  |
| FEMA11 | HG06 | 38 17 02.95288 | 98 07 40.75279 | 576264.755 | 4237702.796 | 496.448 | 468.954 |  |
| FEMA11 | HG07 | 38 17 19.00579 | 97 58 49.68381 | 589161.366 | 4238329.552 | 494.926 | 467.184 |  |
| FEMA11 | HG08 | 38 22 51.43256 | 97 58 46.54110 | 589124.515 | 4248576.758 | 485.299 | 457.634 |  |
| FEMA11 | HG09 | 38 23 06.96986 | 98 05 50.72819 | 578828.102 | 4248948.421 | 530.593 | 503.146 |  |
| FEMA11 | HG10 | 38 14 41.21038 | 98 18 17.69711 | 560823.477 | 4233202.899 | 511.736 | 484.575 |  |
| FEMA11 | HG11 | 38 16 55.90970 | 98 24 57.13091 | 551088.140 | 4237287.476 | 533.368 | 506.527 |  |
| FEMA11 | HG12 | 38 21 05.92899 | 98 20 51.72728 | 556995.756 | 4245033.466 | 523.603 | 496.715 |  |
| FEMA11 | HG13 | 38 26 03.89652 | 98 25 27.58370 | 550242.982 | 4254173.034 | 527.322 | 500.739 |  |
| FEMA11 | HG14 | 38 30 52.78412 | 98 23 45.90636 | 552649.639 | 4263093.165 | 540.307 | 513.751 |  |
| FEMA11 | HG15 | 38 30 56.38315 | 98 15 50.17656 | 564169.547 | 4263287.994 | 537.356 | 510.471 |  |
| FEMA11 | HG16 | 38 21 30.76004 | 98 12 15.94989 | 569507.947 | 4245896.974 | 518.681 | 491.434 |  |
| FEMA11 | HG17 | 38 26 57.93570 | 98 12 19.15629 | 569343.237 | 4255980.784 | 528.483 | 501.359 |  |
| FEMA11 | HG18 | 38 30 47.47364 | 98 09 31.86780 | 573333.525 | 4263091.901 | 534.550 | 507.383 |  |
| FEMA11 | HG19 | 38 30 52.92365 | 98 02 00.06549 | 584273.463 | 4263367.386 | 515.903 | 488.464 |  |
| FEMA11 | HG20 | 38 26 04.53505 | 98 01 05.60054 | 585687.245 | 4254492.192 | 492.284 | 464.733 |  |
| FEMA12 | BE01 | 38 18 27.69623 | 98 53 29.40142 | 509486.037 | 4239960.706 | 580.256 | 554.166 |  |
| FEMA12 | BE02 | 38 15 49.47284 | 98 58 52.57560 | 501638.450 | 4235078.618 | 593.619 | 567.566 |  |
| FEMA12 | BE03 | 38 22 36.15988 | 98 58 00.16292 | 502907.591 | 4247613.765 | 586.908 | 560.934 |  |
| FEMA12 | BE04 | 38 27 13.67615 | 99 00 50.95515 | 498764.995 | 4256166.999 | 583.432 | 557.472 |  |
| FEMA12 | BE05 | 38 31 01.64966 | 98 56 10.61168 | 505554.844 | 4263195.559 | 614.880 | 588.917 |  |
| FEMA12 | BE06 | 38 38 19.39487 | 98 57 32.71313 | 503560.675 | 4276687.079 | 609.475 | 583.481 |  |
| FEMA12 | BE07 | 38 21 50.96426 | 98 39 50.61708 | 529348.201 | 4246273.623 | 554.081 | 527.875 |  |
| FEMA12 | BE08 | 38 16 31.72375 | 98 42 02.88508 | 526170.295 | 4236423.013 | 569.388 | 543.066 |  |
| FEMA12 | BE09 | 38 13 53.28290 | 98 33 07.62532 | 539199.006 | 4231592.137 | 542.219 | 515.563 |  |
| FEMA12 | BE10 | 38 05 53.23965 | 98 32 46.54211 | 539783.973 | 4216799.163 | 550.792 | 523.921 |  |
| FEMA12 | BE11 | 38 21 10.20920 | 98 34 27.33481 | 537199.142 | 4245049.831 | 543.973 | 517.606 |  |
| FEMA12 | BE12 | 38 25 11.30423 | 98 52 59.35653 | 510199.959 | 4252401.581 | 574.719 | 548.742 |  |
| FEMA12 | BE13 | 38 31 12.10262 | 98 46 08.67332 | 520130.524 | 4263541.089 | 561.301 | 535.305 |  |
| FEMA12 | BE14 | 38 38 21.53023 | 98 46 30.32513 | 519573.824 | 4276776.093 | 599.165 | 573.194 |  |
| FEMA12 | BE15 | 38 38 16.92759 | 98 38 43.50260 | 530859.796 | 4276669.860 | 583.608 | 557.565 |  |
| FEMA12 | BE16 | 38 37 32.16915 | 98 34 53.04820 | 536437.429 | 4275313.719 | 588.034 | 561.921 |  |
| FEMA12 | BE17 | 38 23 03.52837 | 98 48 40.85566 | 516476.268 | 4248473.635 | 567.219 | 541.175 |  |
| FEMA12 | BE18 | 38 31 41.65906 | 98 32 13.31156 | 540354.246 | 4264528.394 | 552.422 | 526.172 |  |
| FEMA12 | BE19 | 38 26 02.44493 | 98 32 09.14749 | 540507.771 | 4254073.378 | 547.781 | 521.449 |  |
| FEMA12 | BE20 | 38 21 56.52310 | 98 42 43.89752 | 525142.646 | 4246430.752 | 559.702 | 533.551 |  |
| FEMA12 | BE21 | 38 26 05.82968 | 98 37 34.58264 | 532617.543 | 4254141.844 | 548.143 | 521.963 |  |
| FEMA12 | BR01 | 38 18 24.75330 | 98 53 34.40955 | 509364.516 | 4239869.858 | 580.152 | 554.062 |  |
| FEMA12 | BR02 | 38 15 48.88840 | 98 58 51.85328 | 501656.006 | 4235060.608 | 593.218 | 567.164 |  |
| FEMA12 | BR03 | 38 22 35.93344 | 98 58 07.39242 | 502732.185 | 4247606.724 | 587.525 | 561.552 |  |
| FEMA12 | BR04 | 38 27 14.85003 | 99 00 47.87168 | 498839.735 | 4256203.170 | 583.387 | 557.427 |  |
| FEMA12 | BR05 | 38 31 00.05726 | 98 56 13.89727 | 505475.314 | 4263146.423 | 615.897 | 589.934 |  |
| FEMA12 | BR06 | 38 38 21.44257 | 98 57 28.82639 | 503654.609 | 4276750.239 | 608.277 | 582.284 |  |
| FEMA12 | BR07 | 38 21 50.90949 | 98 39 52.65923 | 529298.650 | 4246271.754 | 554.123 | 527.918 |  |
| FEMA12 | BR08 | 38 16 37.47112 | 98 42 02.38234 | 526181.936 | 4236600.195 | 569.592 | 543.273 |  |
| FEMA12 | BR09 | 38 13 54.30209 | 98 33 19.51876 | 538909.708 | 4231622.157 | 541.114 | 514.465 |  |
| FEMA12 | BR10 | 38 06 05.11126 | 98 32 47.26115 | 539764.673 | 4217164.970 | 551.146 | 524.280 |  |
| FEMA12 | BR11 | 38 21 06.87125 | 98 34 25.87869 | 537234.959 | 4244947.112 | 543.676 | 517.307 |  |
| FEMA12 | BR12 | 38 25 15.69030 | 98 53 14.33485 | 509836.593 | 4252536.317 | 574.677 | 548.702 |  |
| FEMA12 | BR13 | 38 31 14.08553 | 98 46 09.69130 | 520105.720 | 4263602.147 | 561.476 | 535.481 |  |
| FEMA12 | BR14 | 38 38 23.21892 | 98 46 11.20489 | 520035.925 | 4276829.291 | 597.780 | 571.808 |  |
| FEMA12 | BR15 | 38 38 17.15301 | 98 38 58.26503 | 530502.881 | 4276675.437 | 585.913 | 559.875 |  |
| FEMA12 | BR16 | 38 37 32.14010 | 98 34 55.04680 | 536389.108 | 4275312.603 | 587.467 | 561.355 |  |
| FEMA12 | BR17 | 38 33 55.89253 | 98 36 44.87804 | 533761.552 | 4268635.481 | 566.956 | 540.851 |  |
| FEMA12 | BR18 | 38 31 44.85006 | 98 32 07.50145 | 540494.426 | 4264627.460 | 553.290 | 527.038 |  |
| FEMA12 | BR19 | 38 26 04.49411 | 98 31 52.67436 | 540906.823 | 4254138.560 | 545.994 | 519.653 |  |
| FEMA12 | BR20 | 38 21 55.56316 | 98 42 51.85269 | 524949.693 | 4246400.564 | 558.006 | 531.857 |  |
| FEMA12 | BR21 | 38 22 57.98940 | 98 48 40.82184 | 516477.438 | 4248302.915 | 567.030 | 540.984 |  |
| FEMA12 | HG01 | 38 18 21.58876 | 98 53 39.90457 | 509231.174 | 4239772.168 | 580.085 | 553.996 |  |
| FEMA12 | HG02 | 38 15 52.47786 | 98 58 45.66782 | 501806.292 | 4235171.272 | 592.821 | 566.767 |  |
| FEMA12 | HG03 | 38 22 56.25995 | 98 58 39.06376 | 501963.594 | 4248233.004 | 588.257 | 562.289 |  |
| FEMA12 | HG04 | 38 22 54.69067 | 98 48 41.28325 | 516466.451 | 4248201.218 | 567.612 | 541.565 |  |
| FEMA12 | HG05 | 38 31 01.47393 | 98 55 53.59366 | 505966.954 | 4263190.439 | 610.502 | 584.539 |  |
| FEMA12 | HG06 | 38 38 20.81930 | 98 57 31.84074 | 503581.746 | 4276730.994 | 609.017 | 583.024 |  |
| FEMA12 | HG07 | 38 21 51.15015 | 98 39 51.27475 | 529332.220 | 4246279.294 | 554.034 | 527.829 |  |
| FEMA12 | HG08 | 38 16 26.55272 | 98 42 03.67785 | 526151.548 | 4236263.571 | 569.599 | 543.275 |  |
| FEMA12 | HG09 | 38 13 53.14356 | 98 31 54.59557 | 540974.489 | 4231596.626 | 535.321 | 508.625 |  |
| FEMA12 | HG10 | 38 06 11.35213 | 98 32 47.53073 | 539757.168 | 4217357.286 | 552.242 | 525.379 |  |
| FEMA12 | HG11 | 38 21 06.92108 | 98 34 28.39973 | 537173.763 | 4244948.365 | 543.741 | 517.373 |  |
| FEMA12 | HG12 | 38 26 56.04771 | 98 38 14.06777 | 531654.192 | 4255685.865 | 548.623 | 522.467 |  |
| FEMA12 | HG13 | 38 26 05.89334 | 98 37 32.28678 | 532673.195 | 4254144.032 | 548.060 | 521.880 |  |
| FEMA12 | HG14 | 38 38 22.13431 | 98 46 11.48699 | 520029.189 | 4276795.842 | 597.649 | 571.678 |  |
| FEMA12 | HG15 | 38 38 17.08586 | 98 38 55.60196 | 530567.270 | 4276673.613 | 585.754 | 559.715 |  |
| FEMA12 | HG16 | 38 37 32.92636 | 98 34 56.40136 | 536356.245 | 4275336.689 | 587.441 | 561.329 |  |
| FEMA12 | HG17 | 38 33 56.21622 | 98 35 57.57271 | 534906.288 | 4268650.367 | 560.477 | 534.354 |  |
| FEMA12 | HG18 | 38 32 12.00839 | 98 31 56.80901 | 540749.056 | 4265465.874 | 548.879 | 522.628 |  |
| FEMA12 | HG19 | 38 26 06.05209 | 98 32 01.16677 | 540700.693 | 4254185.536 | 546.690 | 520.355 |  |
| FEMA12 | HG20 | 38 22 02.88431 | 98 42 49.32559 | 525010.317 | 4246626.407 | 559.168 | 533.021 |  |