<@34>001CA—Catoosa silt loam, 0 to 2 percent slopes

<@27>Map Unit Composition

<@33>Catoosa: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Catoosa

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Ridge on upland

<@33><i>Parent material:<p> Residuum weathered from limestone

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silt loam

<@31>H2—11 to 16 inches; silty clay loam

<@31>H3—16 to 27 inches; silty clay loam

<@31>R—27 to 35 inches; unweathered bedrock

<@28>Minor Components

<@28>Kenoma

<@31><i>Composition:<p> About 6 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Zaar

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>001CB—Catoosa-Rock outcrop complex, 1 to 8 percent slopes

<@27>Map Unit Composition

<@33>Catoosa: 58 percent

<@33>Rock outcrop: 29 percent

<@33>Minor components: 13 percent

<@27>Component Descriptions

<@43>Catoosa

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Residuum weathered from limestone

<@33><i>Slope:<p> 1 to 8 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silt loam

<@31>H2—11 to 16 inches; silty clay loam

<@31>H3—16 to 27 inches; silty clay loam

<@31>R—27 to 35 inches; unweathered bedrock

<@43>Rock outcrop

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Drainage class:<p> Well drained

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Land capability (nonirrigated):<p> 8

<@28>Minor Components

<@28>Eram

<@31><i>Composition:<p> About 13 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>001CC—Collinsville-Bates complex, 2 to 15 percent slopes

<@27>Map Unit Composition

<@33>Collinsville: 50 percent

<@33>Bates: 41 percent

<@33>Minor components: 9 percent

<@27>Component Descriptions

<@43>Collinsville

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandstone residuum

<@33><i>Slope:<p> 2 to 15 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately rapid (About 2.00 in/hr)

<@33><i>Available water capacity:<p> Very low (About 2.0 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; fine sandy loam

<@31>H2—7 to 15 inches; fine sandy loam

<@31>R—15 to 23 inches; unweathered bedrock

<@43>Bates

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from sandstone, unspecified over sandy and

silty residuum weathered from sandstone-shale

<@33><i>Slope:<p> 2 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.2 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; loam

<@31>H2—8 to 11 inches; loam

<@31>H3—11 to 32 inches; clay loam

<@31>Cr—32 to 36 inches; unweathered bedrock

<@28>Minor Components

<@28>Catoosa

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>001EC—Eram silty clay loam, 2 to 7 percent slopes, eroded

<@27>Map Unit Composition

<@33>Eram: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 2 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>H2—8 to 30 inches; silty clay

<@31>Cr—30 to 34 inches; weathered bedrock

<@28>Minor Components

<@28>Bates

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>031EB—Eram silt loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Eram: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope, summit

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silt loam

<@31>H2—10 to 28 inches; silty clay

<@31>Cr—28 to 32 inches; weathered bedrock

<@28>Minor Components

<@28>Collinsville

<@31><i>Composition:<p> About 11 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@28>Dwight

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 1 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe35-42)

<@34>031EC—Eram silt loam, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Eram: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope, summit

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silt loam

<@31>H2—10 to 28 inches; silty clay

<@31>H3—28 to 36 inches;

<@28>Minor Components

<@28>Collinsville

<@31><i>Composition:<p> About 8 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 4 to 15 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@28>Shidler

<@31><i>Composition:<p> About 7 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 4 to 8 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Limy (pe35-42)

<@34>031EP—Eram-Apperson silty clay loams, 4 to 7 percent slopes

<@27>Map Unit Composition

<@33>Eram: 50 percent

<@33>Apperson: 35 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 4 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.0 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>H2—8 to 26 inches; silty clay

<@31>Cr—26 to 34 inches; weathered bedrock

<@43>Apperson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Clayey residuum

<@33><i>Slope:<p> 4 to 7 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.0 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 12 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 14 inches; silty clay loam

<@31>H3—14 to 42 inches; silty clay

<@31>R—42 to 46 inches; unweathered bedrock

<@28>Minor Components

<@28>Shidler

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 5 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Limy (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 5 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Rock outcrop

<@31><i>Composition:<p> About 5 percent

<@34>031ER—Eram-Collinsville complex, 4 to 15 percent slopes

<@27>Map Unit Composition

<@33>Eram: 65 percent

<@33>Collinsville: 20 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope, summit

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 5 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silt loam

<@31>H2—10 to 28 inches; silty clay

<@31>Cr—28 to 32 inches; weathered bedrock

<@43>Collinsville

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Shoulder, summit

<@33><i>Parent material:<p> Loamy residuum weathered from sandstone

<@33><i>Slope:<p> 4 to 15 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately rapid (About 2.00 in/hr)

<@33><i>Available water capacity:<p> Very low (About 2.4 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; loam

<@31>R—14 to 18 inches; unweathered bedrock

<@28>Minor Components

<@28>Summit

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Apperson

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Bates

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>031LU—Lula silt loam, 0 to 2 percent slopes

<@27>Map Unit Composition

<@33>Lula: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Lula

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Residuum weathered from limestone, unspecified

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silt loam

<@31>H2—9 to 16 inches; silty clay loam

<@31>H3—16 to 48 inches; silty clay loam

<@31>R—48 to 56 inches;

<@28>Minor Components

<@28>Apperson

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 4 to 5 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 2 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dwight

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 1 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe35-42)

<@28>Shidler

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 4 to 8 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Limy (pe35-42)

<@34>059CM—Clareson-Eram silty clay loams, 3 to 15 percent slopes

<@27>Map Unit Composition

<@33>Clareson: 55 percent

<@33>Eram: 30 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Clareson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone

<@33><i>Slope:<p> 3 to 15 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Shallow Flats (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 7 inches; silty clay loam

<@31>BA—7 to 15 inches; silty clay loam

<@31>Bt—15 to 26 inches; flaggy silty clay loam

<@31>R—26 to 30 inches; unweathered bedrock

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 3 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 38 inches; silty clay

<@31>Cr—38 to 42 inches; weathered bedrock

<@28>Minor Components

<@28>Bates

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Rock outcrop

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@34>059DN—Dennis-Bates complex, 2 to 6 percent slopes

<@27>Map Unit Composition

<@33>Dennis: 60 percent

<@33>Bates: 30 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Dennis

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 2 to 6 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 10.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 12 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silt loam

<@31>H2—10 to 15 inches; silty clay loam

<@31>H3—15 to 60 inches; silty clay

<@43>Bates

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Sandy and silty residuum weathered from sandstone over sandy and silty residuum

weathered from sandstone and shale

<@33><i>Slope:<p> 2 to 6 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.5 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 19 inches; loam

<@31>H2—19 to 26 inches; loam

<@31>H3—26 to 34 inches; gravelly loam

<@31>Cr—34 to 38 inches; unweathered bedrock

<@28>Minor Components

<@28>Eram

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 7 to 12 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Osage

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@34>059EA—Eram-Lebo silty clay loams, 7 to 12 percent slopes

<@27>Map Unit Composition

<@33>Eram: 55 percent

<@33>Lebo: 35 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 7 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 38 inches; silty clay

<@31>Cr—38 to 42 inches; weathered bedrock

<@43>Lebo

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Loamy residuum weathered from sandstone and shale

<@33><i>Slope:<p> 8 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.5 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; silty clay loam

<@31>H2—14 to 28 inches; very channery silty clay loam

<@31>Cr—28 to 36 inches; weathered bedrock

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 2 to 6 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>059EC—Eram-Lula complex, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Eram: 60 percent

<@33>Lula: 25 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 38 inches; silty clay

<@31>Cr—38 to 42 inches; weathered bedrock

<@43>Lula

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope, summit

<@33><i>Parent material:<p> Fine-silty residuum weathered from limestone

<@33><i>Slope:<p> 3 to 5 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silt loam

<@31>H2—7 to 12 inches; silty clay loam

<@31>H3—12 to 44 inches; silty clay loam

<@31>R—44 to 52 inches; unweathered bedrock

<@28>Minor Components

<@28>Bates

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Olpe

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> paleoterrace on upland

<@31><i>Slope:<p> 1 to 5 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>059LN—Lebo-Rock outcrop complex, 20 to 40 percent slopes

<@27>Map Unit Composition

<@33>Lebo: 75 percent

<@33>Rock outcrop: 15 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Lebo

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Residuum weathered from shale-sandstone

<@33><i>Slope:<p> 20 to 40 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 7e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; stony silty clay loam

<@31>H2—7 to 14 inches; channery silty clay loam

<@31>H3—14 to 28 inches; very channery silty clay loam

<@31>Cr—28 to 36 inches; weathered bedrock

<@43>Rock outcrop

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Ridge on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Limestone

<@33><i>Drainage class:<p> Well drained

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Land capability (nonirrigated):<p> 8e

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@34>059OV—Osage silty clay, occasionally flooded

<@27>Map Unit Composition

<@33>Osage: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Osage

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.4 inches)

<@33><i>Shrink-swell potential:<p> Very high (About 17.0 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Ponding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 2 to 9 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 17 inches; silty clay

<@31>H2—17 to 60 inches; silty clay

<@28>Minor Components

<@28>Verdigris

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>602CM—Clareson-Rock outcrop complex, 2 to 15 percent slopes

<@27>Map Unit Composition

<@33>Clareson: 60 percent

<@33>Rock outcrop: 20 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Clareson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone, unspecified

<@33><i>Slope:<p> 7 to 15 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Shallow Flats (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silty clay loam

<@31>H2—11 to 15 inches; very flaggy silty clay loam

<@31>H3—15 to 32 inches; extremely flaggy silty clay

<@31>R—33 to 37 inches;

<@43>Rock outcrop

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Ridge on upland

<@33><i>Hillslope position:<p> Shoulder

<@33><i>Drainage class:<p> Well drained

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Land capability (nonirrigated):<p> 8

<@28>Minor Components

<@28>Catoosa

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 6 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 4 to 8 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Lebo

<@31><i>Composition:<p> About 4 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 15 to 30 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>602EF—Eram-Lebo silty clay loams, 5 to 20 percent slopes

<@27>Map Unit Composition

<@33>Eram: 55 percent

<@33>Lebo: 35 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 5 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 27 inches; silty clay

<@31>Cr—27 to 31 inches;

<@43>Lebo

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Residuum weathered from shale-sandstone

<@33><i>Slope:<p> 8 to 20 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silty clay loam

<@31>H2—12 to 28 inches; channery silty clay loam

<@31>H3—28 to 38 inches;

<@31>Cr—38 to 42 inches;

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 7 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 2 to 5 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>602LB—Lebo channery silty clay loam, 15 to 30 percent slopes

<@27>Map Unit Composition

<@33>Lebo: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Lebo

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Residuum weathered from shale-sandstone

<@33><i>Slope:<p> 15 to 30 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.8 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; channery silty clay loam

<@31>H2—11 to 28 inches; channery silty clay loam

<@31>H3—28 to 38 inches;

<@31>Cr—38 to 42 inches;

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 8 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 7 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 5 to 12 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>AED—Arents, Earthen Dam

<@34>BOP—Borrow Pits

<@33><i>General Considerations:<p> An open excavation from which soil and underlying material have been removed usually for construction purposes.

<@34>Cb—Catoosa silt loam, 0 to 3 percent slopes

<@27>Map Unit Composition

<@33>Catoosa: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Catoosa

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Residuum weathered from limestone

<@33><i>Slope:<p> 0 to 3 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silt loam

<@31>H2—8 to 12 inches; silty clay loam

<@31>H3—12 to 28 inches; silty clay loam

<@31>R—28 to 32 inches; unweathered bedrock

<@28>Minor Components

<@28>Lula

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>Cc—Clareson complex, 1 to 4 percent slopes

<@27>Map Unit Composition

<@33>Clareson: 60 percent

<@33>Rock outcrop: 20 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Clareson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone, unspecified

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Shallow Flats (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; flaggy silty clay loam

<@31>H2—7 to 24 inches; very flaggy silty clay

<@31>R—24 to 32 inches; unweathered bedrock

<@43>Rock outcrop

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@28>Minor Components

<@28>Catoosa

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 0 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Talihina

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>Cd—Collinsville complex, 2 to 15 percent slopes

<@27>Map Unit Composition

<@33>Collinsville: 65 percent

<@33>Bates: 20 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Collinsville

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Shoulder, summit

<@33><i>Parent material:<p> Sandstone residuum

<@33><i>Slope:<p> 2 to 15 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderately rapid (About 2.00 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.4 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very low

<@33><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 6 inches; loam

<@31>H2—6 to 11 inches; loam

<@31>R—11 to 15 inches; unweathered bedrock

<@43>Bates

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from sandstone over sandy and silty residuum

weathered from sandstone and shale

<@33><i>Slope:<p> 2 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.3 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 6 inches; loam

<@31>H2—6 to 32 inches; gravelly clay loam

<@31>H3—32 to 36 inches; gravelly clay loam

<@31>Cr—36 to 40 inches; unweathered bedrock

<@28>Minor Components

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 2 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Talihina

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 2 to 7 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 2 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 2 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Db—Dennis silt loam, 1 to 4 percent slopes

<@27>Map Unit Composition

<@33>Dennis: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Dennis

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit, footslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.8 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 12 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silt loam

<@31>H2—10 to 14 inches; silty clay loam

<@31>H3—14 to 56 inches; silty clay

<@31>Cr—56 to 60 inches;

<@28>Minor Components

<@28>Eram

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Talihina

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Okemah

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>De—Dennis silty clay loam, 1 to 4 percent slopes, eroded

<@27>Map Unit Composition

<@33>Dennis: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Dennis

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Footslope, summit

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.8 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 12 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>H2—10 to 14 inches; silty clay loam

<@31>H3—14 to 56 inches; silty clay

<@31>Cr—56 to 60 inches;

<@28>Minor Components

<@28>Eram

<@31><i>Composition:<p> About 8 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>Eb—Eram silty clay loam, 1 to 4 percent slopes

<@27>Map Unit Composition

<@33>Eram: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Shoulder

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>H2—15 to 33 inches; silty clay

<@31>Cr—33 to 37 inches; weathered bedrock

<@28>Minor Components

<@28>Talihina

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Catoosa

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Collinsville

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 2 to 15 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@34>Ec—Eram silty clay loam, 4 to 7 percent slopes

<@27>Map Unit Composition

<@33>Eram: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Footslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 4 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>H2—15 to 33 inches; silty clay

<@31>Cr—33 to 41 inches; weathered bedrock

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Catoosa

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Collinsville

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 2 to 15 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@28>Talihina

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Ed—Eram Soils, 1 to 4 percent slopes, eroded

<@27>Map Unit Composition

<@33>Eram: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Footslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>H2—15 to 33 inches; silty clay

<@31>Cr—33 to 41 inches; weathered bedrock

<@28>Minor Components

<@28>Talihina

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Collinsville

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 2 to 15 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@34>Eh—Eram Soils, 4 to 7 percent slopes, eroded

<@27>Map Unit Composition

<@33>Eram: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 4 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>H2—15 to 33 inches; silty clay

<@31>Cr—33 to 41 inches; weathered bedrock

<@28>Minor Components

<@28>Talihina

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Collinsville

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 2 to 15 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@34>Ek—Eram-Clareson complex, 1 to 15 percent slopes

<@27>Map Unit Composition

<@33>Eram: 60 percent

<@33>Clareson: 20 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 1 to 15 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.1 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>H2—15 to 33 inches; silty clay

<@31>Cr—33 to 41 inches; weathered bedrock

<@43>Clareson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone

<@33><i>Slope:<p> 1 to 15 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Shallow Flats (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; flaggy silty clay loam

<@31>H2—7 to 24 inches; very flaggy silty clay

<@31>R—24 to 32 inches; unweathered bedrock

<@28>Minor Components

<@28>Rock outcrop

<@31><i>Composition:<p> About 5 percent

<@28>Summit

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 15 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Catoosa

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 1 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Talihina

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 15 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 15 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Eo—Eram-Gullied land complex, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Eram: 40 percent

<@33>Gullied land: 40 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Shoulder

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.1 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>H2—15 to 33 inches; silty clay

<@31>Cr—33 to 41 inches; weathered bedrock

<@43>Gullied land

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>

<@28>Minor Components

<@28>Dennis

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Talihina

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Collinsville

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 1 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>Ep—Eram-Talihina silty clay loams, 5 to 20 percent slopes

<@27>Map Unit Composition

<@33>Eram: 45 percent

<@33>Talihina: 35 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 5 to 20 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>H2—15 to 33 inches; silty clay

<@31>Cr—33 to 37 inches; weathered bedrock

<@43>Talihina

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Clayey residuum weathered from shale

<@33><i>Slope:<p> 5 to 20 percent

<@33><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Very low (About 2.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>Cr—15 to 23 inches; weathered bedrock

<@28>Minor Components

<@28>Summit

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Collinsville

<@31><i>Composition:<p> About 3 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Sandstone (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Ev—Eram-Verdigris complex, 0 to 8 percent slopes

<@27>Map Unit Composition

<@33>Eram: 50 percent

<@33>Verdigris: 15 percent

<@33>Minor components: 35 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 2 to 8 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>H2—15 to 33 inches; silty clay

<@31>Cr—33 to 37 inches; weathered bedrock

<@43>Verdigris

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 12.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 18 inches; silt loam

<@31>H2—18 to 38 inches; silty clay loam

<@31>H3—38 to 65 inches; silt loam

<@28>Minor Components

<@28>Talihina

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 8 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 8 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 8 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 8 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Woodson

<@31><i>Phase:<p> Eroded

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Mason

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@28>Osage

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@34>GRP—Gravel Pit

<@27>Map Unit Composition

<@33>Gravel Pits: 100 percent

<@27>Component Descriptions

<@43>Gravel Pits

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@34>Kb—Kenoma silt loam, 1 to 4 percent slopes

<@27>Map Unit Composition

<@33>Kenoma: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Kenoma

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Loess over ancient clayey alluvium and/or residuum weathered from limestone and

shale

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> High (About 10.4 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silt loam

<@31>H2—7 to 26 inches; silty clay

<@31>H3—26 to 61 inches; silty clay

<@28>Minor Components

<@28>Catoosa

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Olpe

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 2 to 4 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Woodson

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>Kd—Kenoma Soils, 1 to 4 percent slopes, eroded

<@27>Map Unit Composition

<@33>Kenoma: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Kenoma

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loess over ancient clayey alluvium and/or residuum weathered from limestone and

shale

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> High (About 10.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 26 inches; silty clay

<@31>H3—26 to 61 inches; silty clay

<@28>Minor Components

<@28>Catoosa

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Olpe

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 2 to 4 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Woodson

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>Kh—Kenoma-Olpe complex, 2 to 7 percent slopes

<@27>Map Unit Composition

<@33>Kenoma: 40 percent

<@33>Olpe: 35 percent

<@33>Minor components: 25 percent

<@27>Component Descriptions

<@43>Kenoma

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loess over ancient clayey alluvium and/or residuum weathered from limestone and

shale

<@33><i>Slope:<p> 2 to 7 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> High (About 10.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 26 inches; silty clay

<@31>H3—26 to 61 inches; silty clay

<@43>Olpe

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Paleoterrace on upland

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 2 to 7 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; gravelly silt loam

<@31>H2—10 to 16 inches; very gravelly silty clay loam

<@31>H3—16 to 60 inches; extremely gravelly silty clay

<@31>H4—60 to 70 inches; silty clay

<@28>Minor Components

<@28>Catoosa

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 0 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 2 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Woodson

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>La—Lanton silty clay loam, occasionally flooded

<@27>Map Unit Composition

<@33>Lanton: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Lanton

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty and clayey alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Ponding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 12 to 24 inches

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 80 inches; silty clay loam

<@28>Minor Components

<@28>Mason

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@28>Osage

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@28>Verdigris

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>Le—Leanna silt loam, occasionally flooded

<@27>Map Unit Composition

<@33>Leanna: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Leanna

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty and clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> High (About 10.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 6 to 24 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 17 inches; silt loam

<@31>H2—17 to 56 inches; silty clay

<@31>H3—56 to 64 inches; silty clay

<@28>Minor Components

<@28>Lanton

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@28>Osage

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@28>Verdigris

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>Lh—Lula silt loam, 0 to 3 percent slopes

<@27>Map Unit Composition

<@33>Lula: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Lula

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Residuum weathered from limestone, unspecified

<@33><i>Slope:<p> 0 to 3 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.8 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silt loam

<@31>H2—13 to 18 inches; silty clay loam

<@31>H3—18 to 49 inches; silty clay loam

<@31>R—49 to 57 inches; unweathered bedrock

<@28>Minor Components

<@28>Catoosa

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 0 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>LIQ—Limestone Quarry

<@34>Me—Mason silt loam, rarely flooded

<@27>Map Unit Composition

<@33>Mason: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Mason

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Stream terrace on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 10.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Rare

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silt loam

<@31>H2—13 to 65 inches; silty clay loam

<@28>Minor Components

<@28>Lanton

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@28>Osage

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@28>Welda

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Savannah (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Verdigris

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>Mf—Mayes silty clay loam, 0 to 1 percent slopes

<@27>Map Unit Composition

<@33>Mayes: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Mayes

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Silty and clayey pedisediment

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> High (About 9.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 12 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silty clay loam

<@31>H2—11 to 64 inches; silty clay

<@28>Minor Components

<@28>Osage

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@28>Woodson

<@31><i>Phase:<p> Eroded

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>Ok—Okemah silt loam, 0 to 2 percent slopes

<@27>Map Unit Composition

<@33>Okemah: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Okemah

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Colluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.8 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 12 to 24 inches

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silt loam

<@31>H2—9 to 39 inches; silty clay

<@31>H3—39 to 60 inches; silty clay

<@28>Minor Components

<@28>Eram

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Catoosa

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Woodson

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Lula

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Op—Olpe gravelly silt loam, 3 to 15 percent slopes

<@27>Map Unit Composition

<@33>Olpe: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Olpe

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Paleoterrace on upland

<@33><i>Parent material:<p> Cherty clayey alluvium

<@33><i>Slope:<p> 3 to 15 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; gravelly silt loam

<@31>H2—10 to 16 inches; gravelly silty clay loam

<@31>H3—16 to 60 inches; extremely gravelly silty clay

<@31>H4—60 to 70 inches; silty clay

<@28>Minor Components

<@28>Dennis

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Catoosa

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Os—Osage silty clay loam, occasionally flooded

<@27>Map Unit Composition

<@33>Osage: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Osage

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.5 inches)

<@33><i>Shrink-swell potential:<p> Very high (About 17.0 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Ponding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 0 to 12 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silty clay loam

<@31>H2—13 to 64 inches; silty clay

<@28>Minor Components

<@28>Lanton

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@28>Verdigris

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>Se—stony Land-Talihina complex, 15 to 45 percent slopes

<@27>Map Unit Composition

<@33>Stony Land: 60 percent

<@33>Talihina: 20 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Stony Land

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Residuum

<@33><i>Slope:<p> 15 to 45 percent

<@33><i>Drainage class:<p> Excessively drained

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Land capability (nonirrigated):<p> 7e

<@43>Talihina

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Clayey residuum weathered from shale

<@33><i>Slope:<p> 6 to 15 percent

<@33><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Very low (About 2.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 7e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>Cr—15 to 23 inches; weathered bedrock

<@28>Minor Components

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 6 to 15 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Phase:<p> Eroded

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 6 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 6 to 15 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 6 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@34>Sf—Summit silty clay loam, 1 to 4 percent slopes

<@27>Map Unit Composition

<@33>Summit: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Summit

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from calcareous shale

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 24 to 36 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>H2—8 to 12 inches; silty clay

<@31>H3—12 to 69 inches; silty clay

<@28>Minor Components

<@28>Catoosa

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Okemah

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Lula

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Woodson

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>Sh—Summit silty clay loam, 4 to 7 percent slopes

<@27>Map Unit Composition

<@33>Summit: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Summit

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from calcareous shale

<@33><i>Slope:<p> 4 to 7 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 24 to 36 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>H2—8 to 12 inches; silty clay

<@31>H3—12 to 69 inches; silty clay

<@28>Minor Components

<@28>Dennis

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Clareson

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Okemah

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Sk—Summit Soils, 1 to 4 percent slopes, eroded

<@27>Map Unit Composition

<@33>Summit: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Summit

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, calcareous

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 24 to 36 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>H2—8 to 12 inches; silty clay

<@31>H3—12 to 69 inches; silty clay

<@28>Minor Components

<@28>Eram

<@31><i>Phase:<p> Eroded

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 2 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Woodson

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>So—Summit-Eram complex, 4 to 7 percent slopes, eroded

<@27>Map Unit Composition

<@33>Summit: 55 percent

<@33>Eram: 30 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Summit

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, calcareous

<@33><i>Slope:<p> 4 to 7 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 24 to 36 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>H2—8 to 12 inches; silty clay

<@31>H3—12 to 69 inches; silty clay

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 4 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>H2—15 to 33 inches; silty clay

<@31>Cr—33 to 37 inches; weathered bedrock

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>VC—Verdigris silt loam, channeled

<@27>Map Unit Composition

<@33>Verdigris: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Verdigris

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 12.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 24 inches; silt loam

<@31>H2—24 to 60 inches; silt loam

<@28>Minor Components

<@28>Leanna

<@31><i>Phase:<p> Drained

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@28>Osage

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@34>Vf—Verdigris silt loam, occasionally flooded

<@27>Map Unit Composition

<@33>Verdigris: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Verdigris

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 12.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 18 inches; silt loam

<@31>H2—18 to 38 inches; silt loam

<@31>H3—38 to 65 inches; silty clay loam

<@28>Minor Components

<@28>Lanton

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@28>Mason

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>Vx—Verdigris Soils, frequently flooded

<@27>Map Unit Composition

<@33>Verdigris: 55 percent

<@33>Minor components: 45 percent

<@27>Component Descriptions

<@43>Verdigris

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 12.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 18 inches; silt loam

<@31>H2—18 to 38 inches; silt loam

<@31>H3—38 to 65 inches; silty clay loam

<@28>Minor Components

<@28>Mason

<@31><i>Composition:<p> About 15 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@28>Lanton

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@28>Summit

<@31><i>Phase:<p> Eroded

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Osage

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@28>Leanna

<@31><i>Phase:<p> Drained

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@34>W—Water

<@34>Wb—Welda silt loam, 0 to 2 percent slopes

<@27>Map Unit Composition

<@33>Welda: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Welda

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey sediments

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 11.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Savannah (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silt loam

<@31>H2—12 to 42 inches; silty clay

<@31>H3—42 to 60 inches; silty clay loam

<@28>Minor Components

<@28>Dennis

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Mason

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>Wc—Welda silt loam, 2 to 6 percent slopes

<@27>Map Unit Composition

<@33>Welda: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Welda

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey sediments

<@33><i>Slope:<p> 2 to 6 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 11.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Savannah (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silt loam

<@31>H2—12 to 42 inches; silty clay loam

<@31>H3—42 to 60 inches; silty clay loam

<@28>Minor Components

<@28>Summit

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 6 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Wd—Woodson silt loam, 0 to 1 percent slopes

<@27>Map Unit Composition

<@33>Woodson: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Woodson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Divide on upland

<@33><i>Parent material:<p> Silty and clayey alluvium over silty and clayey residuum weathered from shale,

clayey

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 4 to 9 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silt loam

<@31>H2—8 to 29 inches; silty clay

<@31>H3—29 to 64 inches; silty clay

<@28>Minor Components

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Okemah

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Wf—Woodson silt loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Woodson: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Woodson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Paleoterrace on upland

<@33><i>Parent material:<p> Silty and clayey alluvium over silty and clayey residuum weathered from shale,

clayey

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.7 inches)

<@33><i>Shrink-swell potential:<p> Very high (About 13.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 4 to 9 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silt loam

<@31>H2—8 to 29 inches; silty clay

<@31>H3—29 to 64 inches; silty clay

<@28>Minor Components

<@28>Eram

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Wh—Woodson Soils, 1 to 3 percent slopes, eroded

<@27>Map Unit Composition

<@33>Woodson: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Woodson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Divide on upland

<@33><i>Parent material:<p> Silty and clayey alluvium over silty and clayey residuum weathered from shale,

clayey

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 4 to 9 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silty clay loam

<@31>H2—5 to 25 inches; silty clay

<@31>H3—25 to 64 inches; silty clay

<@28>Minor Components

<@28>Kenoma

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 7 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)