<@34>017RA—Reading silt loam, 0 to 1 percent slopes, rarely flooded

<@27>Map Unit Composition

<@33>Reading: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Reading

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Terrace on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 11.7 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Rare

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 17 inches; silt loam

<@31>H2—17 to 48 inches; silty clay loam

<@31>H3—48 to 60 inches; silty clay loam

<@28>Minor Components

<@28>Kahola

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@28>Chase

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@34>017TU—Tully Cherty silty clay loam, 5 to 15 percent slopes

<@27>Map Unit Composition

<@33>Tully: 70 percent

<@33>Minor components: 30 percent

<@27>Component Descriptions

<@43>Tully

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Clayey colluvium

<@33><i>Slope:<p> 5 to 15 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.8 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; gravelly silty clay loam

<@31>H2—14 to 60 inches; silty clay

<@28>Minor Components

<@28>Martin

<@31><i>Composition:<p> About 15 percent

<@31><i>Slope:<p> 2 to 6 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@28>Clime

<@31><i>Composition:<p> About 15 percent

<@31><i>Slope:<p> 3 to 25 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe30-36)

<@34>035LE—Labette-Sogn silty clay loams, 2 to 8 percent slopes

<@27>Map Unit Composition

<@33>Labette: 60 percent

<@33>Sogn: 30 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone-shale

<@33><i>Slope:<p> 2 to 8 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.0 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 36 inches; silty clay

<@31>R—36 to 40 inches; unweathered bedrock

<@43>Sogn

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loamy residuum weathered from limestone, unspecified

<@33><i>Slope:<p> 2 to 8 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 2.0 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Shallow Limy (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 7s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>R—10 to 14 inches; unweathered bedrock

<@28>Minor Components

<@28>Rock outcrop

<@31><i>Composition:<p> About 10 percent

<@31><i>Drainage class:<p> Moderately well drained

<@34>035MA—Martin silty clay loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Martin: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Martin

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Footslope, backslope

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone-shale over silty and clayey

residuum weathered from limestone-shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 15 inches; silty clay loam

<@31>H3—15 to 60 inches; silty clay

<@28>Minor Components

<@28>Tabler

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe24-32)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>035MB—Martin silty clay loam, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Martin: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Martin

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope, footslope

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone-shale over silty and clayey

residuum weathered from limestone-shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>H2—8 to 15 inches; silty clay loam

<@31>H3—15 to 60 inches; silty clay

<@28>Minor Components

<@28>Clime

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 5 to 9 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Limy Upland (pe30-36)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>035SD—Sogn silty clay loam, 0 to 10 percent slopes

<@27>Map Unit Composition

<@33>Sogn: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Sogn

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loamy residuum weathered from limestone, unspecified

<@33><i>Slope:<p> 0 to 10 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 2.0 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Shallow Limy (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 7s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>R—10 to 14 inches; unweathered bedrock

<@28>Minor Components

<@28>Clime

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 7 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Limy Upland (pe30-36)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>049CK—Clime stony silty clay loam, 20 to 30 percent slopes

<@27>Map Unit Composition

<@33>Clime: 100 percent

<@27>Component Descriptions

<@43>Clime

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, calcareous

<@33><i>Slope:<p> 20 to 30 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Limy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 7e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 2 inches; stony silty clay loam

<@31>H2—2 to 10 inches; silty clay loam

<@31>H3—10 to 27 inches; silty clay

<@31>H4—27 to 33 inches; silty clay

<@31>Cr—33 to 37 inches; unweathered bedrock

<@34>049FM—Florence-Martin complex, 2 to 12 percent slopes

<@27>Map Unit Composition

<@33>Florence: 60 percent

<@33>Martin: 30 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Florence

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope, summit

<@33><i>Parent material:<p> Limestone, cherty

<@33><i>Slope:<p> 2 to 12 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 6 inches; gravelly silt loam

<@31>H2—6 to 12 inches; gravelly silt loam

<@31>H3—12 to 32 inches; extremely gravelly silty clay

<@31>H4—32 to 49 inches; extremely cobbly clay

<@31>R—49 to 53 inches; unweathered bedrock

<@43>Martin

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone and shale over silty and

clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 2 to 11 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 60 inches; silty clay

<@28>Minor Components

<@28>Clime

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Limy Upland (pe30-36)

<@28>Dwight

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@34>073LS—Labette-Sogn silty clay loams, 0 to 8 percent slopes

<@27>Map Unit Composition

<@33>Labette: 50 percent

<@33>Sogn: 35 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 1 to 8 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 26 inches; silty clay

<@31>R—26 to 30 inches; unweathered bedrock

<@43>Sogn

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loamy residuum weathered from limestone

<@33><i>Slope:<p> 0 to 8 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 3.0 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Shallow Limy (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 7s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; silty clay loam

<@31>R—15 to 19 inches; unweathered bedrock

<@28>Minor Components

<@28>Clime

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe30-36)

<@28>Rock outcrop

<@31><i>Composition:<p> About 5 percent

<@34>073MB—Martin silty clay loam, 4 to 7 percent slopes

<@27>Map Unit Composition

<@33>Martin: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Martin

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone and shale over silty and

clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 4 to 7 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silty clay loam

<@31>H2—11 to 52 inches; silty clay

<@31>H3—52 to 60 inches; silty clay

<@28>Minor Components

<@28>Clime

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe30-36)

<@34>079DE—Detroit silty clay loam, rarely flooded

<@27>Map Unit Composition

<@33>Detroit: 100 percent

<@27>Component Descriptions

<@43>Detroit

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> River valley, flood plain

<@33><i>Parent material:<p> Alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 10.8 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> Rare

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Terrace (pe25-34)

<@33><i>Land capability (irrigated):<p> 1

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silty clay loam

<@31>H2—11 to 36 inches; silty clay

<@31>H3—36 to 60 inches; silty clay loam

<@34>079GC—Geary silt loam, 0 to 1 percent slopes

<@27>Map Unit Composition

<@33>Geary: 100 percent

<@27>Component Descriptions

<@43>Geary

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Upland, hillslope

<@33><i>Parent material:<p> Loess

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.1 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (irrigated):<p> 1

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silt loam

<@31>H2—9 to 35 inches; silty clay loam

<@31>H3—35 to 60 inches; clay loam

<@34>079GD—Geary silt loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Geary: 100 percent

<@27>Component Descriptions

<@43>Geary

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope, upland

<@33><i>Parent material:<p> Loess

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.0 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (irrigated):<p> 2e

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silt loam

<@31>H2—7 to 32 inches; silty clay loam

<@31>H3—32 to 60 inches; silty clay loam

<@34>079GE—Geary silt loam, 3 to 6 percent slopes

<@27>Map Unit Composition

<@33>Geary: 100 percent

<@27>Component Descriptions

<@43>Geary

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Upland, hillslope

<@33><i>Parent material:<p> Loess

<@33><i>Slope:<p> 3 to 6 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.1 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (irrigated):<p> 3e

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silt loam

<@31>H2—9 to 35 inches; silty clay loam

<@31>H3—35 to 60 inches; clay loam

<@34>079HO—Hobbs silt loam, occasionally flooded

<@27>Map Unit Composition

<@33>Hobbs: 100 percent

<@27>Component Descriptions

<@43>Hobbs

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Flood plain

<@33><i>Parent material:<p> Alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 12.6 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Lowland (pe25-34)

<@33><i>Land capability (irrigated):<p> 2w

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 26 inches; silt loam

<@31>H2—26 to 60 inches; silt loam

<@28>Minor Components

<@28>Unnamed Wet Soils

<@31><i>Phase:<p> Loamy, Depression

<@28>Unnamed Wet Soils

<@31><i>Phase:<p> Loamy, Drainageway

<@34>115CH—Chase silty clay loam, occasionally flooded

<@27>Map Unit Composition

<@33>Chase: 95 percent

<@33>Minor components: 5 percent

<@27>Component Descriptions

<@43>Chase

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty and clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 24 to 48 inches

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silty clay loam

<@31>H2—12 to 60 inches; silty clay

<@28>Minor Components

<@28>Verdigris

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@34>115CS—Clime-Sogn silty clay loams, 3 to 20 percent slopes

<@27>Map Unit Composition

<@33>Clime: 65 percent

<@33>Sogn: 20 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Clime

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from calcareous shale

<@33><i>Slope:<p> 8 to 20 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Limy Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 27 inches; silty clay

<@31>Cr—27 to 31 inches; unweathered bedrock

<@43>Sogn

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loamy residuum weathered from limestone

<@33><i>Slope:<p> 3 to 15 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Shallow Limy (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 6s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>R—8 to 12 inches; unweathered bedrock

<@28>Minor Components

<@28>Labette

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 2 to 8 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@28>Tully

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 6 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@34>115LG—Labette-Sogn silty clay loams, 2 to 15 percent slopes

<@27>Map Unit Composition

<@33>Labette: 65 percent

<@33>Sogn: 25 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 2 to 8 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.0 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 36 inches; silty clay

<@31>R—36 to 40 inches; unweathered bedrock

<@43>Sogn

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loamy residuum weathered from limestone

<@33><i>Slope:<p> 8 to 15 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 2.0 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Shallow Limy (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 6s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>R—10 to 14 inches; unweathered bedrock

<@28>Minor Components

<@28>Dwight

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@34>115VC—Verdigris silt loam, channeled

<@27>Map Unit Composition

<@33>Verdigris: 88 percent

<@33>Minor components: 12 percent

<@27>Component Descriptions

<@43>Verdigris

<@33><i>MLRA:<p> 75 - Central Loess Plains,76 - Bluestem Hills

<@33><i>Landform:<p> Flood plain on valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 12.0 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 7 inches; silt loam

<@31>Bw—7 to 60 inches; silt loam

<@28>Minor Components

<@28>Bates

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 3 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Osage

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@28>Rock outcrop

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 20 to 40 percent

<@31><i>Depth to restrictive feature:<p> 0 inches to bedrock (lithic)

<@34>115WB—Wells loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Wells: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Wells

<@33><i>MLRA:<p> 75 - Central Loess Plains,76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Fine-loamy residuum weathered from sandstone

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 10.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 15 inches; loam

<@31>H2—15 to 36 inches; sandy clay loam

<@31>H3—36 to 60 inches; sandy loam

<@28>Minor Components

<@28>Clime

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe25-34)

<@28>Irwin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@34>173EA—Elandco silt loam, rarely flooded

<@27>Map Unit Composition

<@33>Elandco: 100 percent

<@27>Component Descriptions

<@43>Elandco

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Rare

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Terrace (pe24-32)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 40 inches; silt loam

<@31>H2—40 to 60 inches; silt loam

<@28>Minor Components

<@28>Unnamed Hydric Soils

<@28>Unnamed Hydric Soils

<@28>Unnamed Wet Soils

<@31><i>Phase:<p> Loamy, Drainageway

<@34>173EC—Elandco silt loam, frequently flooded

<@27>Map Unit Composition

<@33>Elandco: 100 percent

<@27>Component Descriptions

<@43>Elandco

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Lowland (pe24-32)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 40 inches; silt loam

<@31>H2—40 to 60 inches; silt loam

<@28>Minor Components

<@28>Unnamed Wet Soils

<@31><i>Phase:<p> Loamy, Depression

<@28>Unnamed Wet Soils

<@31><i>Phase:<p> Loamy, Drainageway

<@28>Unnamed Wet Soils

<@31><i>Phase:<p> Loamy, Drainageway

<@34>173VC—Vanoss silt loam, 3 to 6 percent slopes

<@27>Map Unit Composition

<@33>Vanoss: 100 percent

<@27>Component Descriptions

<@43>Vanoss

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Paleoterrace on river valley

<@33><i>Parent material:<p> Alluvium

<@33><i>Slope:<p> 3 to 6 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe24-32)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silt loam

<@31>H2—13 to 16 inches; silty clay loam

<@31>H3—16 to 60 inches; silty clay loam

<@34>AED—Arents, Earthen Dam

<@34>Be—Benfield-Labette Cherty silty clay loams, 2 to 12 percent slopes

<@27>Map Unit Composition

<@33>Benfield: 60 percent

<@33>Labette: 20 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Benfield

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Calcareous clayey shale

<@33><i>Slope:<p> 2 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; gravelly silty clay loam

<@31>H2—9 to 35 inches; gravelly silty clay

<@31>Cr—35 to 39 inches; weathered bedrock

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Shoulder, summit

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone-shale

<@33><i>Slope:<p> 2 to 8 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.0 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; gravelly silty clay loam

<@31>H2—13 to 38 inches; gravelly silty clay

<@31>R—38 to 42 inches; unweathered bedrock

<@28>Minor Components

<@28>Sogn

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 2 to 12 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Limy (pe30-36)

<@28>Clime

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 2 to 12 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe30-36)

<@28>Dwight

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@34>BOP—Borrow Pits

<@33><i>General Considerations:<p> An open excavation from which soil and underlying material have been removed usually for construction purposes.

<@34>Br—Brewer silty clay loam, rarely flooded

<@27>Map Unit Composition

<@33>Brewer: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Brewer

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 10.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> Rare

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Terrace (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; silty clay loam

<@31>H2—14 to 66 inches; silty clay

<@28>Minor Components

<@28>Osage

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe25-34)

<@28>Verdigris

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@34>Cs—Clime-Sogn complex, 3 to 15 percent slopes

<@27>Map Unit Composition

<@33>Clime: 67 percent

<@33>Sogn: 30 percent

<@33>Minor components: 3 percent

<@27>Component Descriptions

<@43>Clime

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, calcareous

<@33><i>Slope:<p> 3 to 15 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.8 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Limy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay

<@31>H2—9 to 33 inches; silty clay

<@31>Cr—33 to 37 inches; unweathered bedrock

<@43>Sogn

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Loamy residuum weathered from limestone, unspecified

<@33><i>Slope:<p> 3 to 15 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Shallow Limy (pe30-36)

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>R—7 to 11 inches; unweathered bedrock

<@28>Minor Components

<@28>Rock outcrop

<@31><i>Composition:<p> About 3 percent

<@34>Dt—Dwight silt loam, 0 to 2 percent slopes

<@27>Map Unit Composition

<@33>Dwight: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Dwight

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Divide on hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone, cherty

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Pan (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 4s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silt loam

<@31>H2—5 to 33 inches; silty clay

<@31>R—33 to 37 inches; unweathered bedrock

<@28>Minor Components

<@28>Irwin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>Dw—Dwight Soils, 1 to 2 percent slopes, eroded

<@27>Map Unit Composition

<@33>Dwight: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Dwight

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone, cherty

<@33><i>Slope:<p> 1 to 2 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Pan (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silty clay loam

<@31>H2—5 to 33 inches; silty clay

<@31>R—33 to 37 inches; unweathered bedrock

<@28>Minor Components

<@28>Labette

<@31><i>Composition:<p> About 15 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>Fc—Florence Cherty silt loam, 5 to 10 percent slopes

<@27>Map Unit Composition

<@33>Florence: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Florence

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Clayey residuum weathered from clayey shale and/or clayey residuum weathered

from cherty limestone

<@33><i>Slope:<p> 5 to 10 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; gravelly silt loam

<@31>H2—14 to 22 inches; very gravelly silty clay

<@31>H3—22 to 45 inches; very gravelly clay

<@31>R—45 to 49 inches; unweathered bedrock

<@28>Minor Components

<@28>Dwight

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@28>Tully

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>FLL—Florence-Labette complex, 2 to 12 percent slopes

<@27>Map Unit Composition

<@33>Florence: 60 percent

<@33>Labette: 25 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Florence

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from cherty limestone

<@33><i>Slope:<p> 2 to 12 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; gravelly silt loam

<@31>H2—13 to 19 inches; very gravelly silty clay loam

<@31>H3—19 to 45 inches; very gravelly clay

<@31>R—45 to 49 inches; unweathered bedrock

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 2 to 8 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 31 inches; silty clay

<@31>R—31 to 35 inches; unweathered bedrock

<@28>Minor Components

<@28>Clime

<@31><i>Composition:<p> About 8 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe30-36)

<@28>Dwight

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@34>Go—Goessel silty clay, 0 to 1 percent slopes

<@27>Map Unit Composition

<@33>Goessel: 80 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Goessel

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Divide on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Old clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 24 to 36 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 2s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 18 inches; silty clay

<@31>H2—18 to 44 inches; silty clay

<@31>H3—44 to 68 inches; silty clay

<@28>Minor Components

<@28>Rosehill

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Ladysmith

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@34>GRP—Gravel Pits

<@27>Map Unit Composition

<@33>Gravel Pits: 100 percent

<@34>Gs—Goessel silty clay, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Goessel: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Goessel

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Shoulder

<@33><i>Parent material:<p> Old clayey alluvium

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 24 to 36 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 18 inches; silty clay

<@31>H2—18 to 44 inches; silty clay

<@31>H3—44 to 68 inches; silty clay

<@28>Minor Components

<@28>Irwin

<@31><i>Composition:<p> About 8 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Rosehill

<@31><i>Composition:<p> About 7 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@34>Ic—Irwin silty clay loam, 0 to 1 percent slopes

<@27>Map Unit Composition

<@33>Irwin: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Irwin

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, clayey

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 2s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silty clay loam

<@31>H2—11 to 42 inches; silty clay

<@31>H3—42 to 66 inches; silty clay

<@28>Minor Components

<@28>Ladysmith

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Norge

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@34>Id—Irwin silty clay loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Irwin: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Irwin

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Shoulder

<@33><i>Parent material:<p> Silty and clayey residuum weathered from clayey shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silty clay loam

<@31>H2—11 to 42 inches; silty clay

<@31>H3—42 to 66 inches; silty clay

<@28>Minor Components

<@28>Dwight

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@28>Ladysmith

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@34>Ie—Irwin silty clay loam, 3 to 5 percent slopes

<@27>Map Unit Composition

<@33>Irwin: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Irwin

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, clayey

<@33><i>Slope:<p> 3 to 5 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silty clay loam

<@31>H2—11 to 42 inches; silty clay

<@31>H3—42 to 66 inches; silty clay

<@28>Minor Components

<@28>Dwight

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@28>Tully

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>If—Irwin silty clay loam, 2 to 5 percent slopes, eroded

<@27>Map Unit Composition

<@33>Irwin: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Irwin

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, clayey

<@33><i>Slope:<p> 2 to 5 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silty clay loam

<@31>H2—5 to 42 inches; silty clay

<@31>H3—42 to 66 inches; silty clay

<@28>Minor Components

<@28>Dwight

<@31><i>Composition:<p> About 15 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@34>INT—Aquolls

<@27>Map Unit Composition

<@33>Aquolls: 100 percent

<@27>Component Descriptions

<@43>Aquolls

<@33><i>MLRA:<p>  -

<@33><i>Landform:<p> Depression on terrace on river valley

<@33><i>Parent material:<p> Alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Very poorly drained

<@33><i>Flooding hazard:<p> None

<@33><i>Ponding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 0 to 0 inches

<@33><i>Runoff class:<p> Negligible

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 72 inches; variable

<@33><i>General Considerations:<p> This map unit was formerly labeled as an Intermittent Water spot symbol.  These depressional areas contain soils that are occasionally ponded for long duration.

<@34>IVC—Ivan silt loam, channeled

<@27>Map Unit Composition

<@33>Ivan: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Ivan

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Channel on flood plain on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 12.9 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 27 inches; silt loam

<@31>H2—27 to 60 inches; silt loam

<@28>Minor Components

<@28>Chase

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@28>Osage

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe30-36)

<@28>Dennis

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@28>Martin

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>IVF—Ivan silt loam, occasionally flooded

<@27>Map Unit Composition

<@33>Ivan: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Ivan

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 13.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 38 inches; silt loam

<@31>H2—38 to 60 inches; silt loam

<@28>Minor Components

<@28>Chase

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@34>La—Labette silty clay loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Labette: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone-shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silty clay loam

<@31>H2—13 to 38 inches; silty clay

<@31>R—38 to 42 inches; unweathered bedrock

<@28>Minor Components

<@28>Dwight

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@28>Irwin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@34>Lb—Labette silty clay loam, 1 to 3 percent slopes, eroded

<@27>Map Unit Composition

<@33>Labette: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone-shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silty clay loam

<@31>H2—5 to 38 inches; silty clay

<@31>R—38 to 42 inches; unweathered bedrock

<@28>Minor Components

<@28>Irwin

<@31><i>Composition:<p> About 8 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Dwight

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@34>Lc—Labette silty clay loam, 3 to 5 percent slopes

<@27>Map Unit Composition

<@33>Labette: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone-shale

<@33><i>Slope:<p> 3 to 5 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silty clay loam

<@31>H2—13 to 38 inches; silty clay

<@31>R—38 to 42 inches; unweathered bedrock

<@28>Minor Components

<@28>Irwin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Tully

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>Ld—Labette-Dwight complex, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Labette: 50 percent

<@33>Dwight: 40 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone-shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silty clay loam

<@31>H2—13 to 38 inches; silty clay

<@31>R—38 to 42 inches; unweathered bedrock

<@43>Dwight

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone, cherty

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Pan (pe30-36)

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silt loam

<@31>H2—5 to 33 inches; silty clay

<@31>R—33 to 37 inches; unweathered bedrock

<@28>Minor Components

<@28>Irwin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Ladysmith

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@34>Le—Labette-Sogn complex, 2 to 8 percent slopes

<@27>Map Unit Composition

<@33>Labette: 50 percent

<@33>Sogn: 45 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone-shale

<@33><i>Slope:<p> 2 to 8 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silty clay loam

<@31>H2—13 to 38 inches; silty clay

<@31>R—38 to 42 inches; unweathered bedrock

<@43>Sogn

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope, upland

<@33><i>Parent material:<p> Loamy residuum weathered from limestone, unspecified

<@33><i>Slope:<p> 2 to 8 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Shallow Limy (pe30-36)

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>R—7 to 11 inches; unweathered bedrock

<@34>Ls—Ladysmith silty clay loam, 0 to 2 percent slopes

<@27>Map Unit Composition

<@33>Ladysmith: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Ladysmith

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Paleoterrace on upland

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.8 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>H2—8 to 38 inches; silty clay

<@31>H3—38 to 66 inches; silty clay

<@28>Minor Components

<@28>Irwin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Dwight

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 2 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe30-36)

<@34>M-W—Miscellaneous Water

<@34>No—Norge silt loam, 0 to 1 percent slopes

<@27>Map Unit Composition

<@33>Norge: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Norge

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Fine-silty alluvium and/or fine-silty loess

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 11.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silt loam

<@31>H2—9 to 72 inches; silty clay loam

<@28>Minor Components

<@28>Ladysmith

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Vanoss

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@34>Nr—Norge silt loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Norge: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Norge

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Fine-silty alluvium and/or fine-silty loess

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 11.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silt loam

<@31>H2—9 to 72 inches; silty clay loam

<@28>Minor Components

<@28>Tully

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@28>Irwin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Vanoss

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@34>Ns—Norge silt loam, 3 to 5 percent slopes

<@27>Map Unit Composition

<@33>Norge: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Norge

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Fine-silty alluvium and/or fine-silty loess

<@33><i>Slope:<p> 3 to 5 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 11.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silt loam

<@31>H2—9 to 72 inches; silty clay loam

<@28>Minor Components

<@28>Olpe

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 2 to 7 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@28>Tully

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>Nt—Norge silty clay loam, 3 to 5 percent slopes, eroded

<@27>Map Unit Composition

<@33>Norge: 85 percent

<@27>Component Descriptions

<@43>Norge

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Fine-silty alluvium and/or fine-silty loess

<@33><i>Slope:<p> 3 to 5 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 11.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silty clay loam

<@31>H2—5 to 72 inches; silty clay loam

<@34>Od—Oil-Waste Land

<@34>On—Olpe-Norge complex, 2 to 7 percent slopes

<@27>Map Unit Composition

<@33>Olpe: 50 percent

<@33>Norge: 30 percent

<@27>Component Descriptions

<@43>Olpe

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Paleoterrace on upland

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 2 to 7 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; gravelly silty clay loam

<@31>H2—10 to 14 inches; gravelly silty clay loam

<@31>H3—14 to 60 inches; extremely gravelly silty clay

<@43>Norge

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Terrace on upland

<@33><i>Parent material:<p> Fine-silty alluvium and/or fine-silty loess

<@33><i>Slope:<p> 2 to 7 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 11.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silt loam

<@31>H2—9 to 72 inches; silty clay loam

<@34>Os—Osage silty clay, occasionally flooded

<@27>Map Unit Composition

<@33>Osage: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Osage

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.4 inches)

<@33><i>Shrink-swell potential:<p> Very high (About 17.0 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Ponding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 0 to 12 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Lowland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 3w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 18 inches; silty clay

<@31>H2—18 to 66 inches; silty clay

<@28>Minor Components

<@28>Brewer

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Terrace (pe25-34)

<@34>QUA—Quarries

<@34>Ro—Rosehill silty clay, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Rosehill: 85 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Rosehill

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Clayey residuum weathered from clayey shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.4 inches)

<@33><i>Shrink-swell potential:<p> Very high (About 17.0 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay

<@31>H2—9 to 36 inches; silty clay

<@31>Cr—36 to 40 inches; unweathered bedrock

<@28>Minor Components

<@28>Irwin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Goessel

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@34>So—Sogn Soils, 0 to 8 percent slopes

<@27>Map Unit Composition

<@33>Sogn: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Sogn

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loamy residuum weathered from limestone, unspecified

<@33><i>Slope:<p> 0 to 8 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Shallow Limy (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 7s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>R—7 to 11 inches; unweathered bedrock

<@28>Minor Components

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 2 to 8 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@28>Clime

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe30-36)

<@34>Ts—Tully silty clay loam, 1 to 4 percent slopes

<@27>Map Unit Composition

<@33>Tully: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Tully

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Clayey colluvium

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>H2—10 to 64 inches; silty clay

<@28>Minor Components

<@28>Norge

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@28>Vanoss

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@34>Tt—Tully silty clay loam, 3 to 6 percent slopes, eroded

<@27>Map Unit Composition

<@33>Tully: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Tully

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Clayey colluvium

<@33><i>Slope:<p> 3 to 6 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.1 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 6 inches; silty clay loam

<@31>H2—6 to 64 inches; silty clay

<@28>Minor Components

<@28>Norge

<@31><i>Composition:<p> About 15 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@34>Tu—Tully silty clay loam, 4 to 7 percent slopes

<@27>Map Unit Composition

<@33>Tully: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Tully

<@33><i>MLRA:<p> 76 - Bluestem Hills

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Clayey colluvium

<@33><i>Slope:<p> 4 to 7 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>H2—10 to 64 inches; silty clay

<@28>Minor Components

<@28>Irwin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe25-34)

<@28>Norge

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@34>Va—Vanoss silt loam, 0 to 1 percent slopes

<@27>Map Unit Composition

<@33>Vanoss: 95 percent

<@33>Minor components: 5 percent

<@27>Component Descriptions

<@43>Vanoss

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Negligible

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silt loam

<@31>H2—12 to 18 inches; silty clay loam

<@31>H3—18 to 35 inches; silty clay loam

<@31>H4—35 to 57 inches; silty clay loam

<@28>Minor Components

<@28>Norge

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@34>Vb—Vanoss silt loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Vanoss: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Vanoss

<@33><i>MLRA:<p> 75 - Central Loess Plains

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Upland (pe25-34)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silt loam

<@31>H2—12 to 18 inches; silty clay loam

<@31>H3—18 to 35 inches; silty clay loam

<@31>H4—35 to 57 inches; silty clay loam

<@28>Minor Components

<@28>Norge

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe25-34)

<@28>Tully

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>Vd—Verdigris silt loam, occasionally flooded

<@27>Map Unit Composition

<@33>Verdigris: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Verdigris

<@33><i>MLRA:<p> 75 - Central Loess Plains,76 - Bluestem Hills

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silt loam

<@31>H2—8 to 57 inches; silty clay loam

<@28>Minor Components

<@28>Brewer

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Terrace (pe25-34)

<@34>Ve—Verdigris Soils, frequently flooded

<@27>Map Unit Composition

<@33>Verdigris: 85 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Verdigris

<@33><i>MLRA:<p> 75 - Central Loess Plains,76 - Bluestem Hills

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 11.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silt loam

<@31>H2—8 to 57 inches; silty clay loam

<@28>Minor Components

<@28>Tully

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 4 to 7 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@34>W—Water