<@34>045KC—Kennebec Soils, channeled

<@27>Map Unit Composition

<@33>Kennebec: 88 percent

<@33>Minor components: 12 percent

<@27>Component Descriptions

<@43>Kennebec

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 12.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> About 40 to 44 inches

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe30-37)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>H2—10 to 60 inches; silty clay loam

<@28>Minor Components

<@28>Wabash

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe30-37)

<@28>Vinland

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Sogn

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Limy (pe35-42)

<@28>Martin

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>045MO—Martin-Oska silty clay loams, 3 to 6 percent slopes

<@27>Map Unit Composition

<@33>Martin: 40 percent

<@33>Oska: 30 percent

<@33>Minor components: 30 percent

<@27>Component Descriptions

<@43>Martin

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone and shale over silty and

clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 3 to 6 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 22 to 26 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 14 inches; silty clay loam

<@31>H3—14 to 60 inches; silty clay

<@43>Oska

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 3 to 6 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silty clay loam

<@31>H2—5 to 38 inches; silty clay

<@31>R—38 to 42 inches; unweathered bedrock

<@28>Minor Components

<@28>Unnamed Soils

<@31><i>Composition:<p> About 25 percent

<@31><i>Slope:<p> 3 to 6 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Sogn

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Limy (pe30-37)

<@28>Vinland

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 5 to 20 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-37)

<@34>045OE—Oska silty clay loam, 3 to 6 percent slopes

<@27>Map Unit Composition

<@33>Oska: 88 percent

<@33>Minor components: 12 percent

<@27>Component Descriptions

<@43>Oska

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 3 to 6 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silty clay loam

<@31>H2—5 to 38 inches; silty clay

<@31>R—38 to 42 inches; unweathered bedrock

<@28>Minor Components

<@28>Gymer

<@31><i>Composition:<p> About 3 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 8 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-37)

<@28>Martin

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Vinland

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Sogn

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 7 to 15 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Limy (pe30-37)

<@34>045SW—Sogn-Vinland complex, 5 to 20 percent slopes

<@27>Map Unit Composition

<@33>Sogn: 55 percent

<@33>Vinland: 30 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Sogn

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loamy residuum weathered from limestone

<@33><i>Slope:<p> 5 to 20 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 2.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Shallow Limy (pe30-37)

<@33><i>Land capability (nonirrigated):<p> 7s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silty clay loam

<@31>R—13 to 17 inches;

<@43>Vinland

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from shale

<@33><i>Slope:<p> 5 to 20 percent

<@33><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.5 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe30-37)

<@33><i>Land capability (nonirrigated):<p> 6s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silty clay loam

<@31>H2—12 to 16 inches; silty clay loam

<@31>Cr—16 to 20 inches; weathered bedrock

<@28>Minor Components

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 8 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-37)

<@28>Oska

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 2 to 6 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-37)

<@28>Sibleyville

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 7 to 12 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-37)

<@34>045SX—Vinland-Rock outcrop complex, 20 to 40 percent slopes

<@27>Map Unit Composition

<@33>Rock outcrop: 60 percent

<@33>Vinland: 26 percent

<@33>Minor components: 14 percent

<@27>Component Descriptions

<@43>Rock outcrop

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Slope:<p> 20 to 40 percent

<@33><i>Depth to restrictive feature:<p> 0 inches to bedrock (lithic)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Land capability (nonirrigated):<p> 8

<@43>Vinland

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from shale

<@33><i>Slope:<p> 20 to 30 percent

<@33><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.5 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 17 inches; silty clay loam

<@31>Cr—17 to 21 inches; weathered bedrock

<@28>Minor Components

<@28>Sogn

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 15 to 20 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Limy (pe30-37)

<@28>Martin

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 7 to 11 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Oska

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 3 to 6 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>045VM—Vinland-Martin complex, 7 to 15 percent slopes

<@27>Map Unit Composition

<@33>Vinland: 40 percent

<@33>Martin: 25 percent

<@33>Minor components: 35 percent

<@27>Component Descriptions

<@43>Vinland

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from shale

<@33><i>Slope:<p> 7 to 15 percent

<@33><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.5 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 17 inches; silty clay loam

<@31>Cr—17 to 21 inches; weathered bedrock

<@43>Martin

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone and shale over silty and

clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 7 to 11 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 22 to 26 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 14 inches; silty clay loam

<@31>H3—14 to 60 inches; silty clay

<@28>Minor Components

<@28>Unnamed Soil

<@31><i>Composition:<p> About 20 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Sibleyville

<@31><i>Composition:<p> About 8 percent

<@31><i>Slope:<p> 7 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Sogn

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 7 to 15 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Limy (pe30-37)

<@34>045WS—Woodson silt loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Woodson: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Woodson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Divide on upland

<@33><i>Parent material:<p> Silty and clayey alluvium over silty and clayey residuum weathered from clayey

shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> High (About 9.1 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 4 to 9 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silt loam

<@31>H2—11 to 45 inches; silty clay

<@31>H3—45 to 78 inches; silty clay loam

<@28>Minor Components

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Pawnee

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe30-37)

<@34>059CM—Clareson-Eram silty clay loams, 3 to 15 percent slopes

<@27>Map Unit Composition

<@33>Clareson: 55 percent

<@33>Eram: 30 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Clareson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone

<@33><i>Slope:<p> 3 to 15 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Shallow Flats (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 7 inches; silty clay loam

<@31>BA—7 to 15 inches; silty clay loam

<@31>Bt—15 to 26 inches; flaggy silty clay loam

<@31>R—26 to 30 inches; unweathered bedrock

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 3 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 38 inches; silty clay

<@31>Cr—38 to 42 inches; weathered bedrock

<@28>Minor Components

<@28>Bates

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Rock outcrop

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@34>059EA—Eram-Lebo silty clay loams, 7 to 12 percent slopes

<@27>Map Unit Composition

<@33>Eram: 55 percent

<@33>Lebo: 35 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 7 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 38 inches; silty clay

<@31>Cr—38 to 42 inches; weathered bedrock

<@43>Lebo

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Loamy residuum weathered from sandstone and shale

<@33><i>Slope:<p> 8 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.5 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; silty clay loam

<@31>H2—14 to 28 inches; very channery silty clay loam

<@31>Cr—28 to 36 inches; weathered bedrock

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 2 to 6 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>059EC—Eram-Lula complex, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Eram: 60 percent

<@33>Lula: 25 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silty clay loam

<@31>H2—7 to 38 inches; silty clay

<@31>Cr—38 to 46 inches; weathered bedrock

<@43>Lula

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope, summit

<@33><i>Parent material:<p> Fine-silty residuum weathered from limestone

<@33><i>Slope:<p> 3 to 5 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silt loam

<@31>H2—7 to 12 inches; silty clay loam

<@31>H3—12 to 44 inches; silty clay loam

<@31>R—44 to 52 inches; unweathered bedrock

<@28>Minor Components

<@28>Bates

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Olpe

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> paleoterrace on upland

<@31><i>Slope:<p> 1 to 5 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>111CA—Chase silty clay loam, occasionally flooded

<@27>Map Unit Composition

<@33>Chase: 100 percent

<@27>Component Descriptions

<@43>Chase

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Silty and clayey alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 10.0 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 24 to 48 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 17 inches; silty clay loam

<@31>H2—17 to 46 inches; silty clay loam

<@31>H3—46 to 60 inches; silty clay loam

<@28>Minor Components

<@28>Osage

<@34>111EA—Elmont silt loam, 1 to 4 percent slopes

<@27>Map Unit Composition

<@33>Elmont: 100 percent

<@27>Component Descriptions

<@43>Elmont

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale-siltstone

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 9.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silt loam

<@31>H2—12 to 32 inches; silty clay loam

<@31>H3—32 to 48 inches; silty clay loam

<@31>Cr—48 to 59 inches; unweathered bedrock

<@34>111EF—Eram And Bates Soils, 6 to 15 percent slopes

<@27>Map Unit Composition

<@33>Eram: 60 percent

<@33>Bates: 40 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 6 to 15 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.1 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 6 to 18 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silt loam

<@31>H2—7 to 27 inches; silty clay

<@31>Cr—27 to 59 inches; weathered bedrock

<@43>Bates

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from sandstone, unspecified over sandy and

silty residuum weathered from sandstone-shale

<@33><i>Slope:<p> 6 to 8 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.2 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; loam

<@31>H2—7 to 31 inches; clay loam

<@31>H3—31 to 35 inches; gravelly sandy clay loam

<@31>H4—35 to 35 inches; unweathered bedrock

<@34>177AN—Kennebec silt loam, channeled

<@27>Map Unit Composition

<@33>Kennebec: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Kennebec

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 13.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> About 40 to 44 inches

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe30-37)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 48 inches; silt loam

<@31>H2—48 to 80 inches; silty clay loam

<@28>Minor Components

<@28>Wabash

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe30-37)

<@34>177BK—Martin-Kennebec complex, 0 to 12 percent slopes

<@27>Map Unit Composition

<@33>Martin: 54 percent

<@33>Kennebec: 43 percent

<@33>Minor components: 3 percent

<@27>Component Descriptions

<@43>Martin

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone and shale over silty and

clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 3 to 12 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 21 to 26 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe30-37)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silty clay loam

<@31>H2—12 to 17 inches; silty clay loam

<@31>H3—17 to 72 inches; silty clay

<@43>Kennebec

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 13.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> About 40 to 44 inches

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe30-37)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 48 inches; silt loam

<@31>H2—48 to 80 inches; silt loam

<@28>Minor Components

<@28>Wabash

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe30-37)

<@34>177BR—Fluvents, Frequently flooded

<@27>Map Unit Composition

<@33>Fluvents: 97 percent

<@33>Minor components: 3 percent

<@27>Component Descriptions

<@43>Fluvents

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on valley

<@33><i>Parent material:<p> Fine-silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.9 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> About 33 to 38 inches

<@33><i>Runoff class:<p> Low

<@33><i>Land capability (nonirrigated):<p> 6w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 60 inches; silty clay loam

<@28>Minor Components

<@28>Wabash

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe30-37)

<@34>177DM—Dwight-Martin silty clay loams, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Dwight: 60 percent

<@33>Martin: 30 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Dwight

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from cherty limestone

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.0 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Pan (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silty clay loam

<@31>H2—5 to 22 inches; clay

<@31>H3—22 to 50 inches; silty clay

<@31>2C—50 to 80 inches; silty clay

<@43>Martin

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone and shale over silty and

clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 21 to 26 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silty clay loam

<@31>H2—12 to 17 inches; silty clay loam

<@31>H3—17 to 60 inches; silty clay

<@28>Minor Components

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@28>Ladysmith

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>177DW—Dwight silty clay loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Dwight: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Dwight

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from cherty limestone

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.0 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Pan (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 5 inches; silty clay loam

<@31>H2—5 to 22 inches; clay

<@31>H3—22 to 50 inches; silty clay

<@31>2C—50 to 80 inches;

<@28>Minor Components

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@28>Ladysmith

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>177EN—Elmont silt loam, 7 to 12 percent slopes

<@27>Map Unit Composition

<@33>Elmont: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Elmont

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale and siltstone

<@33><i>Slope:<p> 7 to 12 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 9.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Limy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silt loam

<@31>H2—7 to 22 inches; silty clay loam

<@31>H3—22 to 50 inches; silty clay loam

<@31>Cr—50 to 60 inches; unweathered bedrock

<@28>Minor Components

<@28>Sibleyville

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 7 to 11 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 7 to 11 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>177LB—Labette silty clay loam, 3 to 6 percent slopes

<@27>Map Unit Composition

<@33>Labette: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 3 to 6 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Limy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silty clay loam

<@31>H2—13 to 38 inches; silty clay

<@31>R—38 to 42 inches; unweathered bedrock

<@28>Minor Components

<@28>Vinland

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 4 to 10 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@28>Sogn

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 20 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Limy (pe30-37)

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>177LC—Labette silty clay loam, 3 to 6 percent slopes, eroded

<@27>Map Unit Composition

<@33>Labette: 95 percent

<@33>Minor components: 5 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 3 to 6 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Limy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 4 inches; silty clay loam

<@31>H2—4 to 36 inches; silty clay

<@31>R—36 to 40 inches; unweathered bedrock

<@28>Minor Components

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>177LD—Ladysmith silty clay loam, 0 to 1 percent slopes

<@27>Map Unit Composition

<@33>Ladysmith: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Ladysmith

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Paleoterrace on upland

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.8 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 21 to 26 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>H2—10 to 48 inches; silty clay

<@31>H3—48 to 60 inches; silty clay

<@28>Minor Components

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@34>177LM—Ladysmith silty clay loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Ladysmith: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Ladysmith

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Paleoterrace on upland

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Impermeable (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.8 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 22 to 26 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>H2—10 to 48 inches; silty clay

<@31>H3—48 to 60 inches; silty clay

<@28>Minor Components

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@28>Pawnee

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe30-37)

<@34>177LS—Ladysmith silty clay loam, 1 to 3 percent slopes, eroded

<@27>Map Unit Composition

<@33>Ladysmith: 100 percent

<@27>Component Descriptions

<@43>Ladysmith

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Paleoterrace on upland

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Impermeable (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.4 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 21 to 26 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 6 inches; silty clay loam

<@31>H2—6 to 48 inches; silty clay

<@31>H3—48 to 60 inches; silty clay

<@34>177MF—Martin silty clay loam, 7 to 11 percent slopes

<@27>Map Unit Composition

<@33>Martin: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Martin

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone and shale over silty and

clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 7 to 11 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 21 to 26 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silty clay loam

<@31>H2—12 to 17 inches; silty clay loam

<@31>H3—17 to 60 inches; silty clay

<@28>Minor Components

<@28>Elmont

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 7 to 11 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 7 to 11 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@34>177RE—Reading silty clay loam, 0 to 2 percent slopes, rarely flooded

<@27>Map Unit Composition

<@33>Reading: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Reading

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Stream terrace on valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 11.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Rare

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Lowland (pe30-37)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; silty clay loam

<@31>H2—14 to 56 inches; silty clay loam

<@31>H3—56 to 70 inches; silty clay loam

<@28>Minor Components

<@28>Wabash

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe30-37)

<@28>Kennebec

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-37)

<@28>Muir

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-37)

<@34>177SU—Sibleyville loam, 7 to 11 percent slopes

<@27>Map Unit Composition

<@33>Sibleyville: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Sibleyville

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from sandstone and shale

<@33><i>Slope:<p> 7 to 11 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.2 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; loam

<@31>H2—14 to 38 inches; sandy clay loam

<@31>Cr—38 to 42 inches; weathered bedrock

<@28>Minor Components

<@28>Vinland

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 4 to 10 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@28>Elmont

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 7 to 11 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@34>177SV—Sogn-Vinland complex, 3 to 25 percent slopes

<@27>Map Unit Composition

<@33>Sogn: 55 percent

<@33>Vinland: 25 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Sogn

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Loamy residuum weathered from limestone

<@33><i>Slope:<p> 3 to 20 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 2.0 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Shallow Limy (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 7s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silty clay loam

<@31>R—10 to 14 inches; unweathered bedrock

<@43>Vinland

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from shale

<@33><i>Slope:<p> 3 to 25 percent

<@33><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.1 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Limy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6s

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 6 inches; silty clay loam

<@31>H2—6 to 15 inches; channery silty clay loam

<@31>Cr—15 to 19 inches; weathered bedrock

<@28>Minor Components

<@28>Elmont

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 7 to 12 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 7 to 11 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Rock outcrop

<@31><i>Composition:<p> About 5 percent

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 6 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@34>177SW—Vinland-Sogn complex, 15 to 45 percent slopes

<@27>Map Unit Composition

<@33>Stony Steep Land: 60 percent

<@33>Vinland: 26 percent

<@33>Minor components: 14 percent

<@27>Component Descriptions

<@43>Stony Steep Land

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Slope:<p> 15 to 45 percent

<@33><i>Drainage class:<p> Excessively drained

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Land capability (nonirrigated):<p> 8

<@43>Vinland

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from shale

<@33><i>Slope:<p> 15 to 30 percent

<@33><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.7 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe30-37)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 11 inches; silty clay loam

<@31>H2—11 to 17 inches; silty clay loam

<@31>Cr—17 to 20 inches; weathered bedrock

<@28>Minor Components

<@28>Sogn

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 15 to 20 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Limy (pe30-37)

<@28>Martin

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 7 to 11 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Labette

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 3 to 6 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@34>197CE—Chase silty clay loam, rarely flooded

<@27>Map Unit Composition

<@33>Chase: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Chase

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Stream terrace on river valley

<@33><i>Parent material:<p> Silty and clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> Rare

<@33><i>Depth to seasonal water saturation:<p> About 22 to 26 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silty clay loam

<@31>H2—12 to 56 inches; silty clay

<@31>H3—56 to 60 inches; silty clay

<@28>Minor Components

<@28>Ivan

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@28>Reading

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@28>Wabash

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe30-37)

<@28>Unnamed Hydric Soil (saturation)

<@31><i>Composition:<p> About 1 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Poorly drained

<@34>197CS—Clime-Sogn silty clay loams, 5 to 20 percent slopes

<@27>Map Unit Composition

<@33>Clime: 60 percent

<@33>Sogn: 20 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Clime

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from calcareous shale

<@33><i>Slope:<p> 5 to 20 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Limy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silty clay loam

<@31>H2—13 to 30 inches; silty clay

<@31>H3—30 to 37 inches; silty clay loam

<@31>Cr—37 to 41 inches; weathered bedrock

<@43>Sogn

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Loamy residuum weathered from limestone

<@33><i>Slope:<p> 5 to 20 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Shallow Limy (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 13 inches; silty clay loam

<@31>H2—13 to 17 inches; channery silty clay loam

<@31>R—17 to 21 inches; unweathered bedrock

<@28>Minor Components

<@28>Labette

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> ridge on upland

<@31><i>Slope:<p> 2 to 5 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-36)

<@28>Ivan

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe30-36)

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Rock outcrop

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@34>197IB—Irwin silty clay loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Irwin: 80 percent

<@33>Minor components: 20 percent

<@27>Component Descriptions

<@43>Irwin

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Shoulder

<@33><i>Parent material:<p> Silty and clayey residuum weathered from clayey shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 12 inches; silty clay loam

<@31>H2—12 to 42 inches; silty clay, clay

<@31>H3—42 to 60 inches; clay

<@28>Minor Components

<@28>Ladysmith

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> divide on upland

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Somewhat poorly drained

<@31><i>Ecological site:<p> Clay Upland (pe30-36)

<@28>Dwight

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe35-42)

<@34>197WE—Wamego silty clay loam, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Wamego: 75 percent

<@33>Minor components: 25 percent

<@27>Component Descriptions

<@43>Wamego

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Sandy and silty residuum weathered from shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.0 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-37)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 10 inches; silt loam

<@31>H2—10 to 27 inches; silty clay loam

<@31>Cr—27 to 31 inches; weathered bedrock

<@28>Minor Components

<@28>Clime

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@28>Elmont

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-37)

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Rock outcrop

<@31><i>Composition:<p> About 5 percent

<@34>AED—Arents, Earthen Dam

<@27>Map Unit Composition

<@33>Arents, Earthen Dam: 100 percent

<@27>Component Descriptions

<@43>Arents, Earthen Dam

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Land capability (nonirrigated):<p> 8

<@34>Bd—Bates loam, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Bates: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Bates

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from sandstone, unspecified over sandy and

silty residuum weathered from sandstone-shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.7 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 15 inches; loam

<@31>Bt—15 to 23 inches; loam

<@31>BC—23 to 30 inches; gravelly fine sandy loam

<@31>Cr—30 to 34 inches; unweathered bedrock

<@28>Minor Components

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 2 to 6 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Olpe

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 1 to 5 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Cm—Clareson-Eram complex, 3 to 15 percent slopes

<@27>Map Unit Composition

<@33>Clareson: 55 percent

<@33>Eram: 30 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Clareson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone

<@33><i>Slope:<p> 3 to 15 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.2 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Shallow Flats (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 8 inches; silty clay loam

<@31>AB—8 to 16 inches; silty clay loam

<@31>Bt—16 to 24 inches; very flaggy silty clay loam

<@31>R—24 to 32 inches; unweathered bedrock

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 3 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>Ap—0 to 9 inches; silty clay loam

<@31>Bt—9 to 28 inches; silty clay

<@31>Cr—28 to 32 inches; weathered bedrock

<@28>Minor Components

<@28>Rock outcrop

<@31><i>Composition:<p> About 10 percent

<@28>Bates

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Cs—Clime-Sogn complex, 3 to 15

percent slopes

<@27>Map Unit Composition

<@33>Clime: 65 percent

<@33>Sogn: 20 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Clime

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from calcareous shale

<@33><i>Slope:<p> 3 to 15 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Limy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 7 inches; silty clay

<@31>Bw—7 to 15 inches; silty clay

<@31>C—15 to 32 inches; silty clay loam

<@31>Cr—32 to 36 inches; unweathered bedrock

<@43>Sogn

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Loamy residuum weathered from limestone

<@33><i>Slope:<p> 3 to 15 percent

<@33><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Somewhat excessively drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very low (About 1.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Shallow Limy (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6s

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 8 inches; silty clay loam

<@31>R—8 to 12 inches; unweathered bedrock

<@28>Minor Components

<@28>Rock outcrop

<@31><i>Composition:<p> About 10 percent

<@28>Lula

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 1 percent

<@31><i>Geomorphic Position:<p> hillslope

upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Dn—Dennis silt loam, 2 to 6 percent slopes

<@27>Map Unit Composition

<@33>Dennis: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Dennis

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 2 to 6 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.5 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>A1—0 to 9 inches; silt loam

<@31>A2—9 to 14 inches; silty clay loam

<@31>BA—14 to 23 inches; silty clay loam

<@31>Bt—23 to 60 inches; silty clay

<@28>Minor Components

<@28>Eram

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Bates

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Ds—Dwight silt loam, 0 to 3 percent slopes

<@27>Map Unit Composition

<@33>Dwight: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Dwight

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone, cherty

<@33><i>Slope:<p> 0 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Pan (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 5 inches; silt loam

<@31>Bt—5 to 37 inches; silty clay

<@31>BC—37 to 60 inches; silty clay

<@28>Minor Components

<@28>Lula

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 2 to 6 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Summit

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Ed—Elmont loam, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Elmont: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Elmont

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale and siltstone

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 10.3 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 8 inches; loam

<@31>AB—8 to 16 inches; loam

<@31>Bt—16 to 42 inches; clay loam

<@31>BC—42 to 52 inches; clay loam

<@31>Cr—52 to 60 inches; unweathered bedrock

<@28>Minor Components

<@28>Eram

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 12 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@34>EL—Elmont silt loam, 4 to 7 percent slopes

<@27>Map Unit Composition

<@33>Elmont: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Elmont

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale and siltstone

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 9.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Limy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; silt loam

<@31>H2—7 to 22 inches; silty clay loam

<@31>H3—22 to 50 inches; silty clay loam

<@31>Cr—50 to 54 inches; unweathered bedrock

<@28>Minor Components

<@28>Sibleyville

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Martin

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>En—Eram silty clay loam, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Eram: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.3 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>Ap—0 to 9 inches; silty clay loam

<@31>Bt—9 to 28 inches; silty clay, silty clay loam

<@31>Cr—28 to 40 inches; weathered bedrock

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 2 to 6 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Elmont

<@31><i>Composition:<p> About 3 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Er—Eram silty clay, 3 to 7 percent slopes, eroded

<@27>Map Unit Composition

<@33>Eram: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Eram

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, unspecified

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.9 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>Ap—0 to 9 inches; silty clay

<@31>B—9 to 28 inches; silty clay

<@31>Cr—28 to 32 inches; weathered bedrock

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Lebo

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 20 to 40 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>INT—Aquolls

<@27>Map Unit Composition

<@33>Aquolls: 100 percent

<@27>Component Descriptions

<@43>Aquolls

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Very poorly drained

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Ponding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> About 0 to 2 inches

<@33><i>Runoff class:<p> Negligible

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 60 inches; variable

<@34>KB—Kennebec silt loam, occasionally flooded

<@27>Map Unit Composition

<@33>Kennebec: 95 percent

<@33>Minor components: 5 percent

<@27>Component Descriptions

<@43>Kennebec

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on valley

<@33><i>Parent material:<p> Fine-silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 13.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 40 to 44 inches

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 48 inches; silt loam

<@31>H2—48 to 60 inches; silt loam

<@28>Minor Components

<@28>Wabash

<@31><i>Composition:<p> About 4 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe30-37)

<@28>Unnamed Hydric Soil

<@31><i>Composition:<p> About 1 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Poorly drained

<@34>Ke—Kenoma silt loam, 1 to 4 percent slopes

<@27>Map Unit Composition

<@33>Kenoma: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Kenoma

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 8 inches; silt loam

<@31>Bt—8 to 53 inches; silty clay

<@31>Cr—53 to 60 inches; unweathered bedrock

<@28>Minor Components

<@28>Lula

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Olpe

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 5 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>LA—Labette silty clay loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Labette: 100 percent

<@27>Component Descriptions

<@43>Labette

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone-shale

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.4 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe30-36)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 8 inches; silty clay loam

<@31>H2—8 to 38 inches; silty clay

<@31>R—38 to 38 inches; unweathered bedrock

<@34>Ln—Lebo-Rock outcrop complex, 20 to 40 percent slopes

<@27>Map Unit Composition

<@33>Lebo: 75 percent

<@33>Rock outcrop: 15 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Lebo

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Residuum weathered from shale-sandstone

<@33><i>Slope:<p> 20 to 40 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.4 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 7e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 7 inches; stony silty clay loam

<@31>H2—7 to 14 inches; channery silty clay loam

<@31>H3—14 to 28 inches; very channery silty clay loam

<@31>Cr—28 to 36 inches; weathered bedrock

<@43>Rock outcrop

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Ridge on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Limestone

<@33><i>Drainage class:<p> Well drained

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Land capability (nonirrigated):<p> 8e

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 10 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@34>Ls—Lebo-Summit silty clay loams, 7 to 12 percent slopes

<@27>Map Unit Composition

<@33>Lebo: 55 percent

<@33>Summit: 35 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Lebo

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Residuum weathered from clayey shale

<@33><i>Slope:<p> 8 to 12 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Low (About 4.8 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 6e

##

<@23><i>Typical Profile:<p>

<@31>Ap—0 to 6 inches; silty clay loam

<@31>Bw—6 to 22 inches; silty clay loam

<@31>C—22 to 30 inches; very channery clay loam

<@31>Cr—30 to 34 inches; weathered bedrock

<@43>Summit

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from calcareous shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 21 to 26 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 8 inches; silty clay loam

<@31>AB—8 to 13 inches; silty clay loam

<@31>Bt—13 to 60 inches; silty clay

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Rock outcrop

<@31><i>Composition:<p> About 5 percent

<@34>LU—Lula silt loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Lula: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Lula

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Summit

<@33><i>Parent material:<p> Residuum weathered from limestone

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.9 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 8 inches; silt loam

<@31>AB—8 to 14 inches; silty clay loam

<@31>Bt—14 to 44 inches; silty clay loam

<@31>R—44 to 52 inches; unweathered bedrock

<@28>Minor Components

<@28>Eram

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Kenoma

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 1 to 4 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Dwight

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Pan (pe35-42)

<@34>M-W—Miscellaneous Water

<@34>MA—Martin silty clay loam, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Martin: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Martin

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey colluvium derived from limestone and shale over silty and

clayey residuum weathered from limestone and shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> High (About 9.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 21 to 26 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 9 inches; silty clay loam

<@31>H2—9 to 14 inches; silty clay loam

<@31>H3—14 to 60 inches; silty clay

<@28>Minor Components

<@28>Sogn

<@31><i>Composition:<p> About 3 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 7 to 15 percent

<@31><i>Depth to restrictive feature:<p> 4 to 20 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Shallow Limy (pe30-37)

<@28>Pawnee

<@31><i>Composition:<p> About 3 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe30-37)

<@28>Oska

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 6 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Gymer

<@31><i>Composition:<p> About 3 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 8 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe30-37)

<@28>Vinland

<@31><i>Composition:<p> About 2 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Sibleyville

<@31><i>Phase:<p> Eroded

<@31><i>Composition:<p> About 1 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Mb—Mason silt loam, rarely flooded

<@27>Map Unit Composition

<@33>Mason: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Mason

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Stream terrace on valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderately slow (About 0.20 in/hr)

<@33><i>Available water capacity:<p> High (About 10.6 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Rare

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 1

##

<@23><i>Typical Profile:<p>

<@31>Ap—0 to 7 inches; silt loam

<@31>A—7 to 15 inches; silty clay loam

<@31>Bt—15 to 42 inches; silty clay loam

<@31>BC—42 to 60 inches; silty clay loam

<@28>Minor Components

<@28>Osage

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@28>Dennis

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 2 to 6 percent

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Oe—Olpe-Kenoma complex, 1 to 5 percent slopes

<@27>Map Unit Composition

<@33>Olpe: 55 percent

<@33>Kenoma: 30 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Olpe

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Paleoterrace on upland

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 1 to 5 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Low (About 3.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 4e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 7 inches; silty clay loam

<@31>B—7 to 50 inches; extremely gravelly silty clay loam

<@31>B—50 to 60 inches; silty clay

<@43>Kenoma

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Silty and clayey residuum weathered from limestone-shale

<@33><i>Slope:<p> 1 to 4 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 9 to 14 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>Ap—0 to 4 inches; silt loam

<@31>A—4 to 10 inches; silt loam

<@31>Bt1—10 to 18 inches; silty clay

<@31>Bt2—18 to 27 inches; silty clay

<@31>Bt3—27 to 41 inches; silty clay

<@31>Bt4—41 to 59 inches; silty clay

<@31>Bt5—59 to 73 inches; silty clay loam

<@28>Minor Components

<@28>Bates

<@31><i>Composition:<p> About 7 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 5 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 3 to 12 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Lula

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 1 to 3 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Op—Orthents, Hilly

<@27>Map Unit Composition

<@33>Arents: 100 percent

<@27>Component Descriptions

<@43>Arents

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Upland

<@33><i>Slope:<p> 5 to 30 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Low (About 5.2 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Very high

<@33><i>Land capability (nonirrigated):<p> 7s

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 6 inches; silty clay

<@31>C—6 to 80 inches; extremely channery silty clay

<@34>Os—Osage silty clay loam, occasionally flooded

<@27>Map Unit Composition

<@33>Osage: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Osage

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.8 inches)

<@33><i>Shrink-swell potential:<p> Very high (About 17.0 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Ponding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 2 to 9 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>Ap—0 to 8 inches; silty clay loam

<@31>A—8 to 16 inches; silty clay

<@31>Bg1—16 to 30 inches; silty clay

<@31>Bg2—30 to 60 inches; silty clay

<@28>Minor Components

<@28>Mason

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@28>Verdigris

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>Ov—Osage silty clay, occasionally flooded

<@27>Map Unit Composition

<@33>Osage: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Osage

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.4 inches)

<@33><i>Shrink-swell potential:<p> Very high (About 17.0 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Ponding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> About 2 to 9 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 17 inches; silty clay

<@31>H2—17 to 60 inches; silty clay

<@28>Minor Components

<@28>Verdigris

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>Ow—Osage silty clay, Frequently flooded

<@27>Map Unit Composition

<@33>Osage: 85 percent

<@33>Minor components: 15 percent

<@27>Component Descriptions

<@43>Osage

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on river valley

<@33><i>Parent material:<p> Clayey alluvium

<@33><i>Slope:<p> 0 to 1 percent

<@33><i>Drainage class:<p> Poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 6.4 inches)

<@33><i>Shrink-swell potential:<p> Very high (About 17.0 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> About 0 to 12 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>Ap—0 to 8 inches; silty clay

<@31>A—8 to 16 inches; silty clay

<@31>Bg1—16 to 30 inches; silty clay

<@31>Bg2—30 to 60 inches; silty clay

<@28>Minor Components

<@28>Verdigris

<@31><i>Composition:<p> About 15 percent

<@31><i>Slope:<p> 0 to 2 percent

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@34>Pt—Pits, Quarries

<@33><i>General Considerations:<p> Pits are open excavations from which soil and commonly underlying material have been removed, exposing either rock or other material. Kinds include Pits, mine; Pits, gravel; and Pits, quarry. Commonly, pits are closely associated with Dumps.

<@34>SI—Sibleyville loam, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Sibleyville: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Sibleyville

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Parent material:<p> Sandy and silty residuum weathered from sandstone and shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 7.2 inches)

<@33><i>Shrink-swell potential:<p> Low (About 1.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Medium

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; loam

<@31>H2—14 to 38 inches; sandy clay loam

<@31>Cr—38 to 42 inches; weathered bedrock

<@28>Minor Components

<@28>Vinland

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 4 to 10 percent

<@31><i>Depth to restrictive feature:<p> 10 to 20 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Somewhat excessively drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@28>Elmont

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Limy Upland (pe35-42)

<@34>Sn—Summit silty clay loam, 1 to 3 percent slopes

<@27>Map Unit Composition

<@33>Summit: 100 percent

<@27>Component Descriptions

<@43>Summit

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from shale, calcareous

<@33><i>Slope:<p> 1 to 3 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 21 to 26 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 8 inches; silty clay loam

<@31>AB—8 to 13 inches; silty clay loam

<@31>Bt—13 to 60 inches; silty clay

<@34>So—Summit silty clay loam, 3 to 7 percent slopes

<@27>Map Unit Composition

<@33>Summit: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Summit

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Hillslope on upland

<@33><i>Hillslope position:<p> Backslope

<@33><i>Parent material:<p> Silty and clayey residuum weathered from calcareous shale

<@33><i>Slope:<p> 3 to 7 percent

<@33><i>Drainage class:<p> Moderately well drained

<@33><i>Slowest permeability:<p> Slow (About 0.06 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.6 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 21 to 26 inches

<@33><i>Runoff class:<p> High

<@33><i>Ecological site:<p> Loamy Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 3e

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 8 inches; silty clay loam

<@31>AB—8 to 13 inches; silty clay loam

<@31>Bt—13 to 60 inches; silty clay

<@28>Minor Components

<@28>Clareson

<@31><i>Composition:<p> About 5 percent

<@31><i>Slope:<p> 3 to 15 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Shallow Flats (pe35-42)

<@28>Lebo

<@31><i>Composition:<p> About 3 percent

<@31><i>Geomorphic Position:<p> hillslope on upland

<@31><i>Slope:<p> 8 to 12 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Lula

<@31><i>Composition:<p> About 2 percent

<@31><i>Slope:<p> 3 to 5 percent

<@31><i>Depth to restrictive feature:<p> 40 to 60 inches to bedrock (lithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@34>Vb—Verdigris silt loam, occasionally flooded

<@27>Map Unit Composition

<@33>Verdigris: 90 percent

<@33>Minor components: 10 percent

<@27>Component Descriptions

<@43>Verdigris

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> Very high (About 12.1 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Occasional

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2w

##

<@23><i>Typical Profile:<p>

<@31>H1—0 to 14 inches; silt loam

<@31>H2—14 to 60 inches; silt loam

<@28>Minor Components

<@28>Osage

<@31><i>Composition:<p> About 10 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@34>Vc—Verdigris silt loam, channeled

<@27>Map Unit Composition

<@33>Verdigris: 88 percent

<@33>Minor components: 12 percent

<@27>Component Descriptions

<@43>Verdigris

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Flood plain on valley

<@33><i>Parent material:<p> Silty alluvium

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Well drained

<@33><i>Slowest permeability:<p> Moderate (About 0.60 in/hr)

<@33><i>Available water capacity:<p> High (About 12.0 inches)

<@33><i>Shrink-swell potential:<p> Moderate (About 4.5 LEP)

<@33><i>Flooding hazard:<p> Frequent

<@33><i>Depth to seasonal water saturation:<p> More than 6 feet

<@33><i>Runoff class:<p> Low

<@33><i>Ecological site:<p> Loamy Lowland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 5w

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 7 inches; silt loam

<@31>Bw—7 to 60 inches; silt loam

<@28>Minor Components

<@28>Bates

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Well drained

<@31><i>Ecological site:<p> Loamy Upland (pe35-42)

<@28>Eram

<@31><i>Composition:<p> About 3 percent

<@31><i>Geomorphic Position:<p> hillside on upland

<@31><i>Slope:<p> 3 to 7 percent

<@31><i>Depth to restrictive feature:<p> 20 to 40 inches to bedrock (paralithic)

<@31><i>Drainage class:<p> Moderately well drained

<@31><i>Ecological site:<p> Clay Upland (pe35-42)

<@28>Osage

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 0 to 1 percent

<@31><i>Drainage class:<p> Poorly drained

<@31><i>Ecological site:<p> Clay Lowland (pe35-42)

<@28>Rock outcrop

<@31><i>Composition:<p> About 3 percent

<@31><i>Slope:<p> 20 to 40 percent

<@31><i>Depth to restrictive feature:<p> 0 inches to bedrock (lithic)

<@34>W—Water

<@34>Wo—Woodson silt loam, 0 to 2 percent slopes

<@27>Map Unit Composition

<@33>Woodson: 100 percent

<@27>Component Descriptions

<@43>Woodson

<@33><i>MLRA:<p> 112 - Cherokee Prairies

<@33><i>Landform:<p> Divide on upland

<@33><i>Parent material:<p> Silty and clayey alluvium over silty and clayey residuum weathered from shale,

clayey

<@33><i>Slope:<p> 0 to 2 percent

<@33><i>Drainage class:<p> Somewhat poorly drained

<@33><i>Slowest permeability:<p> Very slow (About 0.00 in/hr)

<@33><i>Available water capacity:<p> Moderate (About 8.7 inches)

<@33><i>Shrink-swell potential:<p> High (About 7.5 LEP)

<@33><i>Flooding hazard:<p> None

<@33><i>Depth to seasonal water saturation:<p> About 4 to 9 inches

<@33><i>Runoff class:<p> Very high

<@33><i>Ecological site:<p> Clay Upland (pe35-42)

<@33><i>Land capability (nonirrigated):<p> 2s

##

<@23><i>Typical Profile:<p>

<@31>A—0 to 8 inches; silt loam

<@31>Bt1—8 to 18 inches; silty clay

<@31>Bt2—18 to 28 inches; silty clay

<@31>BCg—28 to 36 inches; silty clay

<@31>Cg1—36 to 54 inches; silty clay loam

<@31>Cg2—54 to 60 inches; clay

<@33><i>General Considerations:<p> This soil is suited to pasture and range.It is moderately well suited to dwellings.  The wetness and shrink swell potential are limitations.  It is moderately suited to local roads and streets because of low strength, wetness and shrink swell potential.  It is well suited to sewage lagoons.