Shawn Saving

Nov 21, 2022

Updated Jan 13, 2022

OFFICIAL NG911 STATEWIDE DATA

NG911 metadata can be found here:

https://www.kansasgis.org/initiatives/NG911/KSNG911GISDataModel/v2.2/Kansas_NG911_GIS_Data_Model_v2_2.pdf

MODIFIED NG911 ROADS LAYER ROADCENTERLINE_NG911_ALL

KDOT.ROADCENTERLINE_NG911_ALL is a modified version of the statewide NG911 Road Centerline layer from June 30, 2022.  Alias records were converted to line features and merged onto the primary roads layer resulting in “stacked” arcs wherever a segment has one or more alias road names.  Highway names have been standardized to the KDOT crash mapping standard of I, U, or K followed by a 3-digit number.  Any resulting duplicate arcs created by stacking and highway name standardization have been removed.

NG911 municipal and county fields have been converted to sequential number codes as per KDOT crash mapping standard.  The municipal codes have been further modified by concatenating the 3-digit county and municipal codes, thus giving unincorporated “999” regions in each county a unique code.

Manual editing of road name fields included correction of highway road names/route numbers, and the addition of arcs that were either not present in the original NG911 data (e.g., “K018 HWY” inside of Fort Riley) or were necessary for geocoding because of common use by officers (e.g., “BAZAAR CATTLE XING” which is not a real road but is frequently referenced in crash reports).

DATA DICTIONARY

*Fields that have been modified/edited from the original NG911 data

STEWARD – Unique numerical ID of local data steward

SUBMITID – Unique alphabetic code of data provider

KGSSEGID* – Unique ID created for the crash mapper roads layer; a concatenation of NGKSSEGID and NGKSALIASID

NGKSSEGID – Unique ID from original statewide NG911 primary roads layer

- use for joining back to source NG911 primary roads layer

- NGKSSEGID is null for arcs where KGS_NOTES = “GEOCODING USE ONLY”; 0

NGKSALIASID – Unique ID from original statewide NG911 alias table

- use for joining back to source NG911 alias table

- where NGKSALIASID is NULL, segment is from the primary ROADCENTERLINE_220630 layer

- where NGKSALIASID begins with “KGS”, record is alias created by KGS

L_F_ADD – Left side From address

L_T_ADD – Left side To address

R_F_ADD – Right side From address

R_T_ADD - Right side To address

KGS_LABEL* - Modified full street name; concatenation of PRD, KGS_STP, KGS_RD, KGS_STS and POD

PRD – Pre directional

KGS_STP* - Road pre type; same as original NG911 except where modified for highway name standardization

KGS_RD* - Road name; same as original NG911 except where modified for highway name standardization

KGS_STS* - Road post type; same as original NG911 except where modified for highway name standardization

POD – Post directional

POM – Post road modifier (optional and not currently used for crash mapping)

KGS_MUNI_L* - 6-digit left side MUNI code derived from NG911 MUNI_L field.  City names were converted to a 3-digit code as per KDOT crash mapping standard and preceded by the 3-digit KGS_MUNI_L code; necessary for accurate geocoding in unincorporated (“999”) areas of a county

KGS_MUNI_R* - 6-digit right side MUNI code derived from NG911 MUNI_R field.  City names were converted to a 3-digit code as per KDOT crash mapping standard and preceded by the 3-digit KGS_MUNI_R code; necessary for accurate geocoding in unincorporated (“999”) areas of a county

KGS_COUNTY_L* – 3-digit left side County code derived from NG911 COUNTY_L field. County names were converted to a 3-digit code as per KDOT crash mapping standard (1-105)

KGS_COUNTY_R* – 3-digit right side County code derived from NG911 COUNTY_R field. County names were converted to a 3-digit code as per KDOT crash mapping standard (1-105)

STP_RD_STS – Concatenation of original STP, RD and STS fields for use in highway name standardization process

STATUS – Road status (OPEN, CLOSED, NOT BUILT)

NOTES – Original explanatory notes from NG911

LRSKEY – KDOT LRS KEY (not currently used)

KGS_NOTES* - Notes related to Crash Mapping roads layer development and of interest mostly to KGS and local NG911 stewards

FOR GEOCODING USE ONLY – arcs added by KGS to facilitate geocoding.  Roads may not truly exist, or the name is not considered an official name (primary or alias) by the local authority.  May also apply to a highway segment in a complex interchange that does not correspond to a segment in LRS but is necessary for accurate geocoding

ATTRIIBUTE CHANGE – indicates KGS changed the NG911 highway route number

GEOMETRY CHANGE – indicates that locals need to edit primary linework to align it with current highway configuration

2 ALIASES TO 1 SEG – indicates a primary arc that has two aliases with conflicting information

PROJECTION INFORMATION

Projected Coordinate System:	NAD_83_Kansas_Lambert_Conformal_Conic_Feet(2011)

Projection:	Lambert_Conformal_Conic

false_easting:	1312333.33333333

false_northing:	0.00000000

central_meridian:	-98.25000000

standard_parallel_1:	37.50000000

standard_parallel_2:	39.50000000

scale_factor:	1.00000000

latitude_of_origin:	36.00000000

Linear Unit: 	Foot_US

Geographic Coordinate System:	GCS_NAD_1983_2011

Datum: 	D_NAD_1983_2011

Prime Meridian: 	Greenwich

Angular Unit: 	Degree